Report

© Epic Atlantic Limited

Epic Atlantic Limited

12 Kenneth Agbakuru Street, Lekki Phase I, Lagos, NG

www.epicatlantic.com | info@epicatlantic.com | +234 809 835 1008

This report contains confidential information and is intended only for the use of the individual named above. If you are not the named addressee you should not disseminate, distribute or copy this report without authorization.

SPDC-BFPC UPSTREAM GAS SUPPLY, GATHERING,

EVACUATION & PROCESSING (UGS-GEP) PROJECT

Conceptual Study Memorandum

© Epic Atlantic Limited

Epic Atlantic Limited

12 Kenneth Agbakuru Street, Lekki Phase I, Lagos, NG

www.epicatlantic.com | info@epicatlantic.com | +234 809 835 1008

This report contains confidential information and is intended only for the use of the individual named above. If you are not the named addressee you should not disseminate, distribute or copy this report without authorization.

© Epic Atlantic Limited

Epic Atlantic Limited

12 Kenneth Agbakuru Street, Lekki Phase I, Lagos, NG

www.epicatlantic.com | info@epicatlantic.com | +234 809 835 1008

This report contains confidential information and is intended only for the use of the individual named above. If you are not the named addressee you should not disseminate, distribute or copy this report without authorization.

Endorsed by:

Aloy Onyenucheya Duru, COREN (R.7580)

15th February 2017

Prepared for:

Brass Fertilizer & Petrochemical Company Limited

Doc. No. EA/SPDC-BFPC UGS-GEP/CSM-RPT-001

© Epic Atlantic Limited

Epic Atlantic Limited

12 Kenneth Agbakuru Street, Lekki Phase I, Lagos, NG

www.epicatlantic.com | info@epicatlantic.com | +234 809 835 1008

This report contains confidential information and is intended only for the use of the individual named above. If you are not the named addressee you should not disseminate, distribute or copy this report without authorization.

© Epic Atlantic Limited

Epic Atlantic Limited

12 Kenneth Agbakuru Street, Lekki Phase I, Lagos, NG

www.epicatlantic.com | info@epicatlantic.com | +234 809 835 1008

This report contains confidential information and is intended only for the use of the individual named above. If you are not the named addressee you should not disseminate, distribute or copy this report without authorization.

© Epic Atlantic Limited

Epic Atlantic Limited

12 Kenneth Agbakuru Street, Lekki Phase I, Lagos, NG

www.epicatlantic.com | info@epicatlantic.com | +234 809 835 1008

This report contains confidential information and is intended only for the use of the individual named above. If you are not the named addressee you should not disseminate, distribute or copy this report without authorization.

Statement of Confidentiality

This Conceptual Study Memorandum may contain confidential and proprietary business information of Parties involved in this project. These materials may be printed or photocopied for use in evaluating the proposed project, but are not to be shared with other parties.

Epic Atlantic Limited | 12 Kenneth Agbakuru Street | Lekki Phase I, Lagos, NG | www.epicatlantic.com | info@epicatlantic.com

SUMMARY PAGE

| Client: BRASS FERTILIZER & PETROCHEMICAL COMPANY LIMITED |  |
| --- | --- |
| Project Ref: SPDC-BFPC UGS-GEP | Title: Conceptual Study Memorandum |
| Facilities: Gas Gathering & Evacuation Pipelines, and Gas Plant | Doc No.: EA/SPDC-BFPC UGS-GEP/CSM-RPT-001 |

The administration of this document is in respect of the SPDC-BFPC Upstream Gas Supply, Gathering, Evacuation and Processing (UGS-GEP) Project. The document, in consideration with relational design elements & concepts, presents the project’s concept selection study as a reference & premise to the Basic Design, as well as demonstrates readiness to progress to the Basic Design, and the subsequent FEED (Front End and Engineering Design). It is also (in conjunction with the Basic Design) a presentation to DPR as a statutory requirement in the process of securing regulators’ approval to execute the FEED / Detail Design that will lead to the project’s FID and implementation. The report has relied and used data and reference documents provided by client, and if further document and information become available, Epic Atlantic reserves the right to update the opinion set out in this report. For better appreciation, this document should be read in conjunction with the references listed below, which are the Concept Select Study reports that constitute basis for the Conceptual Study.

KEY WORDS: BASIS FOR DESIGN, PIPELINE, MANIFOLD, PLATFORM, GAS, plant, ROW, PTS

| REferences |  |
| --- | --- |
| Document No. | Description |
| TO-D-AA-00001 | Basis for Design |
| EA/SPDC-BFPC UGS-GEP/CSM-RPT-001 | Gas Gathering & Surface Facilities Concept Screening Study |
| EA/SPDC-BFPC UGS-GEP/SVY-RPT-001 | Pipeline Route Selection Study |
| EA/SPDC-BFPC UGS-GEP/PRC-RPT-001 | Process Simulation Study – HYSYS, PIPESIM, & OLGA |
| EA/SPDC-BFPC UGS-GEP/MAT-RPT-001 | Pipeline Materials Selection & Corrosion Analysis Study |
| EA/SPDC-BFPC UGS-GEP/ELE-RPT-001 | Manifold Platforms Power Supply Options |
| EA/SPDC-BFPC UGS-GEP/MEC-RPT-001 | Manifold Platforms Concept & Integration with SPDC |
| EA/SPDC-BFPC UGS-GEP/PRC-RPT-002 | Gas Processing Technology Option |

Table of Contents

1.	Administration	6

1.1.	Validity & Change Management	6

1.2.	Background and Introduction	7

1.3.	Scope of Document	7

2.	Project Design Data & Basis for Design Outline	9

2.1.	Project Design Data	9

2.1.1.	Project Location & Operational Conditions	9

2.1.2.	Production Forecast & Fluid Characteristics	10

2.1.3.	Environmental Data	12

2.2.	Basis for Design Outline	13

3.	Project Concept and Site for the Project	35

3.1.	The Gas Plant Site	36

3.2.	The Gas Gathering & Evacuation Concept	37

3.3.	The Gas Gathering & Evacuation Options	38

3.4.	The Manifolds Concept	41

3.5.	The Gas Processing	42

4.	Feedstock Supply & Product Evacuation Scheme	46

4.1.	The Feedstock & Products Evacuation Schematics	46

5.	Pipeline Route Selection	47

5.1.	Pipeline Route for Phase-1 Production	47

5.2.	Routes Options Considered	48

5.3.	Field Investigation Works	50

5.3.1.	Other Considerations	51

5.4.	Right of Way (ROW) Acquisition	52

5.5.	PTS Application Plan	52

5.6.	The Full Preparatory Survey	53

6.	Process Simulation	54

6.1.	The Gas Gathering & Process Flow Scheme	56

6.2.	Basis and Assumptions	57

6.2.1.	Software & Simulation Property Package	57

6.2.2.	Flow Rates, Fluid Condition & Composition	58

6.3.	Gas Deliverability & Flow Assurance	59

6.3.1.	Phase-1 Production (BBOU & ELEP)	60

1.1.1.	Phase-2 Production (NUNR & ELEP)	68

1.1.2.	Phase-3 Production (SEIB, NUNR & ELEP)	70

1.1.3.	Phase-4 Production (OPUG, SEIB, & NUNR)	73

7.	Pipeline Materials Selection & Corrosion Analysis	82

7.1.	Intent & Basis	82

7.2.	Purpose & Objective of the Study	83

7.3.	Materials Options	83

7.4.	The System & Fluid Under Consideration	84

7.5.	The Fluid Impurities Composition	85

7.6.	Issues & Considerations	86

7.7.	Recommended Materials	87

7.8.	Materials Specifications	89

8.	Manifold Platforms Power Supply Options	91

8.1.	Intent & Basis	91

Abbreviations

The Following definitions and abbreviations have been used in this document.

Organisational

| BFCL | Brass Fertilizer Company Limited |
| --- | --- |
| EA | Epic Atlantic Limited |
| SPDC | The Shell Petroleum Development Company of Nigeria Limited |

Technical Terms

| BBOU | Bubuowei Bou |
| --- | --- |
| NUNR | Non River |
| SEIB | Seibou |
| ELEP | ELEP |
| OPUG | Opughene |
| T/L | Trunkline |
| B/L | Bulkline |

Units of Measurements

| km | Kilometre |
| --- | --- |
| m | Metre |
| M | Thousand |
| MM | Million |
| MMscfd | Million Standard Cubic Feet per Day |
| MTPD | Metric Tonnes Per Day |

Technical Terms Used for PIPESIM Simulation

| BBOU | Stream coming from Bubuowei Bou manifold |
| --- | --- |
| ELEP | Stream coming from Elepa Satellite manifold |
| ELEP MF | Stream coming from Elepa Field manifold |
| NUNR | Stream coming from Nun River manifold |
| NUNR MF | Stream coming from Nun River Field manifold |
| SEIB | Stream coming from Siebou manifold |
| OPUG | Stream coming from Opughene manifold |

- Administration

This Conceptual Study Memorandum is in respect of the SPDC-BFPC Upstream Gas Supply, Gathering, Evacuation and Processing (UGS-GEP) Project.  It stems from the Concept Select Studies, and presents the selected design concepts as basis for the Basic Design of the project.

It is also an input and accompaniment to the Basic Design Memoranda to be presented to DPR as statutory requirements in the process of securing regulators’ approval to execute the FEED / Detail Design that will lead to the project’s FID and implementation.

  - Validity & Change Management

The report has relied and used data and reference documents provided by client.  Some of the data are preliminary, based on limited information, but to a large extent, however, are adequate for this phase of work: - conceptual study and basic design.

For example, analogue PVT data was used in the absence of actual PVT, and preliminary pipeline distances were estimated based on desktop route selection study.  These design data, and the consequent simulation results will invariably need to be updated during FEED, by which time actual PVT data would have been obtained, and the FP survey done to establish actual distances and elevations of the pipeline routes.

Similarly, the project timelines as set out in the execution schedule was predicated on FID date of June 2018, and that will correspondingly change with actual FID date.

Also, the security situation in the region with its wide spate of vandalism may dictate that certain design provisions be changed during the FEED.  The situation will continue to be monitored, and if still perceived to be a cog in the wheel of innovation, some of the ideal designs will be replaced with what will be practically suitable for the situation.

For example, in as much as it is feasible to lay cable from the IPP to the remote manifold platforms to transmit electricity to motorise the platform valves in the Manifold Platforms Power Supply Options, or install solar systems on the platforms, there will be no need to implement such options if the facilities (cables & solar systems) will eventually be vandalised in no time.  Instead, it will be rather more appealing to limit the process option to manual valve operations, without the features of valve motorisation that will require electricity.

Epic Atlantic therefore reserves the right to update accordingly, the opinion set out in this report, as further information and / or refinement become available.

  - Background and Introduction

The SPDC-BFPC Upstream Gas Supply, Gathering, Evacuation and Processing (UGS-GEP) Project is constituted by Brass Fertilizer & Petrochemical Company Limited (BFPC), in partnership with The Shell Petroleum Development Company of Nigeria (SPDC).

The project scope includes Gas Gathering Pipelines, and a Gas Process Processing Plant that will provide processed gas as feedstock to a Petrochemical Plant and a Power Plant.  The Gas Processing Plant will process natural gas gathered from five SPDC fields – Bubouwe Bou, Elepa, Nun River, Seibou, and Opugbene - in OMLs 33, 32, & 36.

The Gas Plant will comprise three trains, (each with a processing capacity of 150 MMscfd) and condensate storage capacity of 50,000 Tonnes. In this respect, BFPC have concluded the Gas Supply & Processing Agreement (GSPA) with SPDC for 270 MMscfd of the natural gas feedstock.  The Plants (Gas Plant, Petrochemical Plant, and Power Plant) will all be located in the same complex at Akabel, Brass Island, Bayelsa state of Nigeria, near the shores of Atlantic Ocean.

The Petrochemical Plant will comprise a Methanol Plant of 1.75 MTPA capacity, and a Urea Plant of 2.6 MTPA total capacity, built in two phases of 1.3 MTPA each. It will also have storage capacity for 150,000 Metric Tonnes of Methanol and 100,000 Metric Tonnes of Urea.  The facilities complex will also comprise a captive export jetty of 4 berths for up to 35,000 DWT vessels.

The current on-stream date for the project is December 2020, and to achieve that, FID is planned for June, 2018.  In pursuit of these business objectives, BFPC have virtually concluded Project Management & EPC contracts with EIL, Saipem, and TCC, who are internationally renowned contractors in the implementation of the subject projects type.

The Gas Processing Plant, and the associated Gas Gathering System / Pipelines which will transport wellhead gas from the SPDC fields to the Gas Processing Plant, constitute the key upstream enabler facilities for the realisation of the objective supply of processed gas supply as feedstock to the Petrochemical Plant for the product manufacturing of methanol & urea, and to the Power Plant for power generation.

  - Scope of Document

The scope of the document covers the Conceptual Study and its evolution of results that will shape the basic design of the project along the following subjects:

- Project Design Data & Basis for Design Outline

- Project Concept and Site for the Project

- Feedstock Supply & Product Evacuation Scheme

- Pipeline Route Selection

- Process Simulation, - HYSYS Steady State, OLGA Dynamic Simulation, and PIPESIM

- Feedstock Flow Assurance

- Manifold Platforms Power Supply Options

- Manifolds Platform Concept & Integration with SPDC Field Manifolds

- Pipeline Material Selection & Corrosion Analysis

- Gas Processing Technology

- Safety Provisions & Preliminary Environmental Impact Assessment Study

- Infrastructural Support Strategy & Manning

- Decommissioning Philosophy

- Concept Select Drawings

- Project Design Data & Basis for Design Outline

Ref: TO-D-AA-00001-R01_Basis for Design

The Project Design Data & Basis for Design Outline as summarised here form the technical basis for the conceptual study, and subsequent development of the Basic Design and FEED for the project’s EPC, albeit updates that will arise following additional data acquisition or refinement.

  - Project Design Data

The key design data required & used are: Project Location Plan & Operational Conditions, the Natural Gas Conditions, Compositions, & Flow Rates, Production Forecast & Fluid Characteristics, and Environmental Conditions.

    - Project Location & Operational Conditions

The project location plan & operational conditions are summarized in the table below.

Table 2.1: Project Location Plan & Operational Conditions

| 1 | Location | OML 33 / 32 / 36 - with the fields widely spread & on both sides of Nun River |
| --- | --- | --- |
| 2 | Target Fields | BBOU, ELEP, NUNR, SEIB, & OPUG BBOU & ELEP in OML 33 and NUNR in OML 32, - East of Nun River SEIB in OML 32 and OPUG in OML 36, - West of Nun River |
| 3 | Desired Offtake Rate / GSPA Contractual Rate | 300 MMscfd / 270 MMscfd, starting at 110 MMscfd in Year 1, ramping up to 300 MMscfd in Year 4 |
| 4 | Gas Offtake Points | SPDC Field Manifold Headers at the target fields, in phases starting with BBOU, then ELEP, NUNR, SEIB, & OPUG |
| 5 | Feed Stream Inlet Pressure to BFPC P/L System | 80 – 100 barg |
| 5 | Feed Stream Outlet Pressure from BFPC P/L System to Gas Plant | 60 – 65 barg |
| 5 | Gas Plant Processed Gas Outlet Pressure | 50 – 55 barg |
| 6 | Condensate | 5 – 45 Mbpd |
| 7 | Water Production | Starting at 1.5 Mbpd in 2026, increasing to about 20 Mbpd in 2032 |

    - Production Forecast & Fluid Characteristics

The production forecast and fluid characteristics are summarized below.

Figure 2.1: Fields’ Gas Production Forecast – Facilities Design Data

Gas Composition

The gas compositions of the five fields are as summarised in the tables below:

Table 2.1: Fields’ Reservoir Compositions & Conditions – as received.

Table 2.1: Fields’ Reservoir Water-Saturated Compositions & Conditions – as constructed using HYSYS

    - Environmental Data

The environment is mangrove swamp and brackish water of resistivity assumed 1000 - 5000 ohm – cm, based on regional data.

- Environmental (Outdoor) Conditions

| Design Barometric Pressure | 1.013 bara |
| --- | --- |
| Design Ambient Temperature - Maximum | 42oC |
| Mean Maximum Ambient Temperature | 31oC |
| Design Ambient Temperature - Minimum | 18oC |
| Mean Minimum Ambient Temperature | 23oC |
| Relative Humidity | 100% |
| Thermal Insulation Design Criteria | 23oC at 100% Humidity |
| Black Bulb Temperature | Up to 80oC |
| Ground Temperature | 25 – 28 oC |
| Site Altitude | Sea Level |
| Lightning | Frequent in Rain |
| Average Annual Rainfall | 3,800 in |
| Mean Maximum Hourly Rainfall | 100 in |
| Design Wind Speed | 36 m/s (130 km/hr) |
| Corrosion Environment | Brackish Swamp/Waters (50 -100 ohm-cm) |

- Indoor Conditions

The control equipment and HV/LV-boards will be installed in air-conditioned non-hazardous control rooms, with environmental conditions as follows:

| Ambient Air Temperature | 20 – 22 oC |
| --- | --- |
| Relative Humidity | 50 – 75 % |

Electronic Equipment (e.g. signal converters / barriers) shall also be capable of operating at a temperature of at least 75°C.

Equipment shall also be capable of operating at a room temperature of 55°C to cover shutdown of air conditioning.

The operator stations will be installed in an air-conditioned, non-hazardous control room with conditions as follows:

| Ambient Air Temperature | 21 + 3 oC |
| --- | --- |
| Relative Humidity | 50 – 75 % |

Environment / Resistivities  Vs Corrosion Current per m2

The assumption made for the current density  for bare steel metal areas for this desk top calculations is 22 mA / m2 at the assumed resistivity of 1000 – 5000 ohm-cm of the swampy, brackish water  environment closed to the  brass sea

Coat Efficiency

Coat efficiency, Mean Coating Breakdown and coating Breakdown at end 	of life time

  - Basis for Design Outline

The key BFD elements are summarised in the fishbone diagram below, and the matrix that follows.

Figure 2.1: Fields’ Gas Production Forecast – Facilities Design Data

Table 2.2: BFD Matrix

|  | Situations & Systems |  |  | Design Basis |  | Remark / Reference |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | Location |  |  |  |  |  |  |
|  | 1.1 | Gas Source |  |  | OML 33 / 32 / 36 - with Locations Widely Spread & on Both Sides of Nun River |  |  |
|  |  | 1.1.1 | Target Fields |  | BBOU, ELEP, NUNR, SEIB, & OPUG |  |  |
|  |  |  |  |  | BBOU & ELEP in OML 33 and NUNR in OML 32, - East of Nun River |  |  |
|  |  |  |  |  | SEIB in OML 32 and OPUG in OML 36, - West of Nun River |  |  |
|  | 1.2 | Gas Gathering scheme |  |  | Nodal Raw Gas Gathering thru Pipeline Network to ELEP, and from there to Akabel |  |  |
|  |  | 1.2.1 | Receiving Nodes |  | Three Nodes - ELEP, BBOU, & NURN Node - geographically convenient |  |  |
|  |  |  |  |  | NUNR & SEIB Production to NUNR Node S-M/F - geographical convenience |  |  |
|  |  |  |  |  | BBOU & OPUG Wells Production to BBOU Node S-M/F - geographical convenience |  |  |
|  |  |  |  |  | ELEP Wells Production, BBOU Node Inventory, & NUNR Node Inventory to ELEP Node E-M/F - geographyical convenience |  |  |
|  |  | 1.2.2 | Evacuation Node |  | ELEP Manifold |  |  |
|  |  |  |  |  | Inventory from ELEP Node E-M/F to AKAB PLEM, and from there to the Gas Plant |  |  |

|  |  | Situations & Systems |  |  |  | Design Basis |  | Remark / Reference |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 |  | Location (Contd.) |  |  |  |  |  |  |  |
|  |  | 1.3 |  | Gas Gathering System |  |  | A mix of Wellhead Flowlines, Field Manifolds, and Bulk Transport Lines, - optimally configured in terms of cost, layout, and choice of evacuation manifold (Bubouwe Bou or Elepa) |  |  |
|  |  |  |  |  |  |  | Wellhead Flowlines Gather Each Field Production into the Central Field Manifold (F-MF) within the Field, followed by: |  |  |
|  |  |  |  |  |  |  | Transportation of the F-MF inventory through Bulkline to Satellite Manifold (S-MF) / Evacuation Manifold (E-MF). |  |  |
|  |  |  |  |  |  |  | Transportation of the S-MF inventory through Bulklines to the E-MF |  |  |
|  |  |  |  |  |  |  | Transportation of the E-MF inventory through a Trunkline to the Gas Processing Plant. |  |  |
|  |  | 1.4 |  | The Gas Processing Plant |  |  | The Gas Plant will be located at Akabel, Brass Island, Bayelsa State of Nigeria, near the shores of Atlantic Ocean |  |  |
|  |  |  |  |  |  |  | The Plant Site is in the same complex with the Petrochemical Plant and the Power Plant that it will be supplying the processed gas |  |  |
| 2 | Business Case |  |  |  |  |  |  |  |  |
|  | 2.1 |  | Justification |  |  |  | To supply processed gas needed as feedstock to Petrochemical Plant and a Power Plant, - in the value chain for methanol & urea production by the petrochemical plant, and power generation by the power plant |  |  |
|  |  |  |  |  |  |  | GSPA with SPDC in place for the Supply of the Natural Gas to be Processed |  |  |

|  | Situations & Systems |  |  |  | Design Basis | Remark / Reference |
| --- | --- | --- | --- | --- | --- | --- |
| 2 | Business Case (Contd.) |  |  |  |  |  |
|  | 2.2 |  | GSPA |  | Wellhead (Raw) Gas at Each of the Fields' Manifolds for offtake by BFPC through own Bulklines |  |
|  |  |  | 2.2.1 | Gas | 270 MMscfd |  |
|  |  |  | 2.2.2 | Supply Pressure | 80 - 100 barg |  |
|  |  |  | 2.2.3 | Condensate | 5 - 45 Mppd |  |
|  | 2.3 |  | On-Stream Sequence |  | Phased Sequential (Production) Injection: BB ELEP + NUNR + SEIB + OPUG in that order, - with stop points driven / moderated by the GSPA |  |
| 3 | Gas Offtake / Flow Conditions |  |  |  |  |  |
|  | 3.1 | Flow Rates / Fluid Characteristics |  |  |  | 5- 45Mbopd of Condensate Also water production will occur later, starting at 1.5 Mbpd in 2026 increasing to about 20 Mbpd in 2032 Fluid therefore to be considered two phase since no treatment will be done before injection into the Line, - prompting design of the Pipeline for two-phase Handling Two Phase Gas Transportation all the way to the Gas Processing Plant at Akabel Consequence is that Frequent Pigging will be needed to keep the Lines operating |
|  |  | 3.1.1 |  | Gas Production | 300 MMscfd Peak (Start 110 in Year 1, Ramping up to 300 in Year 4) |  |
|  |  |  |  | Condensate Production | Up to 45,000 bpd |  |
|  |  |  |  | Water Production | Up to 350 bpd from the water content of the Natural Gas |  |
|  |  |  |  |  | Up to 20 Mbwpd reservoir production, starting later in the fields' life |  |
|  |  |  |  | CGR | 4 - 36 for BBOU, 62 - 158 for the rest |  |
|  |  | 3.1.2 |  | Gas SG | 0.6 - 0.74 (Air = 1) |  |
|  |  |  |  | Condensate SG | 0.74 |  |
|  |  |  |  | Water SG | 1.00 - 1.02 |  |
|  |  | 3.1.3 |  | Corrosivity Potential | Envisaged, on account of CO2 in the gas stream |  |
|  |  | 3.1.4 |  | Souring Potential | Not Envisaged, - since no H2S in the gas stream |  |
|  |  | 3.1.5 |  | Foaming Potential | Not Envisaged |  |
|  |  | 3.1.6 |  | Scaling Tendencies | Not Envisaged |  |

|  | Situations & Systems |  |  |  |  | Design Basis | Remark / Reference |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 3 | Gas Offtake / Flow Conditions (Contd.) |  |  |  |  |  |  |
|  | 3.2 | Pressure Regime |  |  |  |  | Provision to accommodate higher pressure (up to 110 barg) at the distant field manifolds of Seibou & Opugbene. |
|  |  | 3.2.1 |  | Feed Stream Inlet Pressure |  | 80 - 100 barg |  |
|  |  | 3.2.2 |  | Feed Stream Outlet Pressure |  | 60 - 65 barg |  |
|  | 3.3 | Product Metering |  |  |  | The NAG facility shall be provided with metering devices for gas and condensate flows. These meters shall be allocation type meters. All the instruments provided shall meet the Hazardous area requirements for field instruments |  |
|  |  | 3.3.1 |  | Raw Gas Metering |  | Conditioning Orifice Metering at the Custody Transfer Points at the Field Manifolds |  |
|  |  | 3.3.2 |  | Feed Stream Outlet Pressure |  | 60 - 65 barg |  |
| 4 | Design Philosophy |  |  |  |  |  |  |
|  | 4.1 |  | Fitness for Purpose & Functionality |  |  | Driven by an understanding of what constitutes a good (an excellent) design that will deliver technical & operational excellence; - requiring a good knowledge of how humans interact within the work system |  |
|  |  |  | 4.1.1 |  | Human Factors Engineering (Ergonomics) | Inclusion of User Requirement through: - Consideration to human capabilities, limitations and needs in the interaction between humans, technology and the working environment. - Equipment, Valves, Controls & Accessories Layout, degree of automation, and extent of protective systems. | Operability and Maintainability of the System to be given priority, considering the perculiarities of geographical locations of the Facilities |
|  | 4.2 |  | Mode |  |  | Not normally manned facility, with Automatic Shutdown Capability | Automatic Shutdown to be Fail Safe Consideration to the use of Self Contained Fail Safe Shutdown System that Requires Human Intervention for Reopening |
|  |  |  | 4.2.1 |  | Automation | Remote Shutdown Capability from a PLC based Control Room. |  |
|  |  |  |  |  |  | Remote start-up capability with local permissives from the Field M/F & PLEM, Either as a provision, or intended start-up process for the P/L | This will require the Installation of Patch Panels & Fieldbuses. For such a small installation, it will be more economical to keep automation as simple as possible |
|  |  |  |  |  |  | Local Control Room, with HMI |  |
|  |  |  |  |  |  | PLC with SCADA capability |  |
|  | Situations & Systems |  |  |  |  | Design Basis | Remark / Reference |
| 4 | Design Philosophy (Contd.) |  |  |  |  |  |  |
|  | 4.3 |  | Governing Regulations |  |  | Oil Pipelines Act, Chapter 338, Laws of the Federation of Nigeria 1990 DPR Guidelines for Installation of Gas Plant |  |
|  |  |  | 4.3.1 |  | Regulatory Approvals | PTS (Permit To Survey) OPL (Oil Pipeline Licence) EIA (Environmental Impact Assessment Study) Staged Approvals contained in DPR Guidelines for Gas Plant Facility and Oil & Gas Pipeline Systems |  |
|  |  |  | 4.3.2 |  | Refence DPR Guidelines | Guidelines for the Establishment of Natural Gas Plant Facility in Nigeria DRP P-1P: Guidelines & Procedures for the Design, Construction, Operation and Maintenance of Oil & Gas Pipeline Systems in Nigeria - 2007 |  |
|  | 4.4 |  | Codes & Standards |  |  |  | ISO 13623:2009 § BS EN 14161:200 - § 5.2 Shell DEP 31.40.00.10-Gen - $ 2.1 FLIUD IS TO BE CONSIDERED TWO PHASE SINCE NO TREATMENT WILL BE DONE BEFORE INJECTING INTO THE LINE Applicable Design Code for Category D and E fluids, ref: Shell DEP 31.40.00.10-Gen ISO 13623:2009 / BS EN 14161:200 |
|  |  |  | 4.4.1 |  | Fluid Type (& Category) | Gas (Category D / E) |  |
|  |  |  |  |  |  | Category D: Non-toxic, Single-phase Natural Gas |  |
|  |  |  |  |  |  | Category E: Flammable, and/or toxic fluids which are gases at prevailing ambient temperature and atmospheric pressure conditions and are conveyed as gases and/or liquids. Typical examples are Hydrogen, Natural Gas (not otherwise covered in Category D), Ethane, Ethylene, LPG (such as Propane & Butane), Natural Gas Liquids, Ammonia and Chlorine. |  |
|  |  |  | 4.4.2 |  | Applicable Design Code | ANSI / ASME 31.8 - Pipeline & Field Manifolds ANSI / ASME 31.3 - PLEM |  |
|  |  |  | 4.4.3 |  | Specification Breaks | ANSI / ASME 31.3 - at the Gas Plant Fence, as PLEM is within the Gas Plant Fence; as well as athe Manifold Platforms |  |

|  | Situations & Systems |  |  | Design Basis | Remark / Reference |
| --- | --- | --- | --- | --- | --- |
| 5 | Pipeline Design |  |  |  |  |
|  | 5.1 | Design Provisions |  | Pipelines, Node M/Fs and PLEM |  |
|  |  | 5.1.1 | Features | Coated Linepipe / Field Joints Coating - for external corrosion protection | - 3 Layers PE (Poly Ethylene) Main Line Coating - GTS 65 Field Joints Coating |
|  |  |  |  | Concrete Coating of River Crossing Sections to Combat Buoyancy |  |
|  |  |  |  | Pig Launcher / Receiver with Capability for Intelligent Pigging |  |
|  |  |  |  | Provision for Chemical Injection Facilities on the Manifold Platforms - for Corrosion Inhibition Administration to Mitigate Internal Corrosion |  |
|  |  |  |  | Pipeline End Manifolds on Manifold Platforms at Gas Gathering Nodes |  |
|  |  |  |  | Provision for Power Supply & Related Power Distribution Ancillaries at the Manifold Platforms that will enable powered equipment operations / automation, and data transmission for monitoring of the pipeline. | Concept selected among other options - Solar Power & Indivudual Power Generator options at Each Manifold |
|  |  |  |  | Cathodic Protection System to Combat External Corrosion | Inmpressed Current |
|  |  |  |  | Sectionalisation Valves at River Crossings / at 30 km Intervals | ASME Requirement |
|  |  | 5.1.2 | Retrofit Design | Retrofit Design on Node Manifold Platforms in the light of sequential injection |  |
|  |  | 5.1.3 | Power Supply Options | Power Plant at Akabel Base to generate electricity for the base operations, and export to the Pipelines Manifold Platforms |  |
|  |  | 5.1.4 | Control & Custody | The automated block valves will be remotely controlled from BFPC Control Centre. Custody tranfer metering will be installed at each of the SPDC M/Fs |  |
|  |  | 5.1.5 | Health & Safety | Design will provide for healthy and safe work environments for operation & maintenance. |  |
|  |  | 5.1.6 | General Environmental Responsibility | Protective devices and containment will be installed at the facilities; mainline valves will be remotely operable where practical |  |

|  | Situations & Systems |  |  | Design Basis | Remark / Reference |
| --- | --- | --- | --- | --- | --- |
| 5 | Pipeline Design (Contd.) |  |  |  |  |
|  | 5.2 | Line Sizing |  | To Meet Throughput, Pressure & Velocity Boundary Conditions |  |
|  |  | 5.2.1 | Inlet Pressure | 80 - 100 barg (1,160 - 1,450 psi) | Delivery Pressure Available at CTP |
|  |  | 5.2.2 | Outlet Pressure | 60 - 65 barg (870 - 942 psi) | High enough to satisfy the requirements of the Gas Plant |
|  |  | 5.2.3 | Pressure Drop | Applicable Equation: - Panhandle Equation (Long Distance, Large Diameter Pipelines) |  |
|  |  |  |  | Steady State Hydraulic Simulation done on HYSYS, & PIPESIM |  |
|  |  |  |  | Dynamic Hydraulic Simulation done on OLGA |  |
|  |  |  |  | Line Sizing Cosscheck with Panhandle Equation |  |
|  |  | 5.2.4 | Velocity Considerations | Minimum Velocity to prevent surging, and to keep line swept clear of entrained solids & liquids Maximum Velocity that will not cause Erosion, Excess Noise, or Water Hammer |  |
|  |  | 5.2.5 | Velocity | 5 - 20 m/s: subject to Errosional Velocity Limit | API RP 14E |
|  |  | 5.2.6 | Minimum Velocity | High enough to prevent surging, and to keep line swept clear of entrained solids & liquids | Too low a velocity leads to settlement of water, which may lead to bottom of pipe internal corrosion |
|  |  | 5.2.7 | Maximum Velocity | Low enough to avoid erosion, excess noise, or water hammer | Too high a velocity can increase the overall corrosion rate and also destroy any protective scale or inhibitor films. |

|  | Situations & Systems |  |  | Design Basis | Remark / Reference |
| --- | --- | --- | --- | --- | --- |
| 5 | Pipeline Design |  |  |  |  |
|  | 5.3 | Pipe Wall Thickness |  | To Meet Design Code, Pipe Material & Grade, Fluid's Pressure, Chemical Composition, Velocity, & Velocity Boundary Conditions |  |
|  |  | 5.3.1 | Fluid Category D / E | Category D: Non-toxic Single-phase Natural Gas Category E: Flammable fluids which are gases at prevailing ambient temperature and atmospheric pressure conditions and are conveyed as gases and/or liquids. Typical examples are Natural Gas (not otherwise covered in Category D), Ethane, Ethylene, LPG (such as Propane & Butane), Natural Gas Liquids. | ISO 13623:2009 Shell DEP 31.40.00.10-Gen |
|  |  | 5.3.2 | Location Class | Class 1 / Class 2 Locations | 5 - 30 residential buildings per km |
|  |  | 5.3.3 | Design Factor | Geographical 0.72, Crossings without Casings 0.6 |  |
|  |  | 5.3.4 | Material & Grade | API X65 |  |
|  | 5.4 | Pressure Rating |  | To Meet ANSI Standard B16.5 / API Spec 6A |  |
|  |  | 5.4.1 | Pressure Class | ANSI 900 (Class 900) |  |
|  | 5.5 | Maintenance Provisions |  |  |  |
|  |  | 5.5.1 | Corrosion Management | Provision will be made for corrosion inhibitor injection to mitigate corrosion of the pipeline | The Gas is potentially corrosive on account of its CO2 content above threshhold for corrosion; and water, NGL, & particulates that are inherentltly present |
|  |  |  |  | Provision will also be made for cleaning and in-line inspection tools, shop, and in-situ maintenance |  |
|  |  | 5.5.2 | Condition Monitoring | UT Scanning |  |
|  |  |  |  | Intelligent Pigging |  |
|  |  |  |  | Leak Detection will be installed for pipelines and facilities |  |

|  | Situations & Systems |  |  | Design Basis | Remark / Reference |
| --- | --- | --- | --- | --- | --- |
| 6 | Pipeline Routing |  |  |  |  |
|  | 6.1 | Route Selection |  | Desk-top Exercise, pending PTS / detailed survey that will enable Final Route Selection |  |
|  |  |  |  | Route Optimised in the Light of Environmental Impact / Impact to Settlements |  |
|  |  |  |  | Consideration given to Economic Distances / Workable Routes |  |
|  |  |  |  | Current Concept for BBOU / ELEP is: 16" x 9km from BBOU - ELEP, and 24" x 28.5 km from ELEP - AKAB, running through Southern Ijaw, Nembe, & Brass LGAs |  |
|  |  |  |  | Alternative of 42 km predominatly offshore route, aimed at minimising communities traverse was considered, but dropped, as it did not achieve that objective, in addition to the intelligence report that it will not be acceptable to NIWA. | Further the alternative offshore route will mean higher installation, and risk of hydrate formation for the lower ambient temperature |
|  | 6.2 | Pipeline ROW |  | Desk-top Exercise, pending PTS / detailed survey that will enable Final Route Selection |  |
|  |  | 6.2.1 | Width | 30 m - (15 m for P/L, including FOC, and 15m provision for Construction equipment & activities |  |
|  |  | 6.2.2 | Route Excavation | Propose to excavate route as 1 - 1.5 m deep trench that will be backfilled |  |
|  |  |  |  | The alternaive of open ditch (3m width X 4m depth), was considered in the light than it will be permanently flooded and will therefore pose as deterrent against vandalisation, - but was dropped as that objective is uncertain to be achieved. | Further the alternative open ditch canal option will mean higher installation cost that will include additional equipment and weight (concrete) coating of linepipes |
|  |  | 6.2.2 | Crossings | Crossings of Existing pipelines, Cables, Power Lines, Roads, and Waterways to be maintained at an angle between 60 & 90 degrees |  |
| 7 | Flow Assurance |  |  |  |  |
|  | 7.1 | Essence |  | Process conditions should be such that upstream pressure is sufficient to deliver fluid to destination, as well as avoid precipitation & deposition of solids |  |

|  | Situations & Systems |  |  |  |  | Design Basis | Remark / Reference |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 7 | Flow Assurance (Contd.) |  |  |  |  |  |  |
|  | 7.2 |  | Fluid Flow |  |  | Hydraulic Flow, on own Pressure | Hyraulic Analysis to be performed to establish range of operational parameters of the Pipeline |
|  |  |  | 7.2.1 | Temp - Pressure Relationship |  | T & P to be maintained below equilibrium line for hydrate formation, otherwise chemical (antifreeze) injection |  |
|  |  |  | 7.2.2 | Gas Hydrate |  | Need for the Fluid Traverse to be maintained outside Hydrate Formation Envelop |  |
|  |  |  | 7.2.3 | Dew Point Control |  | Dew Pointing & Hydrate Handling in respect of the Gas Processing will be addresses at the Gas Plant FEED |  |
| 8 | Pipeline Materials Selection & Corrosion Analysis |  |  |  |  |  |  |
|  | 8.1 | Drivers |  |  |  | Internal & External Corrosion Predictions and Design Life, Fitness & Compatibility |  |
|  | 8.2 | Essence |  |  |  | Specification of Materials that can withstand the physical conditions and application, as well as work together |  |
|  | 8.3 | Situations |  |  |  | High Pressure Transportation of Category D/E Fluid (flammable fluid), in the pressure region of 100 barg, and temperature region of 60 oC |  |
|  | 8.4 | Components of Concern |  |  |  | Obnoxious Components of Concern are: CO2, H2S, Mercury, and Naturally Occurring Radioactive Materials (NORM) |  |
|  |  | 8.4.1 |  | CO2 |  | CO2 content is up to 2%, which for the empirically established 0.1 bar partial pressure treshhold for significant CO2 corrosion, implies corrosion of unprotected mild CS at pressures above 5 barg | DeWaard-Milliams CO2 Corrosion Nomogram |
|  |  | 8.4.2 |  | H2S |  | Nil in the gas streams, - hence no sour gas corrosion to contend with |  |
|  |  | 8.4.3 |  | Mercury |  | No Mercury Present , - to cause mercury amalgamation and embrittlenes of alumunium and other metals |  |
|  |  | 8.4.4 |  | NORM |  | No NORM (Naturrally Occuring Radioactive Materials), - Radon, Radium, - which can accumulate within P/L & Process Eqpt to render them radioactive over time |  |
|  |  | Situations & Systems |  |  |  |  | Design Basis | Remark / Reference |
| 8 |  | Pipeline Materials Selection & Corrosion Analysis (Contd.) |  |  |  |  |  |  |
|  | 8.5 | Issues & Consideration |  |  |  | The CO2 content of up to 2%, renders the gas corrosive at pressures higher than 5 barg (corresponding to to the empirically established 0.1 bar partial pressure treshold for significant CO2 corrosion), - prompting the need for CRA materials or matching corrosion management design | Internal Corrosion Issue |
|  |  | 8.5.1 |  | Corrosivity Potential of the Transported Fluid (Gas) |  | The Gas is potentially corrosive on account of its CO2 content; and water, NGL, & particulates that are inherentltly present | Assessments Based on: - HydroCor Softeware and External Resistivity Analysis - DeWaard-Milliams CO2 Corrosion Nomogram |
|  |  |  |  | Corrosivity Potential of the Ambient Environment |  | Humidity, Acidity, and Oxygen in Air, and known potentially corrosive nature of the soil in the area jointly contribute to render the ambient environment corrosive | External Corrosion Issue |
|  |  | 8.5.2 |  | Internal Corrosion Design |  | Materials Selection - NACE Codes, Consideration for MS (WT), MS (WT)+CA, MS+Inhibitor, MS with CRA |  |
|  |  | 8.5.3 |  | External Corrosion |  | Coating Selection and Painting - NACE Code | Main Line Coating with 3 Layers PE (polyethylene) Field Joint Coating with GTS 65 |
|  |  |  |  |  |  | CP Design NACE Code IP |  |
|  |  |  |  |  |  | T/R, Junct Boxes, Cabling, (IAC Code), Groundbed design, Electrical Seperation |  |
|  | 8.6 | Line Pipe Selection |  |  |  | API 5L X65 Mild Carbon Steel | Duplex Stainless Steel for the Flowlines & Field Manifold Headers on the Manifold Platforms |
|  |  |  |  |  |  | High Yield Strength (a favourable trade-off with required wall thickness) that can accommodate high corrosion allowance that will be needed |  |
|  | 8.7 | Corrosion Allowance |  |  |  | Up to 10 mm, and will be supported with Corrosion Inhibition |  |

|  | Situations & Systems |  |  | Design Basis | Remark / Reference |
| --- | --- | --- | --- | --- | --- |
| 9 | Manifold Platforms |  |  |  |  |
|  | 9.1 | Concept |  | Integrated Field Manifold Platform for both SPDC Field Manifold Header & BFPC Bulklines |  |
|  | 9.2 | The Manifold Platform |  | Comprises Field Manifold Header, Incoming & Outgoing Bulklines and Scraper Barrels, Chemical Injection System, Electrical Transformer, & Electric Room |  |
|  |  | 9.2.1 | B/L Pressure Rating | ANSI 900 | ANSI 1500 for the Well Flowlines |
|  |  | 9.2.2 | Provision for Pigging | The Pipeline Manifold equipped with Pigging Facilities (Pig Launcher / Receiver) for pigging / pipe cleaning that will be done during precommissioning, commissioning & decommissioning of pipeline, and routine cleaning and condition monitoring of the pipeline during its operations |  |
|  |  | 9.2.3 | Valving | Valving arranged and labelled so that they can be acessed safely and efficiently; the required valve can be visually identified and distinguished without error; the information required for decision making is provided; and the physical and mental workload required remains within reasonable limits. Task demands should not exceed operator capbilities |  |
|  |  |  |  | IVA (Identification Valve Analysis) to ensure that all valve configurations are well designed from an operational task performance perspective, and to avoid design misfits in category 1 valves (critical). A key objective is to ensure that valves are properly categorised and prioritised at an early stage in design. |  |
|  |  |  |  | Valve Interlock System |  |
|  |  |  |  | Unrestricted Access for Valves used for emergency operations (i.e. Category 1 valves) |  |
|  |  | 9.2.4 | Manifold Platform Property Boundary | Will be Fenced / Caged to Ward off Intrusion |  |
|  |  | 9.2.5 | The Pig Barrel Trap | Horizontal Pig Trap, wiith capability for intelligent pigging |  |
|  |  | 9.2.6 | Pressure Rating | ANSI 900 |  |
|  |  | 9.2.7 | Design Code | ASME 31.8 for the Infield M/F Pig Traps, and ASME Section VIII for Pig Trap EnclosureE ASME 31.3 for the PLEM Piping |  |
|  | Situations & Systems |  |  | Design Basis | Remark / Reference |
| 9 | Manifold Platforms (Contd.) |  |  |  |  |
|  | 9.2 | The Manifold Platform (Contd.) |  |  |  |
|  |  | 9.2.8 | Corrosion Allowance | Yes, - as raw gas contains CO2 at threshhold concentration for Corrosion |  |
|  |  | 9.2.9 | Double Block & Bleed | Two separate valves in series with a bleed (vent) point in between to allow diversion to safe location of any fluid leaking through either valve |  |
|  |  | 9.2.10 | End Closure | End Closure pointing away from personnel areas and critical items of equipment, in the event of pig being ejected from the pig trap | Consideration Given TO ADDITIONAL SAFETY MEASURES LIKE USING INTERLOCKING KEY SYSTEM TO OPEREATE THE PIG TRAP SYSTEM. |
|  |  | 9.2.11 | Valving | Valving arranged and labelled so that they can be acessed safely and efficiently; the required valve can be visually identified and distinguished without error; the information required for decision making is provided; and the physical and mental workload required remains within reasonable limits. Task demands should not exceed operator capbilities | PIPELINE VALVES TO BE DESIGNED AS PER ASME B16.34 AND ASME B16.10 |
|  |  |  |  | IVA (Identification Valve Analysis) to ensure that all valve configurations are well designed from an operational task performance perspective, and to avoid design misfits in category 1 valves (critical). A key objective is to ensure that valves are properly categorised and prioritised at an early stage in design. |  |
|  |  |  |  | Valve Interlock System |  |
|  |  |  |  | Unrestricted Access for Valves used for emergency operations (i.e. Category 1 valves) |  |

|  | Situations & Systems |  |  |  | Design Basis | Remark / Reference |
| --- | --- | --- | --- | --- | --- | --- |
| 10 |  | The Gas Plant Design |  |  |  |  |
|  |  | 10.1 | Basic Concept |  | Full Spectrum Gas Plant with Capability for NGL (Condensate) Extraction |  |
|  |  |  | 10.1.1 | Features | Inlet Gas Separation Unit |  |
|  |  |  |  |  | Condensate Stabilisation |  |
|  |  |  |  |  | Gas Drying and / or Dehydration |  |
|  |  |  |  |  | Gas Chilling & Refrigeration |  |
|  |  |  |  |  | Turbo Expander for NGL (Condensate) Recovery |  |
|  |  |  |  |  | Flexibility for NGL Fractionating |  |
|  |  |  |  |  | Condensate / NGL Storage & Export Facilities |  |
|  |  |  |  |  | Utilities |  |
|  |  | 10.2 | Design Provisions |  |  |  |
|  |  |  | 10.2.1 | Features | Provision for Power Supply & Related Power Distribution Ancillaries that will enable powered equipment operations / automation, and data transmission for monitoring of the Plant performance. |  |
|  |  |  |  |  | Earthing Ring Main |  |
|  |  |  | 10.2.2 | Retrofit Design | Retrofit Design for Booster Compression |  |
|  |  |  | 10.2.3 | Power | Power Plant at Akabel Base to generate electricity for the Gas Plant operations |  |
|  |  |  | 10.2.4 | Control & Custody | The automated block valves will be remotely controlled from BFPC Control Centre. Custody transfer metering will be installed at each of the SPDC M/Fs |  |
|  |  |  | 10.2.5 |  | The automated block valves will be remotely controlled from BFPC Control Centre. Custody transfer metering will be installed at each of the SPDC M/Fs |  |

|  | Situations & Systems |  |  |  |  |  | Design Basis | Remark / Reference |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 10 |  | The Gas Plant Design (Contd.) |  |  |  |  |  |  |
|  |  | 10.2 |  | Design Provisions (Contd.) |  |  |  |  |
|  |  |  |  | 10.2.6 |  | Maintenance Provision | Provision will be made for corrosion inhibitor injection to mitigate corrosion of the pipeline | The Gas is potentially corrosive on account of its CO2 content above threshhold for corrosion; and water, NGL, & particulates that are inherentltly present |
|  |  |  |  | 10.2.7 |  |  | Provision will also be made for cleaning and in-line inspection tools, shop, and in-situ maintenance |  |
|  |  |  |  | 10.2.8 |  | Health & Safety | Design will provide for healthy and safe work environments for operation & maintenance. |  |
|  |  |  |  | 10.2.9 |  | General Environmental Responsibility | Protective devices and containment will be installed at the facilities; mainline valves will be remotely operable where practical |  |
| 11 |  |  | Process Safety Provision & Preliminary EIA Study |  |  |  |  |  |
|  |  |  | 11.1 |  | Process Safety Provision |  | Design Provision ranges from Overpressure Protection, Fire & Gas, Product Leak Detection, and Shutdown | Provision with Safeguarding (ESD) requirement in accordance to Shell standards. |
|  |  |  |  |  | 11.1.1 | Pipeline Overpressure Protection System | A High Integrity Pressure Protection System (HIPPS) shall be provided for the protection of the Bulklines & Manifold Platforms |  |
|  |  |  |  |  |  |  | PSVs will be installed on the pig traps to provide additional safety especially during pigging where it pups up and closes to maintan pressure balance |  |
|  |  |  |  |  |  |  | A complete blowdown / evacuation of the Pipeline will be required, and can only be achieved through Akabel where it will be possible to install such facility |  |
|  |  |  |  |  |  |  | On the other hand, for maintenance purposes, the Spec requires sectionalisation valves to be installed at pre-determined spacings on the Pipeline in order to reduce the volume of inventory to be evacuated in case damage to any section of the Pipeline. This will require the installation of small venting systems around the Sectionalisation Valve Stations which include of course the PLEM to vent the sections of the Pipeline requiring maintenance |  |

|  | Situations & Systems |  |  | Design Basis | Remark / Reference |
| --- | --- | --- | --- | --- | --- |
| 11 | Process Safety Provision & Preliminary EIA Study (Contd.) |  |  |  |  |
|  | 11.1 | Process Safety Provision (Contd.) |  |  |  |
|  |  | 11.1.2 | Fire & Gas | Fire and Gas system signals will be implemented into the Pipelines & Gas Plant safeguarding system |  |
|  |  |  |  | A fire & gas mimic panel will be installed within the control room indicating main F&G detectors/initiators / F & G loops / push button, etc. | The Mimic Panel will be designed in accordance with Shell DEP and SPDC guidelines |
|  |  |  |  | Mimic panel will be operated from the SIS system with dedicated SIS input & outputs to final indication lamps. Horns acknowledge button, lamp test button, ESD button and reset button for activated lamps will also be included on the mimic panel. |  |
|  |  | 11.1.3 | Product Leak Detection | ROW Inspection by Line Patrols |  |
|  |  |  |  | Hydrocarbon Sensing (of External Leaks) via Fibre Optic / Dielectric Cable Cable |  |
|  |  | 11.1.3 | Shutdown | To be initiated as a last resort of the Process Safeguarding System |  |
|  | 11.2 | Shutdown Philosophy |  | Four types of shut downs, viz: - ESD, OSD, Single Train Shutdow, and Unit Shutdown, - will be designed to be initiated depending on process upsets and their hazardous levels. | The process operating, control and start-up / shutdown philosophy to be detailed at FEED |
|  |  | 11.2.1 | Emergency Shutdown System (ESD) | The ESD will be designed to be initiated by fire, manual push buttons or by loss of instrument air supply |  |
|  |  | 11.2.2 | Operational Shutdown (OSD) | The Operational Shut Down (OSD) will cut off the fluid supply from the pipelines as well as block the gas supply to the Petrochemical & Power Plants, albeit that an ESD will always initiate the OSD |  |
|  |  | 11.2.3 | Single Train Shutdown | The single train shutdown will cut off all fluid supply and discharges from a single equipment train allowing the parallel equipment train(s) to continue operation |  |

|  | Situations & Systems |  |  | Design Basis | Remark / Reference |
| --- | --- | --- | --- | --- | --- |
| 11 | Process Safety Provision & Preliminary EIA Study (Contd.) |  |  |  |  |
|  | 11.2 | Shutdown Philosophy (Contd.) |  |  |  |
|  |  | 11.2.4 | Unit Shutdown (USD) | The Unit Shutdown will stop a dedicated unit of the plant |  |
|  |  |  |  | On the other hand, for maintenance purposes, the Spec requires sectionalisation valves to be installed at pre-determined spacings on the Pipeline in order to reduce the volume of inventory to be evacuated in case damage to any section of the Pipeline. This will require the installation of small venting systems around the Sectionalisation Valve Stations which include of course the PLEM to vent the sections of the Pipeline requiring maintenance |  |
|  | 11.3 | Hazardous Area Classification (HAC) |  | Will be in compliance with Shell DEP 80.00.10.10-Gen in cross reference with API RP 505 and IP 15 | Shell DEP 80.00.10.10-Gen |
|  |  | 11.3.1 | Zones Classification | Zone 1 - where a flammable atmosphere is continuously present for long periods Zone 2 - where a flammable atmosphere is likely to occur in normal operation Zone 3 - where a flammable atmosphere is not likely to occur in normal operation and if it occurs it will exist only for short period |  |
|  |  | 11.3.2 | Extent of Zone | The standard 6m Radius will be used as general guide in this Basic Design scope, while specifics & details will be covered at FEED |  |
|  |  | 11.3.3 | Location of Equipment | Will be done with the consideration to avoid ignition of flammable hydrocarbons by minimising the probability of coincidence of a flammable atmosphere and a source of ignition. |  |
|  |  |  |  | In general will be more than 3m away from the launcher and receiver barrels which are the major points of release on the manifold. |  |

|  | Situations & Systems |  |  |  | Design Basis | Remark / Reference |
| --- | --- | --- | --- | --- | --- | --- |
| 11 | Process Safety Provision & Preliminary EIA Study (Contd.) |  |  |  |  |  |
|  | 11.4 | Preliminary EIA Study |  |  | An environmental impact assessment is currently in progress for the gas plant site, and will also be carried out for the Pipeline Route and associated Manifold Platforms |  |
|  |  |  |  |  | Will cover the entire Plant Complex (Gas Plant, Power Plant, and the Petrohemical), the Pipeline Route, and the associated Manifold Platforms |  |
|  |  |  |  |  | Only the phase-1 Pipeline Route (BBOU - ELEP - AKAB) will be covered now. The other routes will be covered later, close to their on-stream dates, otherwise we will end up with obsolete EIA when their times come |  |
| 12 | Power Generation |  |  |  | IPP at the Gas Plant to provide the power needs of the Gas Plant, the Pipelines & Manifold Platforms System, and the Petrochemical Plant Complex | Full Specification to be detailed by the IPP Scope |
|  | 12.1 | Primary Power System |  |  | Gas Turbines | N+1 philosophy, and to be synchronised |
|  |  |  |  |  | Power Generation at 11 kV, and stepped down to 6.6 kV and 415 V for high voltage and low voltage supply to the Gas Plant and Manifold Platforms as required | Emergency ----> essential to safe start up / shoutdown |
|  | 12.2 | Secondary Power System |  |  | Diesel Gen Set | Emergency ----> essential to safe start up / shoutdown |
|  |  |  |  |  | Power requirement and classification - normal or emergency depending on equipment specification. | Reference with Load List Vital & Essential Loads to be defined & agreed |
|  |  | 12.2.1 | Uninterruptible Power System |  | UPS supply via dedicated panel connected to emergency bus (switchboard) to supply vital & essential load | The Vital & Essential Loads to be defined & agreed |
|  |  | 12.2.2 | Solar Power Option for Emergency |  | Option for Emergency Back-up with Solar Power will be considered during FEED |  |

|  | Situations & Systems |  |  | Design Basis | Remark / Reference |
| --- | --- | --- | --- | --- | --- |
| 13 | Power Supply |  |  |  |  |
|  | 13.1 | Power Supply to the Remote Manifold |  | Underground Power Cable to the Manifold Platforms |  |
|  |  | 13.1.1 | Power Supply Options | Six main options evaluated, from which underground power cable was selected |  |
|  |  |  |  | 1. Underground Power Cable at all manifolds - Selected Option | Buried underground cables |
|  |  |  |  | 2. Overhead Power Cable at all manifolds | Power cables run through transmission towers |
|  |  |  |  | 3. Composite Power Cable (with FOC) at all manifolds | Composite cable with power and data transmission capability. |
|  |  |  |  | 4. Solar power at all manifolds | Photo-voltaic cells and power bank arrays with inverters |
|  |  |  |  | 5. Combination of Cable and Solar Power | Part underground and part photo-voltaic cell array |
|  |  |  |  | 6. IPP in all manifolds | Gas engines, thermo-electric generators and micro-turbine generators |
|  |  | 13.1.2 | Distribution of the Cable Power | Underground power cable buried along bulk pipeline From the Power Plant at Akabel to the Remote Manifold Platforms at ELEP, BBOU, NUNR, SEIB, and OPUG | Cable sizing with reference to Load requirement, ampacity, voltage drop and short-circuit current. |
|  |  |  |  | Underground power cable buried along bulk pipeline From the Power Plant at Akabel to the AKAB PLEM |  |
|  |  |  |  | Atex (Ex) / NEMA Junction boxes provided along the MF for cable hook-up. |  |
|  |  | 13.1.3 | Emergency & Essential Services Power Systems | UPS supply via dedicated panel connected to emergency bus (switchboard) to supply vital & essential load | vital load ----> needed to run / be available during emegency s/d for safety of personnel |
|  |  |  |  | Power requirement and classification - normal or emergency depending on equipment specification. | Reference with Load List Vital & Essential Loads to be defined & agreed |
|  |  | 13.1.4 | Uninterruptible Power System | UPS supply via dedicated panel connected to emergency bus (switchboard) to supply vital & essential load | The Vital & Essential Loads to be defined & agreed |
|  | Situations & Systems |  |  | Design Basis | Remark / Reference |
| 13 | Power Supply (Contd.) |  |  |  |  |
|  | 13.1 | Power Supply to the Remote Manifold (Contd.) |  |  |  |
|  |  | 13.1.5 | Solar Power Option for Emergency | Option for Emergency Back-up with Solar Power will be considered during FEED |  |
|  |  | 13.1.6 | MCC | Cubicle provision for supply of field MFs dependent on load requirement. | Cubicle provision and sizing dependent on load requirement |
|  |  | 13.1.7 | Earthing & Equipotential bonding | Earthing of MFs to desired earth resistance ≤ 4Ω | SHELL DEP 33.64.10.33-GEN |
|  |  | 13.1.8 | Lighting, small power and welding receptacle | Minimum illumination provision (10 -20 lux) photocell controlled on MFs with Atex (EX) / NEMA lighting fixture. Small power provision for potable hand tools and welding socket provision | Will be addresses during FEED. Welding during shutdown TAM only |
|  |  |  |  | Perimeter and area lighting for facility location and manifold operation area. Small power welding receptacles to be provided with the hazardous area classification as a consideration. |  |
|  |  | 13.1.9 | Single Line Diagrams | Overall and individual single line diagrams to show flow of power supply from the power generation equipment to the required loads. |  |
|  |  | 13.1.10 | Load List | Detailed load list of each M/F based on equipment that require power supply, area and fence lighting. | To determine the load requirement for each M/F for adequate sizing of generation equipment. |
|  |  | 13.1.11 | Hazardous Area Classification | Will be in compliance with Shell DEP 80.00.10.10-Gen in cross reference with API RP 505 and IP 15 | Shell DEP 80.00.10.10-Gen |
|  |  | 13.1.12 | Foot prints | Detail the extents of the selected option so as to have an estimated Manifold Platform size |  |
|  | 13.2 | Power Supply to the Gas Plant |  |  |  |
|  |  | 13.2.1 | Power Supply Options | Step Down of IPP Voltage from 11 kV to 6.6 kV & 415 V for Power Supply to the Gas Plant |  |
|  |  | 13.2.2 | Distribution of the Cable Power | Underground power cable from the Power Plant to the Gas Plant | Cable sizing with reference to Load requirement, ampacity, voltage drop and short-circuit current. |
|  |  |  |  | Atex (Ex) / NEMA Junction boxes provided along the MF for cable hook-up. |  |
|  | Situations & Systems |  |  | Design Basis | Remark / Reference |
| 13 | Power Supply (Contd.) |  |  |  |  |
|  | 13.2 | Power Supply to the Gas Plant (Contd.) |  |  |  |
|  |  | 13.2.3 | Emergency & Essential Services Power Systems | UPS supply via dedicated panel connected to emergency bus (switchboard) to supply vital & essential load | vital load ----> needed to run / be available during emegency s/d for safety of personnel |
|  |  |  |  | Power requirement and classification - normal or emergency depending on equipment specification. | Reference with Load List Vital & Essential Loads to be defined & agreed |
|  |  | 13.2.4 | Uninterruptible Power System | UPS supply via dedicated panel connected to emergency bus (switchboard) to supply vital & essential load | The Vital & Essential Loads to be defined & agreed |
|  |  | 13.2.5 | MCC | Gas Plant Scope / Co-Design with MCC Vendor |  |
|  |  | 13.2.6 | Earthing & Equipotential bonding | Earthing of Plant Skids to desired earth resistance ≤ 4Ω | Shell DEP 33.64.10.33-Gen |
|  |  | 13.2.7 | Lighting, small power and welding receptacle | Minimum illumination provision (10 -20 lux) photocell controlled on Plant Units with Atex (EX) / NEMA lighting fixture. Small power provision for potable hand tools and welding socket provision | Will be addressed during FEED. Welding during shutdown TAM only |
|  |  |  |  | Perimeter and area lighting for facility operation area. Small power welding receptacles to be provided with the hazardous area classification as a consideration. |  |
|  |  | 13.2.8 | Load List | Will be addressed at FEED |  |
|  |  | 13.2.9 | Hazardous Area Classification | Will be addresses at FEED, and in accordance with Shell DEP 80.00.10.10-Gen in cross reference with API RP 505 and IP 15 |  |
|  |  |  |  | The location of equipment to be more than 3m away from the launcher and receiver barrels of the Plant's PLEM, and Process Vessels which are the major points of release of Hydrocarbon. | Intent is to minimise the probability of coincidence of a flammable atmosphere and a source of ignition. |

- Project Concept and Site for the Project

Ref: EA SPDC-BFPC UGS-GEP CSM RPT-001-A01_Concept Select Study – Project Concept & Proposed Site for the Project

The project’s concept is to gather raw wellhead gas, up to the tune of 300 MMscfd, and transport same to a Gas Processing Plant for processing as feedstock to Petrochemical Plant, and Power Plant.

The gas sources cut across five fields, - Bubouwe Bou, Elepa, Nun River, Seibou, and Opugbene, - in OMLs 33, 32, & 36, – with locations widely spread, and on both sides of Nun River.

The gas gathering & evacuation concept constitutes a mix of wellhead flowlines, field manifolds, bulklines, remote satellite manifolds, evacuation manifolds, and trunkline, as shown in the schematics below, - optimally configured in terms of cost, as driven by manifolds layout and choice of evacuation manifold (Bubouwe Bou or Elepa)

Figure 3.1: Regional Location Plan and the Gas Gathering & Evacuation Concept - Schematics

The Gas Plant will comprise three trains, (each with a processing capacity of 150 MMscfd, - corresponding to 300 MMscfd capacity at 3 x 50%); and condensate storage system capacity of 50,000 Tonnes.

It will be located in Akabel, Brass Island, in Brass LGA of Bayelsa State of Nigeria, near the shores of Atlantic Ocean, - in the same complex as the Petrochemical Plant & Power Plant it will be supplying the processed gas.

  - The Gas Plant Site

The aerial photograph showing the plant site is shown below, holding all the three plants, - the Gas Plant, the Petrochemical Plant, and the Power Plant.

Figure 3.2: Aerial Photograph of Plants Site

  - The Gas Gathering & Evacuation Concept

The gas gathering concept & the gas transportation to the Gas Plant will be as follows:

- Gathering of each field well production through its wellhead flowline to the Field’s Central Manifold Header (FC-MH) / Field’s Central Manifold (F-MF).

- Transportation of the F-MF inventory through Bulkline to a remote Satellite Manifold (S-MF), or to the Evacuation Manifold (E-MF), as the case may be for fields (BBOU & ELEP) that are within the vicinity of the E-MF.

- Transportation of the S-MF inventory through Bulklines to the E-MF.

- Transportation of the E-MF inventory through a Trunkline to the Pipeline End Manifold (PLEM) at Akabel, and from there as feedstock inlet stream to the Gas Processing Plant.

The selected wellhead gas gathering and evacuation concept is shown below, with on-stream schedule for the various phases.  The selection resulted from CAPEX comparison of six options that made the final shortlist in the concept select exercise.

Figure 3.1: Wellhead Gas Gathering & Flow Schematics to Gas Processing Plant

The gas will be delivered raw to the gas processing plant for NGL extraction and conditioning of the lean gas to specifications of the petrochemical / power plants.

  - The Gas Gathering & Evacuation Options

The options considered and their installation cost estimates are shown as follows:

Option – 1: S-MF at BBOU & NUNR, and E-MF at ELEP - With OPUG Production Passing through BBOU to ELEP Capex Estimate $469.9 mln - Eliminates need for BBOU – ELEP 9km section for the 24” OPUG B/L.

Option – 2: S-MF at NUNR & E-MF at ELEP - With OPUG Production Direct to ELEP Capex Estimate $501 mln

Figure 3.2: Wellhead Gas Gathering & Flow Schematics – Options 1 & 2

Option – 3: S-MF at NUNR, and E-MF at BBOU Capex Estimate $497.9 mln

Option – 4: S-MF at DBUC & E-MF at ELEP Capex Estimate $449.8 mln – Cost attractive but with S-MF at an external location at DBUC, - a situation not acceptable to BFPC owing to the additional ER risk it will pose

Figure 3.3: Wellhead Gas Gathering & Flow Schematics – Options 3 & 4

Option – 5: S-MF at DBUC, and E-MF at BBOU Capex Estimate $446.3 mln – Cost Attractive / Lowest but with S-MF at an external location at DBUC, - a situation not acceptable to BFPC owing to the additional ER risk it will pose

Option – 6: Bulkline From Each Field Direct to AKAB Capex Estimate $926.3 mln - Demonstrates the high cost of individual manifold transport, compared with satellite concept

Figure 3.4: Wellhead Gas Gathering & Flow Schematics – Options 5 & 6

  - The Manifolds Concept

Three categories of manifolds were conceptualised, - Field Manifold (F-MF), Satellite Manifold (S-MF), and Evacuation Manifold (E-MF).

- Field Central Manifold (F-MF) that receives the field’s production gathered through its wellheads flowlines to the Field’s Central Manifold Header (FC-MH.

- Satellite Manifolds (S-MF) which receives inventory from the Fields’ Manifolds (F-MF) through their bulklines.  There are two satellite manifolds, - at BBOU and NUNR.

- Evacuation Manifold (E-MF) which receives inventory from the Satellite Manifolds (S-MF) through their bulklines.  The evacuation manifold serves as the last gathering point of all fields production, and delivers same to the gas processing plant.

| Field Manifold (F-MF) At SIEB & OPUG | Satellite Manifold (S-MF) At BBOU & NUNR | Evacuation Manifold (E-MF) At ELEP |
| --- | --- | --- |
| At SIEB & OPUG | At BBOU & NUNR | At ELEP |

Figure 3.5: Manifold Concept & Categories

A desirable situation will be to power the manifolds to enable automation, - motorisation of valves, etc, and in that case underground armoured cables will be installed to transport electricity from the IPP at Akabel to the various remote manifolds.  However, in as much as that will be ideal, the concept will be reviewed during FEED in the light of cost savings, and risk assessment with particular reference to widespread vandalisation of such facilities in the region, - to re-confirm that the concept will be worth doing.  Otherwise, the platform will be designed without the concept of automation, with provision only for solar power to provide basic warning lighting at night.

  - The Gas Processing

The proposed gas plant will be full spectrum gas processing plant with capability for NGL extraction, condensate stabilisation & storage, and conditioning of the lean gas to specifications of the petrochemical & power plants, in the following main steps:

- Gas Separation

- Gas Drying

- NGL Recovery

- Fractionation

- Condensate Stabilisation

The plant will be located in Akabel, in the same complex as the Petrochemical Plant & Power Plant it will be supplying the processed gas, but 28.5 km downstream of the evacuation manifold at Elepa from where it will be receiving its feedstock of wet gas.

It will comprise three trains, (each with a processing capacity of 150 MMscfd) and condensate storage capacity of 50,000 Tonnes.  Two trains will be installed initially, in strategic fit with the current GSPA for 270 MMscfd ACQ, while the 3rd train will be installed later, as will be dictated by additional GSPA.

Processing of gas will start with phase-1 production, - initially from BBOU, then joined by ELEP a couple of years later, and in both cases with peak receipt of 300 MMscfd.

The process flow scheme of the process systems of the gas plant is shown in Fig 3.6 below, including the expected products yields for 300 MMscfd wet gas feed from the BBOU / ELEP phase-1 production, with indications for 150 MMscfd and 450 MMscfd.

Figure 3.6: Process Flow Scheme – AKAB Gas Processing Plant

options considered and their installation cost estimates are shown in table 3.1:

Table 3.1: Wellhead Gas Gathering & Flow Schematics – Options 1, 2, & 3.

|  | Option Description | Cost Estimate $mln | Schematics | Remarks |
| --- | --- | --- | --- | --- |
| 1 | S-MF at NUNR, and E-MF at ELEP With OPUG Production Passing through BBOU on its Way to ELEP | 469.9 |  | Savings on 9km installation cost for OPUG 24" B/L. Though constrained to reduced 16" conduit to ELEP, but that is manageable |
| 2 | S-MF at NUNR, and E-MF at ELEP With OPUG Production Direct to ELEP | 501 |  |  |
| 3 | S-MF at NUNR, and E-MF at BBOU | 497.9 |  | Savings on 9km installation cost for OPUG 24" B/L. Though constrained to reduced 16" conduit to ELEP, but that is manageable |

Table 3.1 (contd.): Wellhead Gas Gathering & Flow Schematics – Options 4, 5, & 6.

|  | Option Description | Cost Estimate $mln | Schematics | Remarks |
| --- | --- | --- | --- | --- |
| 4 | S-MF at NUNR, and E-MF at ELEP With OPUG Production Passing through BBOU on its Way to ELEP | 469.9 |  | Savings on 9km installation cost for OPUG 24" B/L. Though constrained to reduced 16" conduit to ELEP, but that is manageable |
| 4 | S-MF at NUNR, and E-MF at ELEP With OPUG Production Passing through BBOU on its Way to ELEP | 501 |  | Savings on 9km installation cost for OPUG 24" B/L. Though constrained to reduced 16" conduit to ELEP, but that is manageable |
| 6 | S-MF at NUNR, and E-MF at ELEP With OPUG Production Passing through BBOU on its Way to ELEP | 497.9 |  | Savings on 9km installation cost for OPUG 24" B/L. Though constrained to reduced 16" conduit to ELEP, but that is manageable |

- Feedstock Supply & Product Evacuation Scheme

Ref: EA SPDC-BFPC UGS-GEP CSM RPT-001-A01_Concept Select Study – Project Concept & Proposed Site for the Project

The feedstock supply and product evacuation involves the gathering of the fields’ raw wellhead gas, and transporting same through a network of bulklines and trunkline to the Gas Processing Plant for processing into lean gas and condensate.

The lean gas is supplied as feedstock to the Petrochemical Plant for methanol & urea production, and also to the Power Plant for power generation.

The condensate is exported via a delivery to the condensate export jetty from where offtakers take delivery.  The condensate export jetty is within the BFPC plants’ captive export jetty, from where all plants’ products (condensate, methanol, and urea) are exported.

  - The Feedstock & Products Evacuation Schematics

The feedstock and product evacuation schematics is shown below, - involving gathering and transporting the fields’ raw wellhead gas via a network of flowlines & pipelines to the Gas Processing Plant, and the delivery of the Gas Plant lean gas (product) to the the Petrochemical & Power Plants, and the condensate (product) to the condensate export jetty for export to buyers.

Figure 4.1: Feedstock & Products Evacuation Schematics

- Pipeline Route Selection

Ref: EA SPDC-BFPC UGS-GEP SVY RPT-001-A01_Concept Select Study – Pipeline Route Selection Study

The Pipeline Route Selection focused on the requirement for phase-1 production, which involves Bubouwe & Elepa gas transportation to Akabel Gas Plant, and has prescribed an optimal route based on desktop studies.  Indicative routes have also been earmarked for requirement beyond phase-1 production, but proper desktop studies will be done for each of the phases when the time draws near.

Ideal to follow the ‘desk-top’ selected route was the field investigation to evaluate its viability, involving consultations with relevant stake holders.  That was planned but could not be carried out owing to security risk and excessive FTO demand at the time.  It was therefore decided to carry out the investigation as part of the full preparatory (FP) survey by which time the security and FTO issues would have been fully addressed.

  - Pipeline Route for Phase-1 Production

The pipeline route for phase-1 is shown in Fig 5.1 below, - a composite route comprising two segments, viz: BBOU - ELEP, and ELEP – AKAB.

Figure 5.1: Composite Plan of the Selected Pipeline Route – for Phase-1 Production

  - Routes Options Considered

Three routes shown in Fig 5.2 below were in the final screening, with the 9 km BBOU – ELEP segment common to all of them, viz:

- The Proposed Route, - Total Length 37.5 km, comprising: 9km BBOU – ELEP, and 28.5 km ELEP – AKAB.

This route is the shortest of the three, with moderate river crossing lengths (+1km) that will not pose extra-ordinary construction challenges.

- The Alternative Route ‘A’, Total Length 40 km, comprising: 9km BBOU – ELEP, and 31 km ELEP – AKAB.

This route is 2.5 km longer than the proposed route, and with some 4 km wide river crossing length that will pose extra-ordinary construction challenges.

- The Alternative Route ‘B’, Total Length 51 km, comprising: 9km BBOU – ELEP, and 42 km ELEP – AKAB predominantly offshore.

This route is the longest, and with about 35 km of the ELEP – AKAB segment in Brass River / Offshore Coastal water, - rendering it the most expensive to install.  The idea behind route was to minimise traverse through community settlements, but it turned out to be that it is equally within view and reach of those settlements it aimed to avoid.  Furthermore from intelligence report gathered, the route is not what NIMASA will be disposed to support, - as it will compromise on dredging programme that may arise in respect of waterways navigation in the area.

Figure 5.2: Alternative Pipeline Routes – for Phase-1 Production

The features of the three routes are summarised in table 5.1 below; based on which the proposed route is assessed the most acceptable of the three.

Table 5.1: Assessment Matrix of the Proposed Route & Alternative Routes.

| s/n | Description | Features | Positive | Negatives |
| --- | --- | --- | --- | --- |
| 1 | Proposed Route | - Distance 1.1, BB-Elepa = 9km 1.2, Elepa – Akabel = 28.5 km. - Key Communities 2.1, Elepa (Nembe) 2.2, Golubokiri (Nembe) 2.3, Kpongbokiri (Nembe) 2.4, Fantuo (Fantuo) 2.5, Oyankia (Odioama) 2.6, New Jerusalem (Odioama) 2.7, Akabeleu-Ama (Odioama) - LGAs 3.1, Southern Ijaw 3.2, Nembe 3.3, Brass - River Crossings 4.1, Major Crossings (3 x 1.2km). 4.2 Minor Crossings(13 nos) | - Shortest in length. - Easily manageable river crossing lengths. | No significant challenges. |
| 2 | Alternative Route ‘A’ | - Distance 1.1, BB-Elepa = 9km 1.2, Elepa – Akabel = 31km. - Key Communities 2.1, Elepa (Nembe) 2.2, Egwema (Egwema) 2.3, Twon-Brass (Twon-Brass) 2.4, Ewoama (Ewoama/Okpoama) 2.5, Okpoama (Okpoama) 2.6, Diema (Diema) 2.7, Oyankia (Odioama) 2.8, Akabeleu-Ama (Odioama) - LGAs 3.1, Southern Ijaw 3.2, Nembe 3.3, Brass - River Crossings 4.1, Major Crossings (1 x 4km) 4.2 Minor Crossings(13 nos) | Only one major river crossing | Key Installation challenge of 4km river crossing by HDD. Though feasible. |

Table 5.1 (contd): Assessment Matrix of the Proposed Route & Alternative Routes

| s/n | Description | Features | Positive | Negatives |
| --- | --- | --- | --- | --- |
| 3 | Alternative Route ‘B’ | - A section of Route runs near parallel to the sea front. This will pose enormous engineering, construction and logistics challenges, including the Brass estuarine crossing - Distance 2.1 BB-Elepa = 9km 2.2, Elepa – Akabel = 42km. - Key Communities 3.1, Elepa (Nembe) 3.2, Brass - LGAs 4.1, Southern Ijaw 4.2, Nembe 4.3, Brass - River Traverse 5.1, 20% on land 5.2, 30% in Brass River. 5.3, 50% Offshore. | No significant benefit. | - Engineering and installation challenges and high cost associated with offshore terrain - Challenge of managing estuarine wave turbulence during installation works - Rapid sea bed dynamics that will be experienced in the region - Exposure to damages in case of possible future dredging works to improve Brass River navigability. |

Note:-

Recommeded major river crossing method for the pipeline, power and telecommunication cables shall be by HDD process through a single hole in one pull-pass. While the minor river/creek crossing shall be by open cut.

## Field Investigation Works

The field investigation works that will be done along with the FP Survey will address the following:

- The river and creek crossing points shall be identified and investigated for construction convenience and other potential hazards and constrains.

- Assessment of the elevation difference at the overland area beyond the rivers/creeks banks, including height of adjacent banks. Abrupt changes of elevation (i.e. steep banks) to be avoided.

- Due consultation shall be held with the local communities to identify if any, some sacred or forbidden bush, biodiversity, legacy, traditional concerns.

- Due consultation shall be held with the relevant government agencies for information of planned future land use and any environment requirements.

    - Other Considerations

Other key considerations to the route selection are:

- Safety of the public and personnel - the route must provide a safe and secured environment for the pipeline during construction and over its operational life and ideally be routed away from populated areas.

- Minimum negative impact on the environment, nature, biodiversity and minimal land take.

- Ability to meet the project’s economic objectives, without compromising safety and environment, and minimizing local economy impact on communities that the pipeline passes through, and have the smallest footprint feasible (ideally the shortest distance between pipeline start and end points).

- Land ownership related factors e.g. the number of landowners, anticipated ease and cost to obtain/purchase consents.

- Contents of the pipeline and operating conditions, e.g. consideration of leakage of a high vapour pressure liquids.

- Terrain and subterranean conditions, including geotechnical, hydrographical, and meteorological conditions should be acceptable. These include ground stability, and other land uses which may create instability (e.g. sand mining subsidence, land development).

- Cultural heritage sites, should be strictly identified and avoided.

- Existing and future land plan usage, determinable through research of public records and consultation with the local land owners & government planning agencies which should identify:-

- Third-party activities

- Agricultural practice

- Facilities and services

- Future development requirements

- Permanent access; – the pipeline must ideally be accessible for inspection and maintenance all year round over its operational life

- Security; - the pipeline system should be routed to minimise security concerns, particularly due to trespass and sabotage, during both construction and operation.

- Throgh out the operational life span, the pipeline route limits shall as far as possible be made visible, either by regular devegetation or marking with above markers.

- Other unforeseen hazards.

## Right of Way (ROW) Acquisition

Construction Right of Way of 30 m width is proposed.  The idea is to make sufficient provision for the various requirements as shown in the sketch below.

Figure 5.2: Alternative Pipeline Routes – for Phase-1 Production

## PTS Application Plan

PTS Application plan followed DPR guide for processing as two composite gas pipelines: 16” x 9km Bubouwe – Elepa segment, and 24” x 28.5km Elepa - Akabel segment, as shown in the sketch below.

| 16” x 9km BBOU - ELEP | 24” x 28.5km ELEP - AKAB |
| --- | --- |

Figure 5.2: Alternative Pipeline Routes – for Phase-1 Production

The proposed pipeline route skirts major community settlements and traverses mangrove vegetation with multiple river and creek crossings. Four existing pipelines have been identified and will be duly crossed.  It is currently perceived that the entire pipeline route traverses three Local Government Areas of Bayelsa State: - Southern Ijaw, Brass and Nembe Local Government Areas.

The start and end point co-ordinates in Geographic Co-ordinate System (referenced to Minna Datum) of the 16’’ x 9km Bubouwe Bou Manifold ‘’A’’ to Elepa Manifold ‘’B’’, pipeline with wall thickness of 0.875 inches are as contained in Table-1.

Table 5.2: Bubouwe Bou Manifold “A” to Elepa Manifold “B”

| Point | Eastings (m) | Northings (m) | Latitude | Longitude |
| --- | --- | --- | --- | --- |
| Start ‘A’ | 407,242.91 | 50,888.32 | 040 27’ 23.46’’N | 060 07’ 39.49’’E |
| End ‘B’ | 413,949.37 | 46,280.51 | 040 24’ 54.22’’N | 060 11’ 17.36’’E |

The start and end point co-ordinates in Geographic Co-ordinates System (referenced to Minna Datum) of 24” x 28.5km Elepa Manifold ‘’B’’ to Akabel Manifold ‘’C’’ pipeline with wall thickness of 1.25 inches are as contained in Table-2.

Table 5.3: Elepa Manifold “B” to Akabel Manifold “C

| Point | Eastings(m) | Northings(m) | Latitude | Longitude |
| --- | --- | --- | --- | --- |
| Start ‘B’ | 413,949.37 | 46,280.51 | 040 24’ 54.22’’N | 060 11’ 17.36’’E |
| End ‘C’ | 437,351.70 | 34,771.29 | 040 18’ 41.92’’N | 060 23’ 57.06’’E |

## The Full Preparatory Survey

The Full Preparatory (FP) Survey will be carried out following receipt of PTS approval, and all field investigation works will then be carried out along side / as part of the FP Survey.

- Process Simulation

Ref: EA SPDC-BFPC UGS-GEP PRC RPT-001-A01_Concept Select Study – Process Simulation - HYSYS

Ref: EA SPDC-BFPC UGS-GEP PRC RPT-002-A01_Concept Select Study – Process Simulation - PIPESIM

Ref: EA SPDC-BFPC UGS-GEP PRC RPT-003-A01_Concept Select Study – Process Simulation - OLGA

Ref: EA SPDC-BFPC UGS-GEP PRC RPT-004-A01_Concept Select Study – Gas Plant Process Simulation - HYSYS

This chapter summarises the process simulation results of the above reference Process Simulation Studies which are as follows:

Enablers to the FA

Pressure drop is such that we can offtake at inlet and arrive destination at stipulated pressure band.

Velocity, ok – b/w min & max

Hydrate – nil (either as it is or under treatment.

Material that will corrode acceptably (minimally) or with CI treatment to stay within limit that will deliver over the life cycle.

Etc

last life Corrosion or treatment

Deliverin

- Steady State Process Simulation of the Pipeline Network, using HYSYS

- Steady State Process Simulation of the Pipeline Network, using PIPESIM

- Transient State (Dynamic) Process Simulation of the Pipeline Network, using OLGA

- Steady State Process Simulation of the Gas Plant Process, using HYSYS

The objective of the simulations is in two folds:

- To establish a gas gathering concept, with the appropriate line sizes that will guarantee flow assurance and gas deliverability to the Gas Plant at the required pressure; as well as establish operating conditions and treatment required in the pipeline to meet the deliverability

- To also establish gas processing technology and operating conditions to meet product mix specifications at the Gas Plant.

The process simulation delivers this objective through the following scopes:

- Generate mass and material balances for flow sources in-situ the pipelines and at the delivery outlets.

- Establish appropriate line sizes suitable for the anticipated range of flow from the fields.

- Establish flow assurance through the following parameter checks, - pressure, temperature, velocity and hydrate formation.

- Develop a suitable operating flow conditions, especially in respect of maximum velocity to prevent erosion, and minimum velocity to prevent surging, and to keep line swept clear of entrained solids & liquids; and temperature.

The HYSYS, PIPESIM & OLGA Simulation results for the Pipeline Network are quite comparable, and established both deliverability and flow assurance of the feed gas to the Gas Processing Plant.

Also, the HYSYS Simulation results for the Gas Plant established both deliverability and flow assurance of the feed gas to the Gas Processing Plant.

This chapter summarises the process simulation results of the above reference Process Simulation Studies which are as follows:

- Steady State Process Simulation of the Pipeline Network, using HYSYS

- Steady State Process Simulation of the Pipeline Network, using PIPESIM

- Transient State (Dynamic) Process Simulation of the Pipeline Network, using OLGA

- Steady State Process Simulation of the Gas Plant Process, using HYSYS

The objective of the simulations is in two folds:

- To establish a gas gathering concept, with the appropriate line sizes that will guarantee flow assurance and gas deliverability to the Gas Plant at the required pressure; as well as establish operating conditions and treatment required in the pipeline to meet the deliverability

- To also establish gas processing technology and operating conditions to meet product mix specifications at the Gas Plant.

The process simulation delivers this objective through the following scopes:

- Generate mass and material balances for flow sources in-situ the pipelines and at the delivery outlets.

- Establish appropriate line sizes suitable for the anticipated range of flow from the fields.

- Establish flow assurance through the following parameter checks, - pressure, temperature, velocity and hydrate formation.

- Develop a suitable operating flow conditions, especially in respect of maximum velocity to prevent erosion, and minimum velocity to prevent surging, and to keep line swept clear of entrained solids & liquids; and temperature.

The HYSYS, PIPESIM & OLGA Simulation results for the Pipeline Network are quite comparable, and established both deliverability and flow assurance of the feed gas to the Gas Processing Plant.

Also, the HYSYS Simulation results for the Gas Plant established both deliverability and flow assurance of the feed gas to the Gas Processing Plant.

  - The Gas Gathering & Process Flow Scheme

The overall gas gathering and processing scheme, with phased on-stream dates is as shown in Fig 6.1 below.

Figure 6.1: Overall Gas Gathering & Processing Schematics, with phased on-stream dates

Production starts in 2020 with BBOU + ELEP as phase-1, followed in sequence by production from other phases, from 2033 as reflected in the schematics.  The peak production associated with each phase is also indicated.

The 420 MMscfd peak for ELEP – AKAB Trunkline represents a situation where the line handles both the BBOU & ELEP peak at the same time, - simulated to establish robustness of the trunkline up to that capacity level.  That will guide decision in a situation where BFPC would still wish to pursue the aspiration to intake up to 400 MMscfd into the Gas Processing Plant.

  - Basis and Assumptions

The pipeline temperature ambient medium was taken as 25 0C.

Also, it is assumed for this conceptual study, that there are no elevation changes across the pipeline route.

    - Software & Simulation Property Package

The hydraulics process simulation study was undertaken using Aspen HYSYS v7.3, PIPESIM, and OLGA

Water saturation extension was explored to determine the saturated water in the gas key considerations to the route selection are:

The thermodynamic equation of state (EOS) used for the process simulation is; Peng-Robinson (PR) fluid property package.

PVT Compositions

For BBOU stream Analogue Composition (Bonny PVT) was used.

For ELEP stream, ELEP Upper U4 sand PVT was used.

For NUNR stream, NUNR H4 sand PVT was used.

For SEIB stream, the average of PVT compositions of SEIB G1.1M & F9.3M was used.

For OPUG, the average of OPUG F2000 & F2001 was used.

    - Flow Rates, Fluid Condition & Composition

Flow Rates

The forecast production profiles for BBOU, ELEP, NUNR, & SEIB are summarized below.

Figure 2.1: Fields’ Gas Production Forecast – Facilities Design Data

Fluid Condition & Composition

The Reservoir Fluid Conditions and PVT Compositions for BBOU, ELEP, NUNR, SEIB, & OPUG are summarized in table 6.2 below

Table 6.2: BBOU, ELEP, NUNR, SEIB, and OPUG Reservoir Compositions & Conditions.

Water Saturated Compositions

The Re-Constructed Reservoir Fluid PVT Compositions saturated with water is shown below in table 6.3.

Table 6.3: BBOU, ELEP, NUNR, SEIB, and OPUG Reservoir Compositions Saturated with Water

  - Gas Deliverability & Flow Assurance

Gas deliverability flow assurance outlook is presented for the peak flow production for each of the four phases.

The deliverability being adjudged by the following:

- Ability of the system to offtake the gas at 80 – 100 barg from the Custody Transfer Manifold and deliver to the Gas Processing Plant within the stipulated pressure band of 60 – 65 barg.

- Fluid velocity below erosional velocity limit, - to avoid pipewall erosion, excess noise or water hammer; and above the minimum level required for efficient flow, - to keep line swept clear of entrained solids & liquids.

- Fluid traverse outside hydrate formation region of the phase envelope.

    - Phase-1 Production (BBOU & ELEP)

There are three scenarios associated with this phase-1 production:

- BBOU – ELEP Bulkline Flow, with BBOU as the only production to AKAB, flowing at 300 MMscfd; with its resultant flow into ELEP – AKAB Trunkline at the 300 MMscfd BBOU production.

- BBOU – ELEP Bulkline Flow, with production from both BBOU and ELEP, with BBOU flowing at 180 MMscfd and ELEP at 120 MMscfd; with its resultant flow into ELEP – AKAB Trunkline at 300 MMscfd, being the combined BBOU & ELEP production.

- Examines the robustness of the ELEP to AKAB Trunkline to handle up to 420 MMscfd, simulated by combining BBOU peak production of 300 MMscfd with the 120 MMscfd of ELEP, to arrive at the 420 MMscfd flow into the ELEP – AKAB Trunkline.

The ELEP Field – ELEM Manifold Flow is assumed to have the constant scenario of ELEP flowing at 120 MMscfd, and therefore not considered as generating a different scenario.

Scenario-1 (Phase-1 Production)

The flow parameters give positive indication of deliverability and flow assurance, giving the pressure, velocity, & fluid traverse profiles shown in figs 6.3 & 6.4 below.

Flow Parameters – Pressure & Velocity Profile: Scenario-1, Phase-1 Production

| Wellhead Gas Gathering Schematics & Flow Parameters to Gas Processing Plant - HYSYS |  |
| --- | --- |
| Pressure Profile – Pipesim | Velocity Profile – Pipesim |

Figure 6.3: Flow Parameters and Pressure & Velocity Profile – Scenario-1, Phase-1 Production

Flow Parameters – Flow Assurance Analysis

BBOU – ELEP Flow Stream: Scenario-1, Phase-1 Production

ELEP – AKAB Flow Stream: Scenario-1, Phase-1 Production

Figure 6.4: Flow Assurance Analysis – Scenario-1, Phase-1 Production

Scenario-2 (Phase-1 Production)

The flow parameters give positive indication of deliverability and flow assurance, giving the pressure, velocity, & fluid traverse profiles shown in figs 6.5 & 6.6 below.

Flow Parameters – Pressure & Velocity Profile: Scenario-2, Phase-1 Production

| Wellhead Gas Gathering Schematics & Flow Parameters to Gas Processing Plant - HYSYS |  |
| --- | --- |
| Pressure Profile – Pipesim | Velocity Profile – Pipesim |

Figure 6.5: Flow Parameters and Pressure & Velocity Profile – Scenario-2, Phase-1 Production

Flow Parameters – Flow Assurance Analysis

BBOU – ELEP Flow Stream: Scenario-2, Phase-1 Production

ELEP – AKAB Flow Stream: Scenario-2, Phase-1 Production

Figure 6.6: Flow Assurance Analysis – Scenario-2, Phase-1 Production

Scenario-3 (Phase-1 Production)

The flow parameters give positive indication of deliverability and flow assurance, giving the pressure, velocity, & fluid traverse profiles shown in figs 6.7 & 6.8 below.

Flow Parameters – Pressure & Velocity Profile: Scenario-3, Phase-1 Production

| Wellhead Gas Gathering Schematics & Flow Parameters to Gas Processing Plant - HYSYS |  |
| --- | --- |
| Pressure Profile – Pipesim | Velocity Profile – Pipesim |

Figure 6.7: Flow Parameters and Pressure & Velocity Profile – Scenario-3, Phase-1 Production

Flow Parameters – Flow Assurance Analysis

BBOU – ELEP Flow Stream: Scenario-3, Phase-1 Production

ELEP – AKAB Flow Stream: Scenario-3, Phase-1 Production

Figure 6.8: Flow Assurance Analysis – Scenario-3, Phase-1 Production

ELEP – AKAB Flow Parameters & Flow Assurance Analysis - Pipesim:  Phase-1 Scenarios

The flow parameters give positive indication of deliverability and flow assurance, giving the pressure, velocity, & fluid traverse profiles shown in fig 6.9 below.

| Scenario 1: BBOU – ELEP Bulkline Flow, with BBOU as the only production to AKAB: | Pressure (barg) |  | Velocity (m/s) |  | Phase Envelope |
| --- | --- | --- | --- | --- | --- |
|  | Inlet | Outlet | Inlet | Outlet |  |
| ELEP – AKAB Bulkline 300 MMscfd. Resulting From: BBOU – ELEP Bulkline 300 MMscfd ELEP – ELEP MF Bulkline 0 MMscfd | 74.35 | 67.71 | 5.25 | 5.64 |  |
| Scenario 2: BBOU – ELEP Bulkline Flow, with production from both BBOU and ELEP to AKAB. | Pressure (barg) |  | Velocity (m/s) |  | Phase Envelope |
|  | Inlet | Outlet | Inlet | Outlet |  |
| ELEP – AKAB Bulkline 300 MMscfd. Resulting From: BBOU – ELEP Bulkline 180 MMscfd ELEP – ELEP MF Bulkline 120 MMscfd | 76.41 | 67.37 | 5.59 | 6.08 |  |
| Scenario 3: Examines the robustness of the ELEP to AKAB Trunkline to handle up to 420 MMscfd, | Pressure (barg) |  | Velocity (m/s) |  | Phase Envelope |
|  | Inlet | Outlet | Inlet | Outlet |  |
| ELEP – AKAB Bulkline 420 MMscfd. Resulting From: BBOU – ELEP Bulkline 180 MMscfd ELEP – ELEP MF Bulkline 120 MMscfd | 84.01 | 67.63 | 7.00 | 8.41 |  |

Figure 6.9: Flow Assurance Analysis – ELEP - AKAB, Phase-1 Production

    - Phase-2 Production (NUNR & ELEP)

The flow parameters give positive indication of deliverability and flow assurance, giving the pressure, velocity, & fluid traverse profiles shown in figs 6.10 & 6.11 below.

Flow Parameters – Pressure & Velocity Profile: Phase-2 Production

| Wellhead Gas Gathering Schematics & Flow Parameters to Gas Processing Plant - HYSYS |  |
| --- | --- |
| Pressure Profile – Pipesim | Velocity Profile – Pipesim |

Figure 6.10: Flow Parameters and Pressure & Velocity Profile – Phase-2 Production

Flow Parameters – Flow Assurance Analysis

NUNR – ELEP Flow Stream: Phase-2 Production

ELEP – AKAB Flow Stream: Phase-2 Production

Figure 6.11: Flow Assurance Analysis – Phase-2 Production

    - Phase-3 Production (SEIB, NUNR & ELEP)

The flow parameters give positive indication of deliverability and flow assurance, giving the pressure, velocity, & fluid traverse profiles shown in figs 6.12 & 6.13 below

Flow Parameters – Pressure & Velocity Profile: Phase-3 Production

| Wellhead Gas Gathering Schematics & Flow Parameters to Gas Processing Plant - HYSYS |  |
| --- | --- |
| Pressure Profile – Pipesim | Velocity Profile – Pipesim |

Figure 6.12: Flow Parameters and Pressure & Velocity Profile – Phase-3 Production

Flow Parameters – Flow Assurance Analysis

SEIB – NUNR Flow Stream: Phase-3 Production

NUNR - ELEP Flow Stream: Phase-3 Production

ELEP – AKAB Flow Stream: Phase-3 Production

Figure 6.13: Flow Assurance Analysis – Phase-3 Production

    - Phase-4 Production (OPUG, SEIB, & NUNR)

The flow parameters give positive indication of deliverability and flow assurance, giving the pressure, velocity, & fluid traverse profiles shown in figs 6.14 & 6.15 below

Flow Parameters – Pressure & Velocity Profile: Phase-4 Production

| Wellhead Gas Gathering Schematics & Flow Parameters to Gas Processing Plant - HYSYS |  |
| --- | --- |
| Pressure Profile – Pipesim | Velocity Profile – Pipesim |

Figure 6.14: Flow Parameters and Pressure & Velocity Profile – Phase-4 Production

Flow Parameters – Flow Assurance Analysis

OPUG – BBOU Flow Stream: Phase-4 Production

BBOU - ELEP Flow Stream: Phase-4 Production

SEIB – NUNR S-MF Flow Stream: Phase-4 Production

NUNR S-MF – ELEP Flow Stream: Phase-4 Production

ELEP – AKAB Flow Stream: Phase-4 Production

Figure 6.15: Flow Assurance Analysis – Phase-4 Production

2033 Production (Water Production Sensitivity)

The flow parameters give positive indication of deliverability and flow assurance, giving the pressure, velocity, & fluid traverse profiles shown in figs 6.16 & 6.17 below.

Flow Parameters – Pressure & Velocity Profile: 2033 Production

| Wellhead Gas Gathering Schematics & Flow Parameters to Gas Processing Plant - HYSYS |  |
| --- | --- |
| Pressure Profile – Pipesim | Velocity Profile – Pipesim |

Figure 6.16: Flow Parameters and Pressure & Velocity Profile – 2033 Production

Flow Parameters – Flow Assurance Analysis

BBOU F-MF – ELEP Flow Stream: 2033 Production

NUNR – ELEP Flow Stream: 2033 Production

ELEP - AKAB Flow Stream: 2033 Production

Figure 6.17: Flow Assurance Analysis – 2033 Production

ELEP – AKAB Flow Parameters & Flow Assurance Analysis - Pipesim:  Phases 2 - 4

The flow parameters give positive indication of deliverability and flow assurance, giving the pressure, velocity, & fluid traverse profiles shown in fig 6.16 below.

| Phase 2: NUNR– ELEP Bulkline Flow, with production from NUNR and ELEP to AKAB: | Pressure (barg) |  | Velocity (m/s) |  | Phase Envelope |
| --- | --- | --- | --- | --- | --- |
|  | Inlet | Outlet | Inlet | Outlet |  |
| ELEP – AKAB Bulkline 300 MMscfd. Resulting From: NUNR – ELEP Bulkline 240 MMscfd ELEP – ELEP MF Bulkline 60 MMscfd | 80.46 | 64.72 | 5.24 | 6.29 |  |
| Phase 3: SEIB – NUNR, NUNR – ELEP Bulkline Flow, with production from SEIB, NUNR and ELEP to AKAB. | Pressure (barg) |  | Velocity (m/s) |  | Phase Envelope |
|  | Inlet | Outlet | Inlet | Outlet |  |
| ELEP – AKAB Bulkline 300 MMscfd. Resulting From: SEIB – NUNR Bulkline 225 MMscfd NUNR – ELEP Bulkline 55 MMscfd ELEP – ELEP MF Bulkline 20 MMscfd | 78.19 | 63.65 | 5.39 | 6.38 |  |
| Phase 4: SEIB – NUNR, NUNR – ELEP, OPUG – BBOU, BBOU - ELEP Bulkline Flow, with production from OPUG, SEIB, and NUNR to AKAB. | Pressure (barg) |  | Velocity (m/s) |  | Phase Envelope |
|  | Inlet | Outlet | Inlet | Outlet |  |
| ELEP – AKAB Bulkline 300 MMscfd. Resulting From: OPUG – BBOU Bulkline 200 MMscfd SEIB – NUNR MF Bulkline 80 MMscfd NUNR – ELEP MF Bulkline 20 MMscfd | 78.52 | 60.01 | 5.37 | 6.81 |  |

Figure 6.16: Flow Assurance Analysis – ELEP - AKAB, Phases 2 - 4 Production

- Pipeline Materials Selection & Corrosion Analysis

Ref: EA SPDC-BFPC UGS-GEP MAT RPT-001-A01_Concept Select Study – Materials Selection & Corrosion Analysis

This chapter summarises the results of the above referenced Concept Select Study document in respect of the Materials Selection and Corrosion Analysis for the project.

The report covers the following subject areas:

- The Pipelines’ Materials Selection & Corrosion Analysis

- The Coating Materials Choice

- The Cathodic Protection System

  - Intent & Basis

Intent is Pipeline Integrity, and Materials Selection is the key to this intent.

The bases for the materials selection are the process & environmental conditions, and the operational requirements.

The process conditions constitute the physical conditions, - predominantly pressure and temperature, which the system must contend with in respect of stress; and the chemical conditions emanating from the impurities contained in the natural gas stream, - predominantly carbon dioxide, hydrogen sulfide (though not in this case), water vapor, oil, nitrogen, hydrates, which are capable of inducing internal corrosion and erosion of the system.

The environmental condition represents the ambient, which if corrosive will cause external corrosion of the system.

Both corrosions (internal & external) are undesirable and have to be mitigated.

The operational requirements constitute consideration of design, construction, operations, maintenance, threats, hazards, risks, safety and economics; and codes and standards that provide mandatory and optional requirements and guidelines.

Similarly, the bases for the corrosion analysis are the susceptibility of the material to corrosion, the corrosivity potential of the transported fluid that will cause internal corrosion to the pipeline system, and the environment which, if corrosive, will cause external corrosion.

These premises will be followed in the materials selection & corrosion analysis process to ensure proper selection of materials and the relational corrosion management system.

  - Purpose & Objective of the Study

The purpose is three-fold:

- Identification of best pipeline materials for each of the line segments at optimised life cycle costs, utilising best industry practice.

- Assessment of the risks associated with the specific selection.

- Specification of the corrosion management system that will mitigate the risk.

The objective is the optimal specification of materials and the relational corrosion management systems that can contend with the process & environmental conditions, and operational requirements of the pipeline system.

  - Materials Options

The following materials were evaluated:

- Mild Steel (MS) - Carbon Manganese Steels

- MS plus Inhibition

- Various CRA’s (Austenitic SS, Ferritic SS. Martensitic SS, Duplex SS)

- Precipitated Hardened Steel (with Al)

- Pipeline with Internal CRA Lining or Cladding

- Composite Materials

- Non- Ferrous Materials

The technical feasibility followed a four stage evaluation for acceptance, defined on a Green, Yellow, and Red traffic light colouration system.

Red represents: rejection

Yellow represents: not totally acceptable because of some few limitations including costs, properties, maintainability etc.

Green represents total acceptance of the material

The technical feasibility for the materials was demonstrated through the following evaluations.

- Feasibility of Mild Carbon Steel, - without Corrosion Inhibition

- Feasibility of Mild Carbon Steel, - with Corrosion Inhibition

- Feasibility of CRA Materials through the following steps:

- Optional Process for Modification of the Fluid Content through the following steps:

  - The System & Fluid Under Consideration

The system under consideration is the pipeline network as shown in fig 7.1 below.

It comprises the following pipeline segments, coming into service in phases, and transporting raw wellhead gas to the Gas Processing Plant:

- 24” x 28.5 km ELEP – AKAB Trunkline

- 16” x 9 km BBOU – ELEP Bulkline

- 24” x 45 km NUNR – ELEP Bulkline

- 20” x 20 km SEIB – NUNR Bulkline, and

- 24” x 37 km OPUG – BBOU Bulkline

- Each of the five Field Manifold Headers of about 10 m length.

Figure 7.1: The Pipeline Network

The fluid transported is high pressure category D/E fluid (flammable fluid), in the pressure & temperature regions of 100 barg, and 60 oC, and containing CO2 that causes corrosion.

However, it contains no H2S that causes sour corrosion, or Hg that causes liquid metal embrittlement (LME) of alloys, or Naturally Occurring Radioactive Materials (NORM) which can accumulate to render the system radioactive over time.

  - The Fluid Impurities Composition

The impurities composition in the various reservoir fluids are shown in the table below:

Table 7.1: Reservoirs Conditions & Impurities Compositions (Water Saturated Compositions)

|  | Impurities Compositions - Mole % |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Component | BBOU | ELEP | NUNR | SIEB | OPUG |
| N2 | 0.07 | 0.00 | 0.00 | 0.00 | 0.10 |
| CO2 | 0.59 | 2.17 | 0.77 | 1.53 | 2.21 |
| H2S | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 |
| H2O | 0.34 | 2.19 | 0.63 | 0.95 | 1.08 |
| Total | 1.00 | 4.36 | 1.40 | 2.48 | 4.11 |
| Reservoir Conditions |  |  |  |  |  |
| Reservoir Pressure (bar) | 223.8 | 476.6 | 500.6 | 450.6 | 356.9 |
| Reservoir Temp (oC) | 78 | 123 | 102 | 117 | 113.8 |
| pCO2 (bar) | 1.32 | 10.34 | 3.85 | 6.89 | 7.89 |

At the deducible partial pressures of CO2 (pCO2) above, the gas streams will result in significant corrosion of the pipeline system, judging from both the corrosion prediction calculation results from DeWaard-Milliams Model, and its CO2 Corrosion Nomogram shown below.

Fig 7.2: DeWaard-Milliams CO2 Corrosion Nomogram

  - Issues & Considerations

Of the essence will be pressure containment by the system as well as corrosion prevention / mitigation in the system; and in that respect the following issues were given due considerations:

- Materials of good strength & properties to withstand the high pressure under consideration and corrosive nature of the fluid to be transported.

- High Pressure Transportation of Category D/E Fluid (flammable fluid), in the pressure region of 100 barg, and temperature region of 60 OC.

- The Gas is potentially corrosive on account of its CO2 content; and water, NGL, & particulates that are inherently present

- CO2 content is up to 2%, which for the empirically established 0.1 bar partial pressure treshold for significant CO2 corrosion, implies corrosion of unprotected mild CS at pressures above 5 barg

- The CO2 content of up to 2%, renders the gas corrosive at pressures higher than 5 barg (corresponding to the empirically established 0.1 bar partial pressure treshhold for significant CO2 corrosion), - prompting the need for CRA materials or matching corrosion management design.

- The Carbon Steel option would mean more internal wall thickness plus corrosion allowance

- Internal corrosion will be substantial for which the traditional (normal) corrosion allowance of 3 mm will be insufficient provision.

- Higher corrosion allowance, up to the traditional maximum of up to 10 mm will therefore be considered, and in that respect, high yield strength material with smaller wall thickness demand will be used, so as to keep the overall design wall thickness as small as possible.

- Ambient environment is corrosive owing to joint contribution of humidity, acidity, and oxygen in air, and known potentially corrosive nature of the soil in the area.

- Temporal Protection of the linepipes / pipelines, - that will give progressive protection during the period of construction, until the ICCP system is put in place

  - Recommended Materials

The recommended line pipe materials are as follows:

- API 5L X65 Carbon Steel for the main (long distance) Pipelines and Pig Barrels, and

- 25Cr UNS Code S32750 Stainless Steel for the Manifold’s in-plot pipelines & piping.

The pictorial concept is presented in the fig 7.2 below using Elepa Manifold Platform as example that applies to all the Platforms.

Figure 7.3: The Pipeline Materials Selection – Elepa Manifold

The recommended line materials are given on table 7.2, on the basis of traffic lights assessment in the concert of cost, corrosion & corrosion management analysis:

Red representing: total unacceptability.

Amber representing: not totally acceptable because of some few limitations, including cost & maintainability, but could be managed with very strict CMS.

Green representing: total acceptance of the material.

The hydraulic and wall thickness calculations for the carbon steel were based on the API 5L X65 line pipe.

Table 7.2: Traffic Light Assessment

| TRAFFIC LIGHTS COMPARISON – based on Cost & Corrosion |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| s/n | Line Segment | MS | MS with 3 mm ca plus CI Injection | Duplex SS | MS with 10 mm ca plus CI Injection | Selected Material |
| 1 | 24” x 28.5 km ELEP – AKAB T/L |  |  |  |  | MS + 10 mm ca + Corrosion Inhibitor under very strict CMS |
| 2 | 16” x 9 km BBOU – ELEP B/L |  |  |  |  | MS + 10 mm ca + Corrosion Inhibitor |
| 3 | 16” x 10 m BBOU B/L |  |  |  |  | DUPLEX SS |
| 4 | 12” x 10 m ELEP B/L |  |  |  |  | DUPLEX SS |
| 5 | 24” x 45 km NUNR – ELEP B/L |  |  |  |  | MS + 10 mm ca + Corrosion Inhibitor under very strict CMS |
| 6 | 18” x 10 m NUNR B/L |  |  |  |  | DUPLEX SS |
| 7 | 20” x 20 km SEIB - NUNR B/L |  |  |  |  | MS + 10 mm ca + Corrosion Inhibitor under very strict CMS |
| 8 | 20” x 10 m SIEB B/L |  |  |  |  | DUPLEX SS |
| 9 | 24” x 37 km OPUG - BBOU B/L |  |  |  |  | MS + 10 mm ca + Corrosion Inhibitor under very strict CMS |
| 10 | 24” x 10 m OPUG B/L |  |  |  |  | DUPLEX SS |

Note:

- The Duplex SS are amber for the long distance pipelines owing to prohibitively high cost of installation, otherwise would be green on purely technical merit.

The cost, properties and, materials selection justifications were built up from bare MS through, MS with corrosion inhibition at 3mm & 10 mm ca (corrosion allowance) to CRA’s.

The conclusion that was drawn favoured Duplex SS for the Manifold in-plot short distance lines, and API 5L X65 MS for the long distance pipelines.  The API 5L X65 is more acceptable than CRA or Duplex SS, considering cost.

From the corrosivity potential of the transported fluid the MS with 10 mm ca and corrosion inhibitor injection is quite acceptable for the BBOU – ELEP B/L, while the other long distance P/L are only manageably acceptable requiring strict / stringent measures to be taken in terms CMS (Corrosion Management System) in order to achieve the desired service life.

  - Materials Specifications

The materials specifications for the various pipeline segments are follows:

- ELEPA – AKABEL 24” x 28.5 km TRUNKLINE / AKABEL MANIFOLD

- 24” API 5L X65, wt 31.75 mm for the Trunkline; with the use of Corrosion Inhibitor in the operational phase.

- 24” API 5L X65, wt 31.75 mm for the Trunkline; with the use of Corrosion Inhibitor in the operational phase.

- API 5L X65 for the pig barrels, and associated kicker line & balance line.

- Duplex SS – 25Cr UNS Code S32750 for Manifold In-plot Piping

- ELEPA F-MF – ELEP E-MF 12” x 10 m BULKLINE

- Duplex SS – 25Cr UNS Code S32750

- BUBOUWE – ELEPA 16” x 9 km BULKLINE / BUBOUWE MANIFOLD

- 16” API 5L X65, wt 31.75 mm for the Bulkline; with the use of Corrosion Inhibitor in the operational phase.

- 25Cr UNS Code S32750 Stainless Steel for the short piece 16” x 10 m In-plot Manifold Bulkline that offtakes BBOU production from SPDC BBOU Field Manifold Header for delivery into the BBOU – ELEB  Bulkline, either directly or through the Pig Barrel.

- Duplex SS – 25Cr UNS Code S32750 for Manifold In-plot Piping

- Manifold Platforms Power Supply Options

Ref: EA SPDC-BFPC UGS-GEP ELE RPT-001-A01_Concept Select Study – Manifold Platforms Power Supply Options

This chapter summarises the results of the above referenced Concept Select Study document in respect of the Materials Selection and Corrosion Analysis for the project.

The report covers the following subject areas:

- The Pipelines’ Materials Selection & Corrosion Analysis

- The Coating Materials Choice

- The Cathodic Protection System

  - Intent & Basis

Intent is Pipeline Integrity, and Materials Selection is the key to this intent.

The bases for the materials selection are the process & environmental conditions, and the operational requirements.

The process conditions constitute the physical conditions, - predominantly pressure and temperature, which the system must contend with in respect of stress; and the chemical conditions emanating from the impurities contained in the natural gas stream, - predominantly carbon dioxide, hydrogen sulfide (though not in this case), water vapor, oil, nitrogen, hydrates, which are capable of inducing internal corrosion and erosion of the system.