Report

© Epic Atlantic Limited

Epic Atlantic Limited

12 Kenneth Agbakuru Street, Lekki Phase I, Lagos, NG

www.epicatlantic.com | info@epicatlantic.com | +234 809 835 1008

This report contains confidential information and is intended only for the use of the individual named above. If you are not the named addressee you should not disseminate, distribute or copy this report without authorization.

SPDC-BFPC UPSTREAM GAS SUPPLY, GATHERING,

EVACUATION & PROCESSING (UGS-GEP) PROJECT

Materials Selection & Corrosion Analysis

Gas Gathering & Evacuation Pipelines

© Epic Atlantic Limited

Epic Atlantic Limited

12 Kenneth Agbakuru Street, Lekki Phase I, Lagos, NG

www.epicatlantic.com | info@epicatlantic.com | +234 809 835 1008

This report contains confidential information and is intended only for the use of the individual named above. If you are not the named addressee you should not disseminate, distribute or copy this report without authorization.

© Epic Atlantic Limited

Epic Atlantic Limited

12 Kenneth Agbakuru Street, Lekki Phase I, Lagos, NG

www.epicatlantic.com | info@epicatlantic.com | +234 809 835 1008

This report contains confidential information and is intended only for the use of the individual named above. If you are not the named addressee you should not disseminate, distribute or copy this report without authorization.

Prepared for:

Brass Fertilizer & Petrochemical Company Limited

Doc. No. EA/SPDC-BFPC UGS-GEP/MAT-RPT-001

Endorsed by:

Aloy Onyenucheya Duru, COREN (R.7580)

12th April 2017

© Epic Atlantic Limited

Epic Atlantic Limited

12 Kenneth Agbakuru Street, Lekki Phase I, Lagos, NG

www.epicatlantic.com | info@epicatlantic.com | +234 809 835 1008

This report contains confidential information and is intended only for the use of the individual named above. If you are not the named addressee you should not disseminate, distribute or copy this report without authorization.

© Epic Atlantic Limited

Epic Atlantic Limited

12 Kenneth Agbakuru Street, Lekki Phase I, Lagos, NG

www.epicatlantic.com | info@epicatlantic.com | +234 809 835 1008

This report contains confidential information and is intended only for the use of the individual named above. If you are not the named addressee you should not disseminate, distribute or copy this report without authorization.

© Epic Atlantic Limited

Epic Atlantic Limited

12 Kenneth Agbakuru Street, Lekki Phase I, Lagos, NG

www.epicatlantic.com | info@epicatlantic.com | +234 809 835 1008

This report contains confidential information and is intended only for the use of the individual named above. If you are not the named addressee you should not disseminate, distribute or copy this report without authorization.

Statement of Confidentiality

This Materials Selection & Corrosion Analysis Report may contain confidential and proprietary business information of Parties involved in this project. These materials may be printed or photocopied for use in evaluating the proposed project, but are not to be shared with other parties.

Epic Atlantic Limited | 12 Kenneth Agbakuru Street | Lekki Phase I, Lagos, NG | www.epicatlantic.com | info@epicatlantic.com

SUMMARY PAGE

| Client: BRASS FERTILIZER & PETROCHEMICAL COMPANY LIMITED. |  |
| --- | --- |
| Project: SPDC-BFPC UGS-GEP | Title: Materials Selection & Corrosion Analysis |
| Facilities: Gas Gathering & Evacuation Pipelines | Project Ref: EA/SPDC-BFPC UGS-GEP/MAT-RPT-001-A01 |

The administration of this document is in respect of the materials specification and integrity management of the Gas Gathering & Evacuation Pipelines aspects of the SPDC-BFPC Upstream Gas Supply, Gathering, Evacuation and Processing (UGS-GEP) Project. The document defines the basic design concepts, issues, and considerations in the subject matter of materials selection and corrosion analysis, and the attendant specifications and corrosion mitigation systems that will form the basis of the project’s Front End and Engineering Design (FEED). It is also an input & accompaniment to the conceptual study and basic design reports & presentations to DPR as procedural and statutory requirements in the process of securing regulators’ approval to progress to the FEED / Detail Design that will lead to the project’s FID and implementation. The report has relied and used data and reference documents provided by client, and if further information become available, Epic Atlantic reserves the right to update the opinion set out in this report. For better appreciation, this document should be read in conjunction with the references listed below, which are the Concept Select Study reports that constitute basis for the Conceptual Study.

KEY WORDS: BASIS FOR DESIGN, PIPELINE, MANIFOLD, PLATFORM, CORROSIVITY, NOMOGRAM

| REferences |  |
| --- | --- |
| Document No. | Description |
| TO-D-AA-00001 | Basis for Design |
| EA/SPDC-BFPC UGS-GEP/CSM-RPT-001 | Gas Gathering & Surface Facilities Concept Screening Study |
| EA/SPDC-BFPC UGS-GEP/SVY-RPT-001 | Pipeline Route Selection Study |
| EA/SPDC-BFPC UGS-GEP/PRC-RPT-001 | Process Simulation Study – HYSYS, PIPESIM, & OLGA |
| EA/SPDC-BFPC UGS-GEP/ELE-RPT-001 | Manifold Platforms Power Supply Options Study |

Table of Contents

1	Administration	11

1.1	Background and Introduction	11

1.2	Scope of Document	12

2	Material Selection And Corrosion Analysis : Pipeline Materials	13

2.1	Introduction	13

2.2	Purpose	14

2.3	The Principle For Materials Selection	15

2.4	Proposed List Of Pipelines	15

2.5	Methodology	16

2.5.1	Corrosion Rates in the Wet Gas Pipelines	16

2.5.2	Further Considerations of Additional Processes Selection by Fluid  Modification	17

2.6	Information And Data	17

2.6.1	Saturated Gas Compositions	17

2.7	Basic Design Data	18

2.8	Properties Of Optional Materials	19

2.8.1	Pitting Resistant Alloy Number	19

2.8.2	Materials Composition	20

2.9	Corrosion Rates Calculations Results	21

2.9.1	Bare Carbon Steel Corrosion Rates	21

2.9.2	Bare Carbon Steel with Inhibition	22

2.9.3	Corrosion Resistant Alloy (CRA)	23

2.10	Carbon Steel vs Corrosion Resistant Alloys wrt LCC	23

2.10.1	LCC Analysis For Materials Selection	23

Cost Estimate of Options	24

Option 1: CAPEX & OPEX Cost Elements for Carbon Steel with Inhibition for 1yr	25

Option 2: CAPEX & OPEX Cost Elements for 316 SSTL Material for 1yr	25

Option 3 : CAPEX & OPEX cost Elements for 6MOSSTL Material for Iyr	26

Option 4 : CAPEX & OPEX Cost Elements for 22 Cr Duplex SSTL Material	26

Option 5: CAPEX & OPEX Cost Elements for 25 Cr Duplex SSTL Material	27

Option 6 : CAPEX & OPEX Cost Elements for Titanium	27

2.10.2	Cost Summaries	28

2.10.3	CAPEX For the Options	28

Conclusion on the LCC Histogram	30

2.10.4	The use 10 mm higher level of CA and Inhibition	31

2.11	Comparison of Selected Line Materials in Traffic Lights	32

2.12	Conclusion and Recommendation.	33

3	Materials Selection And Corrosion Analysis : Coating Materials.	34

3.1	External Protection of Pipelines	34

3.2	Selection Coverage	34

3.3	Background	34

3.4	Scope	35

3.4.1	Basis of Design	35

3.4.2	Design Requirements	36

3.4.3	Design Life	36

3.5	Selected Coat	37

3.5.1	Coating Material	37

3.5.2	Coating Materials Composition	37

3.5.3	Coating Materials for Bend	38

3.5.4	Coating Materials for Field Joint	38

3.5.5	Coating Materials Repair Works	38

3.5.6	Surface Preparation	39

4	Cathodic Protection Design	40

4.1	Introduction	40

4.2	Project Description	41

4.3	Project Objective	42

4.4	Purpose	42

4.5	Scope	43

5	Cathodic Protection Design (Definition)	44

5.1	General Definition	44

5.2	Abbreviation	44

6	Cathodic Protection Design ( Technical Information/Data)	45

6.1	Design Parameters	45

7	Cathodic Protection Design ( Basis For Design)	48

7.1	Basis For Design	48

7.2	Choice Of CP System	48

7.3	Current  Requirement	49

7.3.1	Current Density	49

7.3.2	Current Demand Calculations	49

7.3.3	Sizing of Transformer – Rectifiers	49

7.3.4	Transformer –Rectifier for group of pipelines (n)	49

8	Design  For Temporary Protection System	51

8.1	Design Data	51

8.1.1	Current Demand	51

8.2	Characteristic of High Output Mg Anode (DEP 30.10.73.10 Gen)	51

8.3	Temporal Anode Spacing	53

8.4	Electrical Separation	54

9	Permanent CD Design	55

9.1	Design Basis	55

9.2	Detail Design Survey	55

9.3	Choice of Electrical Generation System.	55

APPENDIX	56

Appendix-1:	Well Gas Gathering Schematics	56

Appendix-2:	Proposed Pipeline Route	56

Appendix-3:	Pipeline Materials Selection & Manifold Platforms Indication	56

List of Tables

Table 2.1:  Saturation Gas Compositions	17

Table 2.2: Basic Design Requirement Data for Corrosion	18

Table 2.3: Properties of CRA Materials	19

Table 2.4: Typical PREN, CPT, & CCT (Source – R.T. Hull, F.A Ramrez, A.I Perez & B.A, Monty (2012))	19

Table 2.5: API 5L: Composition %	20

Table 2.6: 316 SS :Composition %	20

Table 2.7: 6MO SS : Composition %	20

Table 2.8: DUPLEX SSTL: Composition %	21

Table 2.9: SUPER DUPLEXSSTL : Composition%	21

Table 2.10: Summary of corrosion rate of the bare carbon steel.	22

Table 2.11: Summary of corrosion Rates of the MS is subject to Inhibition and CA of 3mm.	23

Table 2.12: Wall thickness Assessment	24

Table 2.13: Cost Element of Carbon Steel with inhibition	25

Table 2.14: Cost Element of 316 SSTL	25

Table 2.15: Cost Element of 6MO SSTL	26

Table 2.16: Cost Element of 22 Cr Duplex SSTL	26

Table 2.17: Cost Element of 25 Cr Duplex SSTL	27

Table 2.18: Cost Element of Titanium	27

Table 2.19: Summary of Capex Cost estimate	28

Table 2.20: Summary of OPEX	28

Table 2.21: Summary of LCC for Materials	29

Table 2.22: Traffic light Acceptability for CRA’s (Duplex SS)	31

Table 2.23: Corrosion Rates Of MS subject to 10mm CA and Inhibition	31

Table 2.24: Traffic Light Comparism	32

Table 3.1: Pipelines	36

Table 3.2: Pipelines	37

Table 3.3: Coating Materials Information	38

Table 3.4: Coating thickness	39

Table 6.1: Pipeline Data	46

Table 6.2: Criteria for protection Vs Reference electrode	47

Table 6.3:  Coating Breakdown for Design Purpose	48

Table 7.1: Surface Areas and Current Demand for the Pipelines CP.	51

Table 8.1:  Characteristics of the High Output Magnesium Anode	52

Table 8.2:   High Output Magnesium Anode composition	53

Table 8.3: Number of Magnesium Anode required per line for Temporary Protection (on the right column)	54

List of Figures

Fig 2.1 Base Concept	16

Fig 2.2 Cost Histogram	30

Fig 4.1: Propose Pipelines’ Routes (to be Updated before release of document)	43

Abbreviations

The Following definitions and abbreviations have been used in this document.

Organisational

| BB | Bubuowei Bou |
| --- | --- |
| BFPC | Brass Fertilizer Petrochemical Company Limited |
| BYS MEnV | Bayelsa State Ministry of Environment |
| DPR | Department of Petroleum Resources |
| EA | Epic Atlantic Limited |
| FEPA | Federal Environmental Protection Agency |
| FMEnV | Federal Ministry of Environment |
| LGA | Local Government Area |
| NGO | Non Governmental Organisation |
| SPDC | The Shell Petroleum Development Company of Nigeria Limited |

Technical Terms

| B/L | Bulkline |
| --- | --- |
| BFD | Basis For Design |
| CAS | Community Affairs, and Security |
| CPT | Cone Penetrometer Testing, Cone Penetration Test |
| EDMS | Electronic Data Management System |
| EIA | Environmental Impact Assessment |
| EMP | Environmental Management Plan |
| EPC | Engineering, Procurement, and Construction |
| FEED | Front End Engineering Design |
| FID | Final Investment Decision |
| FDP | Field Development Plan |
| FP | Full Preparatory |
| GPS | Global Positioning System |
| HAZID | Hazard Identification |
| HRA | Health Risk Assessment |
| HSE | Health Safety and Environment |
| IFC | Issued for Construction |
| IFD | Issued for Design |
| IFR | Issued for Review |
| ISO | International Standard Organisation, Isometric Drawing |
| MTO | Material Take Off |
| OML | Oil Mining Lease |
| OPL | Oil Pipeline License |
| PFD | Process Flow Diagram |
| P&ID | Piping & Instrumentation Diagram |
| P.I. | Point of Intersection |
| PI | Preliminary Investigation |

Technical Terms – contd.

| POS | Project Overview Statement |
| --- | --- |
| PTS | Permit To Survey |
| QA/QC | Quality Assurance / Quality Control |
| QC | Quality Control |
| QCed | Quality Checked |
| QMS | Quality Management System |
| RFQ | Request for Quotation |
| ROW | Right of Way |
| SPT | Standard Penetrometer Testing |
| T | Pipeline Tie-in Point |
| WHIS | Worksite Hazard Information System |

Units of Measurements

| Ha | Hectare |
| --- | --- |
| km | Kilometre |
| m | Metre |
| M | Thousand |
| MM | Million |
| MMscfd | Million Standard Cubic Feet per Day |
| MTPD | Metric Tonnes Per Day |

# Administration

This Materials Selection & Corrosion Analysis Study is in respect of the SPDC-BFPC Upstream Gas Supply, Gathering, Evacuation and Processing (UGS-GEP) Project.  It stems from the Concept Select Study on the Gas Gathering Concept, and presents the selected design concept as basis for the Basic Design of the project.

It is also an input and accompaniment to the Conceptual Study & Basic Design Memoranda to be presented to DPR as statutory requirements in the process of securing regulators’ approval to execute the FEED / Detail Design that will lead to the project’s FID and implementation.

## Validity & Change Management

The report has relied and used data and reference documents provided by client.  Some of the data are preliminary, based on limited information, but to a large extent, however, are adequate for this phase of work: - conceptual study and basic design.

For example, analogue PVT data was used in the absence of actual PVT, and preliminary pipeline distances were estimated based on desktop route selection study.  These design data, and the consequent simulation results will invariably need to be updated during FEED, by which time actual PVT data would have been obtained, and the FP survey done to establish actual distances and elevations of the pipeline routes.

Similarly, the project timelines as set out in the execution schedule was predicated on FID date of June 2018, and that will correspondingly change with actual FID date.

Also, the security situation in the region with its wide spate of vandalism may dictate that certain design provisions be changed during the FEED.  The situation will continue to be monitored, and if still perceived to be a cog in the wheel of innovation, some of the ideal designs will be replaced with what will be practically suitable for the situation.

For example, in as much as it is feasible to lay cable from the IPP to the remote manifold platforms to transmit electricity to motorise the platform valves in the Manifold Platforms Power Supply Options, or install solar systems on the platforms, there will be no need to implement such options if the facilities (cables & solar systems) will eventually be vandalised in no time.  Instead, it will be rather more appealing to limit the process option to manual valve operations, without the features of valve motorisation that will require electricity.

Epic Atlantic therefore reserves the right to update accordingly, the opinion set out in this report, as further information and / or refinement become available.

## Background and Introduction

The SPDC-BFPC Upstream Gas Supply, Gathering, Evacuation and Processing (UGS-GEP) Project is constituted by Brass Fertilizer & Petrochemical Company Limited (BFPC), in partnership with The Shell Petroleum Development Company of Nigeria (SPDC).

The project scope includes Gas Gathering Pipelines, and a Gas Process Processing Plant that will provide processed gas as feedstock to a Petrochemical Plant and a Power Plant.  The Gas Processing Plant will process natural gas gathered from five SPDC fields – Bubouwe Bou, Elepa, Nun River, Seibou, and Opugbene - in OMLs 33, 32, & 36.

The Gas Plant will comprise three trains, (each with a processing capacity of 150 MMscfd) and condensate storage capacity of 50,000 Tonnes. In this respect, BFPC have concluded the Gas Supply & Processing Agreement (GSPA) with SPDC for 270 MMscfd of the natural gas feedstock.  The Plants (Gas Plant, Petrochemical Plant, and Power Plant) will all be located in the same complex at Akabel, Brass Island, Bayelsa state of Nigeria, near the shores of Atlantic Ocean.

The Petrochemical Plant will comprise a Methanol Plant of 1.75 MTPA capacity, and a Urea Plant of 2.6 MTPA total capacity, built in two phases of 1.3 MTPA each. It will also have storage capacity for 150,000 Metric Tonnes of Methanol and 100,000 Metric Tonnes of Urea.  The facilities complex will also comprise a captive export jetty of 4 berths for up to 35,000 DWT vessels.

The current on-stream date for the project is December 2020, and to achieve that, FID is planned for June, 2018.  In pursuit of these business objectives, BFPC have virtually concluded Project Management & EPC contracts with EIL, Saipem, and TCC, who are internationally renowned contractors in the implementation of the subject projects type.

The Gas Processing Plant, and the associated Gas Gathering System / Pipelines which will transport wellhead gas from the SPDC fields to the Gas Processing Plant, constitute the key upstream enabler facilities for the realisation of the objective supply of processed gas supply as feedstock to the Petrochemical Plant for the product manufacturing of methanol & urea, and to the Power Plant for power generation.

## Scope of Document

The scope of the document covers the basic design concepts, issues, and considerations in the subject matter of materials selection and corrosion analysis, and the attendant specifications and corrosion mitigation systems, structured as follows:

- Pipelines’ Materials Selection & Corrosion Analysis

- Line Pipe Specification

- Coating System Selection & Specification, and

- Cathodic Protection System

## The Pipelines Location

The pipelines location is marshy swampy terrain of Niger Delta of mangrove vegetation, as depicted in the pictorial below.

Fig 1.1:	Situation Map of the Pipeline Network

# Pipeline Material Selection And Corrosion Analysis

## Intent & Basis

Intent is Pipeline Integrity, and Materials Selection is the key to this intent.

The bases for the materials selection are the process & environmental conditions, and the operational requirements.

The process conditions constitute the physical conditions, - predominantly pressure and temperature, which the system must contend with in respect of stress; and the chemical conditions emanating from the impurities contained in the natural gas stream, - predominantly carbon dioxide, hydrogen sulfide (though not in this case), water vapor, oil, nitrogen, hydrates, which are capable of inducing internal corrosion and erosion of the system.

The environmental condition represents the ambient, which if corrosive will cause external corrosion of the system.

Both corrosions (internal & external) are undesirable and have to be eliminated or mitigated.

The operational requirements constitute consideration of design, construction, operations, maintenance, threats, hazards, risks, safety and economics; and codes and standards that provide mandatory and optional requirements and guidelines.

Similarly, the bases for the corrosion analysis are the susceptibility of the material to corrosion, the corrosivity potential of the transported fluid that will cause internal corrosion to the pipeline system, and the environment which, if corrosive, will cause external corrosion.

These premises will be followed in the materials selection & corrosion analysis process to ensure proper selection of materials and the relational corrosion management system.

## Purpose & Objective

The purpose is three-fold:

- Identification of best pipeline materials for each of the line segments at optimised life cycle costs, utilising best industry practice.

- Assessment of the risks associated with the specific selection.

- Specification of the corrosion management system that will mitigate the risk.

The objective is the optimal specification of materials and the relational corrosion management systems that can contend with the process & environmental conditions, and operational requirements of the pipeline system.

## Materials Selection Principles & Criteria

The material selection process follows an evaluation of the systems materials in consideration with the composition of the streams to which the various pipe segments, pieces of equipment, piping and fittings are exposed as well as the temperatures, pressures and velocities which prevail. The steady state conditions of operation are important but equally any excursions which occur during start-up, shut-down or upsets can be key determinants of materials selection and corrosion management.

The following are factors that have been considered in the selection of materials grade, in order to achieve some degree of protection at optimum costs:

  - Design Life

  - Properties of Internal Fluid

  - Surrounding Environment

  - Resistance Level to Corrosion Effect

  - Inspection & Corrosion Monitoring Possibilities

  - Operating & Maintenance Philosophy, and Degree of System Redundancy

  - Mechanical Properties

  - Wall Thickness

  - Weldability

  - Temperature Resistance

  - Weight Requirement

  - Fatigue and Fracture Resistance

  - System Availability Requirement

  - Fabrication and Installation Methods and Procedures

  - Compatibility with Adjoining Materials

  - Market Availability of Materials

  - Local Experience (both with the materials and the corrosion management system that will apply)

  - Economics

## Materials Options

The following materials are considered pre-qualified for the evaluation.

- Mild Steel (MS) - Carbon Manganese Steels

- MS plus Inhibition

- Various CRA’s (Austenitic SS, Ferritic SS. Martensitic SS, Duplex SS)

- Precipitated Hardened Steel (with Al)

- Pipeline with Internal CRA Lining or Cladding

- Composite Materials

- Non-Ferrous Materials

Despite the corrosion rates which occur when carbon steel is exposed to wet CO2 it can be a preferred material because of its low cost. In the presence of H2S corrosion rates may be reduced due to the formation of a protective iron sulphide film. Low temperature carbon steels are also available and are sometimes used to avoid embrittlement when low temperatures might be encountered during blowdown. The corrosion resistance is similar to that of carbon steel.

For the carbon steel pipelines, corrosion allowance and corrosion inhibition form the basis for design life achievement. Where this combination is considered to be insufficient, corrosion resistant alloys are used.

The pragmatic approach therefore rests on the exhaustive investigation of the various material options for the specific function and application.

In this respect, a key factor for the material selection is an up-front activity taking into consideration codes & standards, operational flexibility, cost, availability, sourcing, and the performance of the material for the intended service and application; with due considerations to recommendations of design engineers, materials specialists, manufacturers, installers and operators.

The technical feasibility demonstration for the selection of the appropriate materials follows a  	four stage evaluation.

- Feasibility of Mild Carbon Steel, involving the following steps:

  - The prediction of the potential corrosivity of the bare carbon steel

  - Calculation of the Corrosion Allowance (CA) Life cycle period

  - Determination of the acceptance of the selected material (on a Green, Yellow, and Red traffic light colouration system).

- Feasibility of Mild Carbon Steel, through the following steps:

  - The prediction of the corrosivity

  - Calculation of the improved or new corrosivity using the 85% inhibition efficiency and 10% non – availability.

  - Determination of acceptance of the materials on traffic light colouration

- Feasibility of CRA materials through the following steps:

  - Lifetime Cost Cycle (LCC) comparison of various CRA materials with MS

  - Selection of the CRA on the basis of NPV economic evaluation

  - Acceptance determined on traffic light colouration.

- Optional Process for Modification of the Fluid Content through the following steps:

  - Dehydration of the Wet Gas, at the Manifolds or

  - The Removal of CO2 from the Wet Gas at the Manifolds

  - Brief Review of the processes and  systems available

  - Define critical parameter limits to achieve corrosion control goals

  - Select one of the options on the basis of economic review

  - Define the parameters for design

The report will be run stepwise with the materials’ acceptance defined in traffic colours.

Red represents: rejection

Yellow represents: not totally acceptable because of some few limitations including costs, properties, maintainability etc.

Green represents total acceptance of the material

## The System & Fluid Under Consideration

The system under consideration is the pipeline network as shown in fig 2.1 below.

It comprises the following pipeline segments, coming into service in phases, and transporting raw wellhead gas to the Gas Processing Plant:

- 24” x 28.5 km ELEP – AKAB Trunkline

- 16” x 9 km BBOU – ELEP Bulkline

- 24” x 45 km NUNR – ELEP Bulkline

- 20” x 20 km SEIB – NUNR Bulkline, and

- 24” x 37 km OPUG – BBOU Bulkline

- Each of the five Field Manifold Headers of about 10 m length.

The fluid transported is high pressure category D/E fluid (flammable fluid), in the pressure & temperature regions of 100 barg, and 60 oC, and containing CO2 that causes corrosion.

However, it contains no H2S that causes sour corrosion, or Hg that causes liquid metal embrittlement (LME) of alloys, or Naturally Occurring Radioactive Materials (NORM) which can accumulate to render the system radioactive over time.

Figure 2.1: The Pipeline Network

### The Fluid Impurities Composition

The impurities composition in the various reservoir fluids are shown in the table below:

Table 2.1: Reservoirs Conditions & Impurities Compositions (Water Saturated Compositions)

|  | Impurities Compositions - Mole % |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Component | BBOU | ELEP | NUNR | SIEB | OPUG |
| N2 | 0.07 | 0.00 | 0.00 | 0.00 | 0.10 |
| CO2 | 0.59 | 2.17 | 0.77 | 1.53 | 2.21 |
| H2S | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 |
| H2O | 0.34 | 2.19 | 0.63 | 0.95 | 1.08 |
| Total | 1.00 | 4.36 | 1.40 | 2.48 | 4.11 |
| Reservoir Conditions |  |  |  |  |  |
| Reservoir Pressure (bar) | 223.8 | 476.6 | 500.6 | 450.6 | 356.9 |
| Reservoir Temp (oC) | 78 | 123 | 102 | 117 | 113.8 |
| pCO2 (bar) | 1.32 | 10.34 | 3.85 | 6.89 | 7.89 |

At the deducible partial pressures of CO2 (pCO2) above, the gas streams will result in significant corrosion of the pipeline system, judging from both the corrosion prediction calculation results from DeWaard-Milliams Model, and its CO2 Corrosion Nomogram shown below.

Fig 7.2: DeWaard-Milliams CO2 Corrosion Nomogram

### Issues & Considerations

Of the essence will be pressure containment by the system as well as corrosion prevention / mitigation in the system; and in that respect the following issues were given due considerations:

- Materials of good strength & properties to withstand the high pressure under consideration and corrosive nature of the fluid to be transported.

- High Pressure Transportation of Category D/E Fluid (flammable fluid), in the pressure region of 100 barg, and temperature region of 60 OC.

- The Gas is potentially corrosive on account of its CO2 content; and water, NGL, & particulates that are inherently present

- CO2 content is up to 2%, which for the empirically established 0.1 bar partial pressure treshold for significant CO2 corrosion, implies corrosion of unprotected mild CS at pressures above 5 barg

- The CO2 content of up to 2%, renders the gas corrosive at pressures higher than 5 barg (corresponding to the empirically established 0.1 bar partial pressure treshhold for significant CO2 corrosion), - prompting the need for CRA materials or matching corrosion management design.

- The Carbon Steel option would mean more internal wall thickness plus corrosion allowance

- Internal corrosion will be substantial for which the traditional (normal) corrosion allowance of 3 mm will be insufficient provision.

- Higher corrosion allowance, up to the traditional maximum of up to 10 mm will therefore be considered, and in that respect, high yield strength material with smaller wall thickness demand will be used, so as to keep the overall design wall thickness as small as possible.

- Ambient environment is corrosive owing to joint contribution of humidity, acidity, and oxygen in air, and known potentially corrosive nature of the soil in the area.

- Temporal Protection of the linepipes / pipelines, - that will give progressive protection during the period of construction, until the ICCP system is put in place

### Corrosion Rates in the Wet Gas Pipelines

The methodology that will be used for the selection of the material will be on the basis of 	internal corrosion impact on the pipeline and will be as follows:

- The Calculations for Corrosion Allowance and Comparison with CRA’s through:

- Potential Corrosion Prediction of MS Material utilising:

  - De-Waard and Milliams model (NACE 91 model) for MS Corrosion Rate Calculations and /or

  - De- Waard – Milliams Nomogram checks and further comparison of rates with adjustment for gas rates incorporated

- Calculations of the Corrosion Allowance for the MS and determination of the risks (failure /replacement Likelihood)

- Application of inhibitor, considering 85% Inhibitor Efficiency and 10% non-availability in the corrosion rates calculations (based on local experience and NACE 	standards), and further determine the improved risks

- Comparisons with alternative materials in terms of Life Cycle Costs {LCC}

- Comparison with selected CRA’s in terms of properties.

- Hydraulic Studies for the Pipeline Diameter (Normally, carried out Separately)

- B31.8 Wall Thickness Calculations.

The wall thickness is normally determined by installation stresses and service pressures, the design factors, the CA, and the fabrication tolerance.

### Further Considerations of Additional Processes Selection by Fluid Modification

The transported fluid may be treated to reduce moisture & carbon dioxide contents which are the main causes of corrosion for the system under consideration, - through the following ways:

- Use of Glycol / Desicants to reduce moisture below the threshold that supports corrosion

- Use of Membrane Separation, Physical or Chemical Solvent to Removal of CO2

However these processes will mean installation of facilities on the wellhead platforms, which will be very challenging in terms of cost, environmental disposal in terms of waste 	water into the inlet water ways and the big foot print in the manifolds with increased security 	and intervention needs; and are therefore not considered viable in this situation.

## Information and Data

Various information are needed for the compilation and analysis to attain the internal 	corrosion rate, compare options and select the most suitable material for each of the 	pipelines.  Information and their sources are given thereof:

### Saturated Gas Compositions

Table 2.2:  Saturation Gas Compositions

|  |  | Reservoir Composition (Mol %) |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Composition | Mole Weight | BBOU | ELEP | NUNR | SEIB | OPUG |
| N2 | 28.0 | 0.07 | 0.00 | 0.00 | 0.07 | 0.10 |
| CO2 | 44.0 | 0.59 | 2.17 | 0.77 | 1.53 | 2.21 |
| H2S | 34.1 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 |
| C1 | 16.0 | 92.61 | 79.87 | 72.37 | 77.84 | 67.24 |
| C2 | 30.1 | 3.15 | 6.00 | 7.63 | 5.87 | 7.88 |
| C3 | 44.1 | 1.01 | 2.53 | 4.82 | 3.37 | 4.76 |
| iC4 | 58.1 | 0.61 | 0.73 | 1.45 | 1.21 | 1.94 |
| nC4 | 58.1 | 0.44 | 0.79 | 1.96 | 1.37 | 1.06 |
| iC5 | 72.2 | 0.25 | 0.44 | 0.94 | 0.82 | 1.15 |
| nC5 | 72.2 | 0.19 | 0.32 | 0.76 | 0.57 | 0.68 |
| C6 | 86.2 | 0.23 | 0.64 | 1.02 | 0.91 | 1.06 |
| C7+ | 143 - 169 | 0.53 | 4.31 | 7.65 | 5.48 | 10.86 |
| H2O | 18.01 | 0.34 | 2.19 | 0.63 | 0.95 | 1.08 |

From the table above the main corrosion specie found is CO2 in Gas i.e. sweet corrosion. 	The mechanism is CO2 and oxygen corrosion, - both TOL and BOL attacks are expected

### Basic Design Data

The table 2.3 below gives the relevant design data.

Table 2.3: Basic Design Requirement Data for Corrosion

| S/N | Description | DATA |
| --- | --- | --- |
| 1 | System | Wet Gas Transportation Pipelines From BBOU, ELEP, NUNR, SEIB, & OPUG Fields |
| 2 | Gas Production | 300 MMscfd |
| 3 | Pressure Ranges | 60 -100 bar |
| 4 | Velocity | 6 – 12.5 m/s |
| 5 | Temperatures | 44 – 100 Degree C (cf dew points 100oC – 250oC) |
| 6 | Mode | Automation |
| 7 | Design Life | 30 years |
| 8 | Design Codes/ Standards | ANSI / ASME 31.8 NACE |
| 9 | Pressure Class | ANSI 900 (Class 900) / API 3000 |
| 10 | Dew Points | 100oC – 250oC (cf temps 44oC – 100oC) |
| 11 | Water Production | Initial water prod is ca. 420 bpd, but will increase to 10,000 bpd following water breakthrough |
| 12 | Condensate Production | ca. 12.5 Mbpd |

Important to note is that stream temperature is well above the dew point of gas stream, implying that there will be water drop out that will initiate & propagate corrosion.

## Properties of Optional Materials

The table below gives an overview of the SS CRA materials,

Table 2.4: Properties of CRA Materials

| Properties | Austentic SS | Ferritic SS | Duplex SS | Martensitic SS | Precipitation Hardened |
| --- | --- | --- | --- | --- | --- |
| Strength | = | X | XX | XXX | XXX |
| Wear Resistance | = | = | X | XX | XX |
| Formability | XXX | XX | XX | = | X |
| Weldability | XX | = | XX | = = | = |
| Resistance to General Corrosion | XX | X | XX | = | X |
| Resistance to Pitting Corrosion | XX | X | XX | = | X |
| Resistance to Stress Corrosion Chloride | = | XX | XX | = | = |

Legend:	X - Superior properties		= - Lower properties

### Pitting Resistant Alloy Number

This number is used to determine susceptibility to pitting corrosion and their critical temperatures.

Table 2.4: Typical PREN, CPT, & CCT

(Source – R.T. Hull, F.A Ramrez, A.I Perez & B.A, Monty (2012))

| Material | PREN | CPT (Deg. Centigrade) | CCT (Deg. Centigrade) |
| --- | --- | --- | --- |
| 316 | 24 – 26 | 5 -15 | <5 |
| 6MO | 46 | 65 -80 | 30 -60 |
| 22Cr Duplex SS | 35 | 20 - 41 | 17.5 – 25 |
| 25 Cr Duplex SS | 42 | 55 -88 | 33 – 43 |

Note

PREN – Pitting Resistance Equivalent Number (at 40 and above, no pitting is expected)

CPT – Critical Pitting Corrosion Temperature

CCT – Critical Crevice Corrosion Temperature

### Materials Composition

- API 5L: Composition %

Table 2.5: API 5L: Composition %

| P/L Grade | C | Mn | Si | Al x102 | Ca x103 | Ni | N x102 | Cu | V x102 | Nb x102 | Ti x102 | B x103 | P x103 | S x103 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| API 5L | 0.16 | 1.56 | 0.35 | 4 |  |  | 1.2 |  | 7 | 5 |  | 1 | 3 | 15 |

- 316 SS :Composition %

Table 2.6: 316 SS : Composition %

| Grade | UNS Code | C | Si | Mn | P | S | Cr | Ni | Mo | N |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 316SSTL | TP | 0.08 | 0.75 | 2 | 0.045 | 0.03 | 18 | 14 | 3 | 0.10 |

- 6MO SS : Composition %

Table 2.7: 6MO SS : Composition %

| Grade | UNS Code | C | Si | Mn | P | S | Cr | Ni | Mo | N |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 6MO | S31254 | 0.02 | 0.70 | 1 | 0.30 | 0.010 | 20 | 18 | 61 | 0.20 |

- DUPLEX SSTL: Composition %

Table 2.8: DUPLEX SSTL: Composition %

| Grade | UNS Code | C | Cr | N1 | M | W | Cu | N |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| DUPLEX | S31803 | 0.02 | 22 | 5.5 | 3.0 |  |  | 0.17 |
|  | S32205 |  | 22.5 | 5.8 | 3.2 |  |  | 0.17 |

- SUPER DUPLEXSSTL : Composition%

Table 2.9: SUPER DUPLEX SSTL: Composition%

| Grade | UNS Code | C | Cr | N1 | M | W | Cu | N |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| SUPER DUPLEX | S32750 | 0.02 | 25 | 7 | 4 |  | 0.5 | 0.23 |
|  | S32760 | 0.03 | 25 | 7 | 3.5 | 0.6 | 0.5 | 0.25 |

## Corrosion Rates Calculations Results

### Bare Carbon Steel Corrosion Rates

The corrosion rate prediction calculation were based on De Waards and Milliams model 	(NACE 91), - given by the formula below:

Log Vcor = 5.8 – [1710/ (t +273)] + 0.67 log (PCO2)

pCO2 – Partial pressure of CO2 in the gas in equilibrium with water, bar

t – Temperature oC (the simulated temperatures were given & used

Vcor – Corrosion Rate

All rates calculated were cross checked with De Waard and Milliam monogram to ensure 	accuracy of the rates calculated.

The table below illustrates the summary of the various calculations undertaken

Table 2.10: Summary Of Bare Carbon Steel Corrosion Rates.

The results, from the table 2.10 above, indicate that all the MS pipelines reviewed, have very high corrosion rates. The MS bare steel materials are not found to be suitable as the pipeline material for this project. Note that the percentage life time Corrosion Allowance compared to the recommended Corrosion Allowance must be less than 100% for the material to be 	suitable for purpose. But this is not so. It was found that there were rates comparison between 2667% and 4430%. These are indications of very high corrosion rate. And consequently, this will lead to intolerable high running costs and intervention throughout the project life. Conclusively:

- The bare mild steel is not acceptable in all cases under review.

- Modifiers of the fluid are needed to reduce these very high corrosion rates. Hence, the need inhibition was further reviewed.

### Bare Carbon Steel with Inhibition

The table 2.11 shown below is the summary of the corrosion rate prediction for mild steel 	with inhibition. The inhibition efficiency is considered to be 85%, based on NACE standards. And the availability of the pipeline is 90% (ie for an estimated period of 329 days per year). This is conservative considering that the area is a high flash point for militancy. This may further affect operations.

The formula utilised for calculation is given below

CR = F x CRm + (1-f) x Cru

F = availability = fraction of time corrosion control system is working (to be obtained from 	corrosion monitoring) NACE standard 85% to be obtained from monitoring.

CRm = mitigated corrosion rate.

Cru = unmitigated corrosion rate.

Corrosion rate accuracy generally controlled by accuracy of availability and input data not 	by model.

CA = corrosion allowance calculation

CA = Cri N(1-D/365) + CRv N D/365

Table 2.11: Summary of corrosion Rates of the MS is subject to Inhibition and CA of 3 mm.

Table 2.11

Overall this option showed a lot of improvement on the corrosion rates but still did not achieve the requirement for the design life cycle, except for the 16”x 9 km BBOU – ELEP bulkline.

In conclusion:

- The inhibition has not been able to sufficiently alleviate the corrosion to acceptable levels.

- Only one line is found to be just okay with less stringent standards (16” x 9 km BBOU – ELEP BL)

- Consideration for the need for CRA has now become very relevant on the basis of the corrosion rates obtained.

### Corrosion Resistant Alloy (CRA)

The NPV of the CRA is now relevant in comparison with bare carbon steel under inhibition. 	This is treated in the section 2.10 below.

## Carbon Steel vs Corrosion Resistant Alloys wrt LCC

### LCC Analysis For Materials Selection

This LCC is done for further investigation of different materials options at this Engineering 	stage and to provide the best materials option at the best LCC.

The options selected for this evaluation include:

- Mild Steel Inhibited

- 316 SSTL

- 6MO SSTL

- 22Cr Duplex SSTL

- 25Cr Duplex SSTL

- Titanium

For a balanced comparison and uniform appraisal, 24” lines used are given below with 	their wall thickness assessed on standard schedule.

Table 2.12: Wall thickness Assessment

| PIPE MATERIAL | MATERIAL STANDARD | DIA (INCH) | DIA (mm) | PRESURE class | WALL THICKNESS | SCH |
| --- | --- | --- | --- | --- | --- | --- |
| Carbon Steel | A333 Gr6 API 5L x52 | 24 | 610 | 1500 | 34,93 | 80 |
| 316 SSTL | A312 TP316 | 24 | 610 | 1500 | 53,34 | 140 |
| 6MO SSTL | A312 S31254 | 24 | 610 | 1500 | 31,75 | 80 |
| 22Cr Duplex SSTL | A790 S31803 | 24 | 610 | 1500 | 28,58 | 60 |
| 25Cr Duplex SSTL | A790 S32750/760 | 24 | 610 | 1500 | 22.23 | 40 |
| Titanium | B861 Gr2 | 24 | 610 | 1500 | 22.23 | 40 |

### Cost Estimate of Options

The following assumptions are made for the costs estimation comparison of the different optional CRA’s:

- The line types are standardised at 24” for uniformity

- The length of all lines are 100 m for the LCC calculation

- The expected life used is 25 years

- The inhibited CS corrosion at appendix – indicate the likelihood one or more 	replacements within 25year period. One replacement at 15th year assumed.

- The uncertainty factor of 2% inflation and discounted rate of 10% for present worth 	value

- The cost of differed production not included

- Cost of Clamped leaks not included.

### Option 1: CAPEX & OPEX Cost Elements for Carbon Steel with Inhibition for 1yr

Table 2.13: Cost Element of Carbon Steel with inhibition

| CAPEX Cost Element | $ meter/weld | Cost for 100 m length($) | OPEX Cost Element | $ |
| --- | --- | --- | --- | --- |
| Pipe Material | 1,756.48 | 175,648.00 | Inhibition | 218,846.66 |
| Flanges & Fitting |  | 60,603.12 | Inspection | 20,000.00 |
| Fabrication | 3,378.29 | 236,459.85 | Man Hour | 116,482.69 |
| Installation | 168.89 | 16,218.00 | Energy | 8,736.20 |
| Commissioning | 58.24 | 5,824.00 | Spare | 15876.60 |
| Design & Admin | 58.24 | 5,824.00 |  |  |
| Total |  | 500,576.97 |  | 379,942.15 |

### Option 2: CAPEX & OPEX Cost Elements for 316 SSTL Material for 1yr

Table 2.14: Cost Element of 316 SSTL

| CAPEX Cost Element | $ meter/weld | Cost for 100 m length | OPEX Cost Element | $ |
| --- | --- | --- | --- | --- |
| Pipe Material | 8,771.20 | 877,120.00 | Inhibition | 0 |
| Flanges & Fitting |  | 238,148.84 | Inspection | 12,736.20 |
| Fabrication | 5,411.15 | 307,397.85 | Man Hour | 116,482-69 |
| Installation | 188.50 | 18,850.00 | Energy | 8,736.20 |
| Commissioning | 58.24 | 5.824.00 | Spare | 41,887.16 |
| Design & Admin | 58.24 | 5,824.00 |  |  |
| Total |  | 1,454,164.69 |  | 179,842.25 |

### Option 3 : CAPEX & OPEX cost Elements for 6MOSSTL Material for Iyr

Table 2.15: Cost Element of 6MO SSTL

| CAPEX Cost Element | $ meter/weld | Cost for 100 m length | OPEX Cost Element | $ |
| --- | --- | --- | --- | --- |
| Pipe Material | 14,472.95 | 1,447,295.95 | Inhibition | 0 |
| Flanges & Fitting |  | 909,821.44 | Inspection | 12,736.20 |
| Fabrication | 5,375.65 | 376,297.33 | Man Hour | 116,482.69 |
| Installation | 188.50 | 18,850.00 | Energy | 8,736.20 |
| Commissioning | 58.24 | 5,824.00 | Spare | 144,729.13 |
| Design & Admin | 58.24 | 5,824.00 |  |  |
| Total |  | 2,763,912.72 |  | 282,684.22 |

### Option 4: CAPEX & OPEX Cost Elements for 22 Cr Duplex SSTL Material

Table 2.16: Cost Element of 22 Cr Duplex SSTL

| CAPEX Cost Element | $ meter/weld | Cost for 100 m length | OPEX Cost Element | $ |
| --- | --- | --- | --- | --- |
| Pipe Material | 9,796.171 | 97,9617.1 | Inhibition | 0 |
| Flanges & Fitting |  | 270,239.84 | Inspection | 12,736.20 |
| Fabrication | 5,375.65 | 376,297.33 | Man Hour | 116,482.69 |
| Installation | 188.5 | 18,850.00 | Energy | 8,736.20 |
| Commissioning | 58.24 | 5,824.00 | Spare | 59,115.00 |
| Design & Admin | 58.24 | 5,824.00 |  |  |
| Total |  | 1,656,594.27 |  | 200,070.06 |

### Option 5: CAPEX & OPEX Cost Elements for 25 Cr Duplex SSTL Material

Table 2.17: Cost Element of 25 Cr Duplex SSTL

| CAPEX Cost Element | $ meter/weld | Cost for 100 m length | OPEX Cost Element | $ |
| --- | --- | --- | --- | --- |
| Pipe Material | 10,134.92 | 1,013,492 | Inhibition | 0 |
| Flanges & Fitting |  | 405,359.77 | Inspection | 12.736.20 |
| Fabrication | 5,375.65 | 376,297.33 | Man Hour | 116482.69 |
| Installation | 188.5 | 18,8850.00 | Energy | 8736.20 |
| Commissioning | 58.24 | 5824.00 | Spare | 84,448,00 |
| Design & Admin | 58.24 | 5824.00 |  |  |
| Total |  | 1,825,647.1 |  | 222,403.09 |

### Option 6 : CAPEX & OPEX Cost Elements for Titanium

Table 2.18: Cost Element of Titanium

| CAPEX Cost Element | $ meter/weld | Cost for 100 m length | OPEX Cost Element | $ |
| --- | --- | --- | --- | --- |
| Pipe Material | 17565.30 | 1,756,530.00 | Inhibition | 0 |
| Flanges & Fitting |  | 1520099.12 | Inspection | 0 |
| Fabrication | 5,375.65 | 364823..0 | Man Hour | 116482.69 |
| Installation | 188.5 | 188850 | Energy | 5824.00 |
| Commissioning | 58.24 | 5824.00 | Spare | 330810.83 |
| Design & Admin | 58.24 | 5824-00 |  |  |
| Total |  | 3,671,985.13 |  | 453117.52 |

### Cost Summaries

Note from Appendix – it is predictive and most likely that carbon steel with inhibitor may be 	replaced at about fifteen years of LCC and at an inflated cost of $238,137 considering 2% 	inflation rate for the dollar. Hence one replacement of CS was used for the costing.

### CAPEX For the Options

Table 2.19: Summary of Capex Cost estimate

|  | Carbon Steel with Inhibition | 316SSTL | 6MOSSTL | 22Cr Duplex SS | 25 Cr Duplex SS | Titanium |
| --- | --- | --- | --- | --- | --- | --- |
| Initial Capex | 500,577 | 1,454,165 | 2,763,912 | 1,656,594 | 1,825,647 | 3,671,985 |
| 15 yrs Later (Repl) | 673,711 | 0 | 0 | 0 | 0 | 0 |
| Total | 1,114,287 | 1,454,165 | 2,763,912 | 1,656,594 | 1,825,647 | 3,671,985 |

Table 2.20: Summary of OPEX

| Year | Carbon Steel with Inhibitor | 316 SSTL | 6MO SS TL | 22 Cr Duplex SSTL | 25 Cr Duplex SSTL | Titanium |
| --- | --- | --- | --- | --- | --- | --- |
| 1 | 784,729 | 344937 | 542,188 | 387,734 | 426,568 | 869,079 |
| 5 | 1,931,865 | 922,563 | 1,437,340 | 956,209 | 1,129,726 | 2,165,616 |
| 10 | 3,116,738 | 1,478,059 | 2,320,535 | 1522256 | 1,690.998 | 3,447,597 |
| 15 | 4,106.320 | 1,919,870 | 3,040,325 | 1,994,844 | 2,280,713 | 4,495,262 |
| 20 | 4968,410 | 2,257,005, | 3,668,569 | 2,389,718 | 2,769,595 | 5,412,219 |
| 25 | 5,773,306 | 2,547,623 | 4,253,677 | 2,767,716 | 3,218,105 | 6,268.308 |

Table 2.21: Summary of LCC for Materials

|  | LCC For The Materials Selection (Cumulative Costing) For 100m |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Year | Carbon Steel With Inhibition | 316 SSTL | 6 MO SSTL | 22 Cr Duplex SSTL | 25 Cr Duplex SSTL | Titanium |
| CAPEX Ini | 500,577 | 1,454,165 | 2,763,912 | 1,656;594 | 1,825,647 | 3,671,985 |
| 1 | 1,285,396 | 1,799,103 | 3,306,100 | 2,044,328 | 2,252,215 | 4,541,064 |
| 5 | 2,432,442 | 2400612 | 4,201,225 | 2,612,803 | 2,955,373 | 5,837,601 |
| 10 | 3,417,315 | 2,932,224 | 5,084,475 | 3,178,850 | 3,516,645 | 7,119,583 |
| 15 + Repl. CAP, | 5,220,607 | 3,374,035 | 5 804,237 | 3,681,738 | 4,106,360 | 8,167,247 |
| 20 | 6;082,681 | 3,711,170 | 6,432,481 | 4,046,612 | 4,595,243 | 9,084,204 |
| 25 | 6,887;587 | 4,001,788 | 6,917,589 | 4,424,310 | 5,143,752 | 9,940,293 |

FURTHER REALITY CHECKS ON LCC FOR ELEP – AKAB TL

This LCC comparison is done to cover the following materials

- MS with 3mm CA

- MS with 10mm CA

- Duplex Stainless Steel

And to further ensure that the selected material for the 24”x 28.5 ELEP-AKAB TL, is the best choice.

The cost estimates of materials are based on option 1 phase 1 Epic cost consideration spreadsheet without including the Manifold costs. This plus the construction cost makes up the CAPEX cost.

The OPEX cost estimates are based on Inhibition for one year using CK337 wet gas inhibitor from Nalco, a company in Trans Amadi industrial layout. Other costs elements are Inspection & monitoring, Maintenance, ROW considerations, energy, sparing etc. the following are obtained:

3mm CA cost estimate for CAPEX is $84.82m and OPEX is $4.48m per year whilst,

10mm CA cost estimate is $90.62m and OPEX is $4.544m

DSS material is estimated as $214m CAPEX and OPEX of $4.09m

The NPV was done using dollar inflation rate of 2% and discounted rate of 10% .

The overall outcome is given in the Histogram attached. Hence the material selected is MS with 10mm CA.

### Conclusion on the LCC Histogram

Outright the LCC histogram has not favoured the use of Titanium from the year 1 to year 25.

The MS with inhibition is the most favour materials of choice for the first 10 year life of the 	line. After which period the stainless steels, including the duplex steels, become the materials 	that merit more consideration. A more comprehensive will cover properties, external 	environment, community problems and security impact on the costing..

316 SS TL- Austenitic stainless steel has poor properties in surrounding brackish water 	environment which is saline. It can suffer pitting, crevice corrosion and CSCC (Chloride 	Stress Corrosion Cracking) in this environment. Again, the risk of hydro testing in aerated 	brackish water is severe, It is because of conjoint action of stress and chloride corrosion.  	The area is a flash point for militancy this may attract high cost maintenance that are way 	above MS steel

This has not favoured its use austenitic stainless steel..

The 22Cr duplex and 25Cr have similar cost profile. The 25Cr is slightly more in LCC but 	has slightly more superior properties and higher strength. This is what may have makes it the 	favoured choice for the pipelines .But a big drawback is the initial capital and technical cost 	outlay as result unforeseen maintenance cost impact due to militancy, as for 316L SS.

Consequently, the short lines within fenced manifolds where security is 	highly controlled will favour use of the Super Duplex SS. However, for long 	line that are off – plot only partial acceptance may be considered.. Hence the 	traffic lights coloration (ref table 2.22)   .

Table 2.22: Traffic light Acceptability for CRA’s (Duplex SS)

The use 10 mm higher level of CA and Inhibition

This option is analysed for corrosions and the traffic light indications as given below (see 	table 2.23) The corrosion allowance of 10mm is used. This high corrosion allowance gives 	much more room for better corrosion management system. It most important, in this case, to 	select an inhibitor that has a much better efficiency than 85% which was recommended by 	standards. The selected inhibitor should have an efficiency o f 95% and above with a down 	time of less than 5% per year. The risk to failure to pipelines should be well established, 	practically, in the first 10 years of the service life. The relevant lines that these measures 	should be undertaken are:

- 20”x 20 km SEIB – NUNR BL

- 24”x 45 km NUNR – ELEP TL and

- 24”x 28.5 km ELEP – AKAB TL.

It should be emphasised that these lines should be closely monitored and inspected. Any 	unavailability as result of shutdown shall be accompanied with mothballing. If this is followed 	and managed strictly we may be able to achieve the goal and acceptability of 10 mm CA.

Table 2.23: Corrosion Rates Of MS Subject to 10 mm CA and Inhibition

## Comparison of Selected Line Materials in Traffic Lights

Table 7.2: Traffic Light Assessment

| TRAFFIC LIGHTS COMPARISON – based on Cost & Corrosion |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| s/n | Line Segment | MS | MS with 3 mm ca plus CI Injection | Duplex SS | MS with 10 mm ca plus CI Injection | Selected Material |
| 1 | 24” x 28.5 km ELEP – AKAB T/L |  |  |  |  | MS + 10 mm ca + Corrosion Inhibitor under very strict CMS |
| 2 | 16” x 9 km BBOU – ELEP B/L |  |  |  |  | MS + 10 mm ca + Corrosion Inhibitor |
| 3 | 16” x 10 m BBOU B/L |  |  |  |  | DUPLEX SS |
| 4 | 12” x 10 m ELEP B/L |  |  |  |  | DUPLEX SS |
| 5 | 24” x 45 km NUNR – ELEP B/L |  |  |  |  | MS + 10 mm ca + Corrosion Inhibitor under very strict CMS |
| 6 | 18” x 10 m NUNR B/L |  |  |  |  | DUPLEX SS |
| 7 | 20” x 20 km SEIB - NUNR B/L |  |  |  |  | MS + 10 mm ca + Corrosion Inhibitor under very strict CMS |
| 8 | 20” x 10 m SIEB B/L |  |  |  |  | DUPLEX SS |
| 9 | 24” x 37 km OPUG - BBOU B/L |  |  |  |  | MS + 10 mm ca + Corrosion Inhibitor under very strict CMS |
| 10 | 24” x 10 m OPUG B/L |  |  |  |  | DUPLEX SS |

Note:

- The Duplex SS are amber for the long distance pipelines owing to prohibitively high cost of installation, otherwise would be green on purely technical merit.

The cost, properties and, materials selection justifications were built up from bare MS through, MS with corrosion inhibition at 3mm & 10 mm ca (corrosion allowance) to CRA’s.

The conclusion that was drawn favoured Duplex SS for the Manifold in-plot short distance lines, and API 5L X65 MS for the long distance pipelines.  The API 5L X65 is more acceptable than CRA or Duplex SS, considering cost.

From the corrosivity potential of the transported fluid the MS with 10 mm ca and corrosion inhibitor injection is quite acceptable for the BBOU – ELEP B/L, while the other long distance P/L are only manageably acceptable requiring strict / stringent measures to be taken in terms CMS (Corrosion Management System) in order to achieve the desired service life.

  - Materials Specifications

The materials specifications for the various pipeline segments are follows:

- ELEPA – AKABEL 24” x 28.5 km TRUNKLINE / AKABEL MANIFOLD

- 24” API 5L X65, wt 31.75 mm for the Trunkline; with the use of Corrosion Inhibitor in the operational phase.

- 24” API 5L X65, wt 31.75 mm for the Trunkline; with the use of Corrosion Inhibitor in the operational phase.

- API 5L X65 for the pig barrels, and associated kicker line & balance line.

- Duplex SS – 25Cr UNS Code S32750 for Manifold In-plot Piping

- ELEPA F-MF – ELEP E-MF 12” x 10 m BULKLINE

- Duplex SS – 25Cr UNS Code S32750

- BUBOUWE – ELEPA 16” x 9 km BULKLINE / BUBOUWE MANIFOLD

- 16” API 5L X65, wt 31.75 mm for the Bulkline; with the use of Corrosion Inhibitor in the operational phase.

- 25Cr UNS Code S32750 Stainless Steel for the short piece 16” x 10 m In-plot Manifold Bulkline that offtakes BBOU production from SPDC BBOU Field Manifold Header for delivery into the BBOU – ELEB  Bulkline, either directly or through the Pig Barrel.

- Duplex SS – 25Cr UNS Code S32750 for Manifold In-plot Piping

Table 2.24: Traffic Light Comparism

| TRAFFIC LIGHTS COMPARISON |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| S/N | LINES | MS | MS With Inhibitor + 3mm CA | Duplex SS | MS With Inhibitor + 10mm CA | Conclusion Material Selected |
| 1 | 20” x 20 Km SEIB-NUNR BL |  |  |  |  | MS+ inhibition+ 10 mm CA recommended with very strict CMS |
| 2 | 18” x 10 m NUN R MF |  |  |  |  | DUPLEX SS |
| 3 | 20” x 45 Km NUN R – ELEP TL |  |  |  |  | MS + Inhibition + 10 mm CA recommended with very strict CMS |
| 4 | 24” x 37 Km OPUG – BBOU BL |  |  |  |  | NO INFORMATION ON |
| 5 | 16” x 9 Km BBOU – ELEP |  |  |  |  | MS +Inhibition and 10 mm CA |
| 6 | 12” x 10 m ELEP BL |  |  |  |  | DUPLESS SS |
| 7 | 24” x 28.5 Km ELEP – AKAB |  |  |  |  | MS + Inhibition +10 mm CA recommended with very strict CMS |

From the above table 2.24 these lines are outright acceptable (green light):

18” x 10 m NUNR MF and 12”x 10 ELEP MF lines will be Duplex SS

Whilst the 16”x 9 km BBOU – ELEP will be Carbon Steel with 10 mm Corrosion Allowance 	(CA).Three other lines will require a choice between Duplex SS and MS with inhibitor with 	10 mm CA. These lines are:

- 20”x 20 km SEIB – NUNR BL

- 24” x 45 km NUNR – ELEP TL and

- 24”x 28.5 km ELEP – AKAB TL.

A preference for MS with Inhibitor and 10 mm CA over Duplex SS is it’s lower cost, ease of 	fabrication, maintainability, and possibly welding issues in brackish, swampy and mangrove 	environment with high flash point area.

The API 5L X65 is the chosen grade because of the relative thinner wall thickness most of the 	other PSL1 lines. It also has yield strength and tensile strength advantage with a good carbon 	equivalent.

## Conclusion and Recommendation.

The cost, properties and, materials selection justifications were built up from bare MS 	through, MS with inhibition at 3 mm CA (Corrosion Allowance) to CRA’s with MS having 	10 mm CA with inhibition.

The conclusion that was drawn favoured Duplex SS for the small length lines and for the 	longer length lines, API 5L X65 were acceptable in comparison to CRA or Duplex SS.

Stringent measures shall be taken in terms CMS (Corrosion Management System) in order to 	achieve the goals and targets set for three of the MS lines.

The recommended line materials are given on the basis of table 22 above, the hydraulic and 	thickness calculations were done using API 5L X65 for the Carbon steel. They are as follows:

- 20” x 20 km SEIB – NUNR S-MF BL is 26.19 mm  thickness, Sch.80,API 5L X65 with 10 mm CA, no other optional size for this line. The use inhibition applicable in operations phase).

- 18” x 10 m NUNR MF is 10.31mm, Sch. 40, Duplex SS – 25Cr UNS Code S32750.

- 24” x 45 Km NUNR – ELEP TL is 31.98 mm thickness, Sch.80, API 5L X65 with 10 mm CA, with an option is 22” (26.79 mm thick) The use of inhibition applicable in the operations phase.

- 24” x 37 Km OPUG – BBOU BL …….(For future analysis).

- 16” x 9 Km BBOU – ELEP BL is 22.23 mm, Sch.80, API 5L X65 with 10 mm CA, option is 14” (22.82 mm thick). The use of inhibition also applicable in the operational phase.

- 12” x 10 m ELEP – BL is 9.27 mm thickness, Sch. 40, Duplex SS – 25Cr UNS Code S32750.

- 24” x 28.5 Km ELEP – AKAB TL is 31.98” thickness, Sch. 80, API 5L X65 with 10 mm CA, option is 22” (26.79 mm thick). The use of inhibition is also applicable in the operational phase.

Different operators and national and international standards use different criteria and thresholds to determine whether carbon steel with corrosion allowance or corrosion resistant alloys should be deployed. Differences in corrosion modelling and corrosion inhibitor availability philosophies can lead to carbon steel with minimal corrosion allowance or corrosion resistant alloys being selected for the same field conditions. Even where corrosion resistant alloys are selected factors such as sour service chloride stress cracking exposure to microbes and oxygenated conditions during storage and HISC from cathodic protection can strongly influence the type of CRA selected. Materials selection can also be dominated by mechanical integrity requirements such as strength. This publication examines the various corrosion models and philosophies put forward in different standards and by different operators in the European sector and describes how differing levels of conservatism lead to a wide variation in materials selection. It also examines some of the additional integrity threats that may occur with the selection of corrosion resistance alloys and the methodologies used to mitigate these integrity threats at the design stage.

In general, the Carbon Manganese Steels (MS) are the first economic choice, due to cost, ease of fabrication and construction as pipeline.  It is only when the MS materials are found unacceptable that CRA materials are considered.

In general, the Carbon Manganese Steels (MS) are the first economic choice, due to cost, ease of fabrication and construction as pipeline.  It is only when the MS materials are found unacceptable that CRA materials are considered.

For carbon steel pipelines corrosion allowance and corrosion inhibition form the basis for design life achievement. Where this combination is considered to be insufficient corrosion resistant alloys are used. Different operators and national and international standards use different criteria and thresholds to determine whether carbon steel with corrosion allowance or corrosion resistant alloys should be deployed. Differences in corrosion modelling and corrosion inhibitor availability philosophies can lead to carbon steel with minimal corrosion allowance or corrosion resistant alloys being selected for the same field conditions. Even where corrosion resistant alloys are selected factors such as sour service chloride stress cracking exposure to microbes and oxygenated conditions during storage and HISC from cathodic protection can strongly influence the type of CRA selected. Materials selection can also be dominated by mechanical integrity requirements such as strength. This publication examines the various corrosion models and philosophies put forward in different standards and by different operators in the European sector and describes how differing levels of conservatism lead to a wide variation in materials selection. It also examines some of the additional integrity threats that may occur with the selection of corrosion resistance alloys and the methodologies used to mitigate these integrity threats at the design stage.

In general, the Carbon Manganese Steels (MS) are the first economic choice, due to cost, ease of fabrication and construction as pipeline.  It is only when the MS materials are found unacceptable that CRA materials are considered.

The pragmatic approach therefore rests on the exhaustive investigation of the various available material options for the intended function and application. In this stage, a key factor for the material selection is an up-front activity taking into consideration operational flexibility, cost, availability or sourcing and, finally, the performance of the material for the intended service and application.

This report on materials & corrosion is set to address the following:

- Pipeline Materials Selection & Corrosion Analysis

- Line Pipe Specification

- Coating Systems Selection & Specification, and

- Cathodic Protection Systems

It deals with processes and methodology for the selection of appropriate pipeline materials at 	the optimised Life Cycle Cost (LCC}.

For economic reasons, the Carbon Manganese Steels (MS) are the first choice materials for use whenever possible, because of their cheap cost, 	their ease of fabrication and construction in the pipeline industry. It is only, when the MS 	materials are found unacceptable after their corrosion analysis that other consideration for CRA’s alternatives are given.

The attributes of the project are ............, for which the following materials are considered as options that will suit or fit:

- MS

- MS plus Inhibition

- Various CRA’s (Austenitic SS, Ferritic SS. Martensitic SS, Duplex SS)

- Precipitated Hardened Steel (with AL)

- Pipeline with Internal CRA Lining or Cladding

- Composite Materials

- Non- Ferrous Materials

Other Materials, such as Composite Materials, Monel (Ni enriched Steel}, Non-Ferrous Materials also flavour as alternatives but are not yet tested locally in pipeline construction to present enough justification for their consideration in this selection process.  Also some of them come in small sizes and consequently, may involve the use of many more pipelines (say three or more parallel lines) to achieve the same service as the traditional large diameter size, while some (e.g. the thermoplastic composite) have a wide range of failure modes that render them practically unsuitable.

# Materials Selection And Corrosion Analysis : Coating Materials.

## External Protection of Pipelines

- Coating Material Selection

- Cathodic Protection

## Selection Coverage

- Surface Preparation

- Application of Primer

- Coating Repairs

- Materials

- Storage

- Inspection And Testing

## Background

The proposed BFPC pipelines are to be constructed to supply wet gas from SPDC JV locations widely spread on both, sides of the Nun River at the Brass area, bordered by the Atlantic Ocean at Bayelsa state of Nigeria.

This report forms part of the second section of a three parts report on materials selection for External corrosion Protection which deals with coating materials choice. The concrete addition on the coat is not treated here but separately treated on the main design document because it is considered on the principle of floatation rather than corrosion.

This aspect of the report is a build up from the materials selection report which provide the pipe grade and specifications to be used. The effectiveness of coating is augmented by the CP system for the full pipeline protection.

The coating fabrication on the pipeline is normally carried out in contractor’s yard before it is conveyed to site for installation. It should be noted that handling precaution is necessary to eliminate coating damage.

## Scope

The scope of coating materials will cover both MS and duplex SS as indicated below in table 1.

Table 1 Selected

Table 3.1: Pipelines

| SN | PIPELINES | SELECTED MATERIALS | THICKNESS AND SPECIFICATIONS |
| --- | --- | --- | --- |
| 1 | 20” x 2O km SEIB – NUNR BL | MS + INHIBITION | 26.19 mm thickness, Sch.80,API 5L X65 with 10 mm CA |
| 2 | 18” x 10 m NUNR MF BL | DUPLEX SS | 10.31 mm, Sch. 40, Duplex SS – 25Cr UNS Code S32750 |
| 3 | 24” x 45 km NUNR – ELEP TL | MS + INHIBITION | 31.98 mm thickness, Sch.80, API 5L X65 with 10 mm CA, |
| 4 | 24” x 37 km 0PUG – BBOU BL |  | Not Evaluated |
| 5 | 16” x 9 km BBOU – ELEP BL | MS +INHIBITION | i22.23 mm, Sch.80, API 5L X65 with 10 mm CA |
| 6 | 12” x 10 m ELEP MF BL | DUPLEX SS | 9.27 mm thickness, Sch. 40, Duplex SS – 25Cr UNS Code S32750. |
| 7 | 24” x 28.5 km ELEP – AKAB TL | MS + INHIBITION | 31.98” thickness, Sch. 80, API 5L X65 with 10 mm CA |

### Basis of Design

Risk of external corrosion can be mitigated by applying anti corrosion coating on line pipes to be buried, forming a barrier between the steel and the environment. Application of good coating reduces the C.P current requirement as both (Coating and CP} in conjunction provide external protection

### Design Requirements

The selection of coating material is based on

- The selection of materials must meet the accepted standard from an HSE view point

- The material should provide high level of protection for the entire life of the pipeline

- The adhesion between the steel and coating shall be effective throughout the life of 		the pipeline and the  disbondment of the coating minimal

- Durability of the resistance to chemical, physical and biological degradation

- Maximum and minimum temperatures

- Strength and impact resistance

- Ease of repairs of the damage coat

- Compatibility with concrete weight

- Acceptable unit price

- Ease of application

- low permeability to water and salt

- permeability of oxygen low

### Design Life

30 YEARS

Design Parameter

Pipelines

Table 3.2: Pipelines

|  | MATERIALS | PIPE SIZE Inch | PL LENGTH km | MAX 0PERATING TEMP. 0C | MATERIALS GRADE | COMMENTS |
| --- | --- | --- | --- | --- | --- | --- |
| SEIB - NUNR BL | MS | 20 | 20 | 50-100 | Sch.80,API 5L X65 |  |
| NUNR MF BL | DUPLEX SS | 18 | 0.01 | 50-100 | Sch. 40, – S32750 |  |
| NUNR - ELEP TL | MS | 24 | 45 | 50-100 | Sch.80, API 5L X65 , |  |
| OPUG - BBOU BL |  |  |  |  |  | Not Evaluated |
| BBOU – ELEP BL | MS | 16 | 9 | 50-100 | Sch.80, API 5L X65 |  |
| ELEP MF BL | DUPLEX SS | 12 | 0.01 | 50-100 | Sch. 40, S32750. |  |
| ELEP – AKAB TL | MS | 24 | 28.5 | 50-100 | Sch. 80, API 5L X65 |  |

Coating Materials Information

X 	- Unaccepted performance

- - Accepted performance

Table 3.3: Coating Materials Information

| S/N | MATERIALS | OPERATING TEMPERATURES | DISBONDMENT (Life time) | EASE OF APPLICATION | HSE | COMMENTS |
| --- | --- | --- | --- | --- | --- | --- |
| 1 | Asphalt | X | X |  | X |  |
| 2 | CTE | X | X |  | X |  |
| 3 | Polyethylene Wraps | X | XX | - X2 |  |  |
| 4 | Epoxy/PE/PP | - X2 | - X2 |  |  |  |
| 5 | FBE | - X2 | - X2 |  |  |  |

## Selected Coat

### Coating Material

On the basis of the analysis above, the selection of suitable coating material for this project should be cost effective and also depend on the operating temperature of between 50oc and 100 oc. Hence the higher temperature will favour the use of 3 layer polypropylene in accordance with ISO 21500 -x12011. Besides meeting the technical requirements it is available in the country.

### Coating Materials Composition

1st layer epoxy

2nd Layer Adhesive

3rdLayer PP top coat

It should be noted that the qualifications of coatings will comprise:

- Batch Certification

- Coating System Qualifications General

- Procedure Qualification

- Inspection and Test Plan

- Visual Appearance

- Coating Thickness see below table on minimum recommended thickness for 		coating

Table 3.4: Coating thickness

| Design Parameter | Unit |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Pipe size | Inch | 10 | 12 | 16 | 20 | 24 |
| Coating Thickness | mm | 2.0 | 2.9 | 2.9 | 2.9 | 2.9 |

Other relevant items are:

- Holiday detection

- Impact Resistance

- Indentation Resistance

- Elongation

- Thermal Ageing

- Hot Water Resistance

- C.P disbanding

- Degree of Cure of FBE Primer.

### Coating Materials for Bend

Bends are factory coated using 3LPP as it relates to the pipeline

### Coating Materials for Field Joint

The recommended field joint is the shrinkage sleeves type. However, the contractor may come with another system. The procedure application must be qualified before use and also proved by BFPC before use.

### Coating Materials Repair Works

The Contractor should provide the procedure of the repair methods to be used and also the material for prequalification exercise before approval for use.

### Surface Preparation

The line pipe will be prepared to SA2 .5 ISO 8501.4 standards by blasting. It should be noted that for duplex SS the grit chips should be such that it does not leave any free FE on the surface. A solution may be the use of Al grit chips.

# Cathodic Protection Design

## Introduction

Cathodic Protection is one part of a co-joint requirement along with coating application to provide the full external protection of pipelines networks or systems.

This is a preliminary CP design works for the  BFPC pipelines that will transport wet Gas from various SPDC locations in the Brass/Nun River areas of Bayelsa state. At this stage only desk top analysis on the protection will be done. And the CP will be temporary until all relevant studies and construction decisions of the pipelines have been made. Chemical, Resistivity Surveys along the ROW, selection of Ground bed locations and types will be based on general information and assumptions as the true field data and surveys will be conducted before the final design is implemented.

CP design is not done in isolation of the coating applied on the pipelines. The function of the coatings is to reduce the surface area to be protected by CP. The Coating, normally, is intended to form a continuous film of an electrically insulating barrier material over the pipeline “surface to be protected from direct contact with the surrounding electrolytes and to interpose such a high electrical resistance that electro-chemical “corrosion” reactions cannot ready occur. In reality, all coatings, regardless of overall quality, contain holes or holidays that are formed during application or developed during transportation or soil stress and other disbondment from pipe surface including the degradation of the coating during service.

For this CP design the full information on the selected coating material, performance, characteristics, and limitations will be required and considered.

A good understanding of the basic theory of CP is necessary for this work.

Cathodic Protection is a technique to reduce the corrosion rate of a metal surface by making it a cathode of an electrical cell.

The two main ways CP are practically applied are through:

- Impressed Current on the Pipeline

- Sacrificial Anode in Galvanic System with the Pipeline being the Cathode

The following consideration will be reviewed in this design work.

- Criteria for CP

- Selection of Type, Size, and Spacing of the System

- The Needed Information that will Influence the Choice of the System (sacrificial or 			Impressed Current)

- Total Current Requirements

- Special Conditions requirements

A more comprehensive requirement will be the following;

- Corrosivity of the Environment and Soil Structure and resistivity Survey

- Coating Protection  level/ Anticipated Deterioration level

- The metal / Alloy used  For the Pipeline

- The Size of the Pipeline and its Ability to Conduct  CP current

- The Presence of Foreign Structures , Crossing etc

- The Presence of Stray Current Protection

- Chemical Reaction Survey and

- Attenuation Survey.

## Project Description

It is envisaged that the Brass fertilizer plant (owned by BFPC) will come into production in 2018 and later on to full capacity production of:

- 4,400 MTPD of Ammonia

- 3000 MTPD of Methanol

- 7,700 MTPD of Urea

- Nitrite Fertilizer Product

The plant and its infrastructural facilities will be located bordering at the Atlantic Ocean in Brass Island, Bayelsa state of Nigerian. It will have facilities widely spread on both sides of the River Niger (West and East).The plant feedstock will be Gas from SPDC JV wells at BBD3 Analogue , Nun River,  Seibou  and Tollogbene fields.

The wet gas will be transported from SPDC’s manifolds through pipelines at these identified   locations through the bulklines via the main pipelines to the Gas  plant  via the main Gas Transmission lines to the gas plant where it is treated , dried and pass to the Fertilizer plant as shown  in the Fig 4.1

Fig 4.1: Propose Pipelines’ Routes

## Project Objective

The objective of this project is to carry out Cathodic Protection preliminary design of the BFPC wet gas bulklines and pipelines from SPDC to the BFPC Fertilizer Plant

## Purpose

The document details the preliminary cathodic protection for the external protection of corrosion of the BFPC bulklines and pipelines.

## Scope

The lines on Table 4.1 below are the scope of work

Table 4.1: Pipelines Information

| SN | PIPELINES | SELECTED MATERIALS | COMMENTS |
| --- | --- | --- | --- |
| 1 | 20” x 2O km SEIB – NUNR BL | MS | 10mm CA |
| 2 | 18” x 10 m NUNR MF BL | DUPLEX SS | No CA |
| 3 | 24” x 45 km NUNR – ELEP TL | MS | 10mm CA |
| 4 | 24” x 37 km 0PUG – BBOU BL |  | Not Evaluated |
| 5 | 16” x 9 km BBOU – ELEP BL | MS | 10mm CA |
| 6 | 12” x 10 m ELEP MF BL | DUPLEX SS | No CA |
| 7 | 24” x 28.5 km ELEP – AKAB TL | MS | 10mmCA |

# Cathodic Protection Design (Definition)

## General Definition

The Contractor is the party which conduct all or part of the design. The Principal may undertake all or part of the duties.

The Manufacturer/supplier is the party which manufacture or supplies equipment and services to perform the duties specified by the contractor.

The Principal/Company is the party which contracts the project and completely pay for it’ design and construction. The Principal will generally specify the technical requirement. The Principal may also include an agent or consultant authorized to act for and on behalf of the Principal.

The word shall indicates requirement.

The word should indicates recommendation.

## Abbreviation

API 		American Petroleum Institute

ASTM	 	American Society for Testing and Material

CP 		Cathodic Protection

DEP 		Shell Design Practice

GB 		Ground Bed

ICCP 		Impressed Current Cathodic Protection

IJ 		Isolation Joint

ISO 		International Standard Organisation

NACE 		National Association of Corrosion Engineer

PMT 		Project Management Team

SPDC 		Shell Petroleum Development Company

# Cathodic Protection Design ( Technical Information/Data)

## Design Parameters

- Pipeline Information

Table 6.1: Pipeline Data

| S/N | PIPELINE | LINE DIA. | LINE LENGHT | DESIGN PRESS | AMB. TEMP.0C | TEMP. 0 C | THICKNESS + CA (mm) | SELECTED COATING TYPE |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | SEIB – NUNR BL MS | 20” | 20 km | 3,000 | 35 | 50-100 | 16.19 +10= 26.19 | 3LPP |
| 2 | NUNR MF BL (DUP. S/STEEL) | 18” | 10 m | 3,000 | 35 | 50-100 | 10.31mm | 3LPP |
| 3 | NUNR – ELEP TL MS. | 24” | 45 km | 3,000 | 35 | 50-100 | 21.98 +10 =31.98 or 16 .79 +10 =26.79 | 3LPP |
| 4 | 0PUG – BBOU BL | 24” | 37 km |  |  |  |  |  |
| 5 | BBOU – ELEP BL MS | 16” | 9 km | 3.000 | 35 | 50-100 | xxxxx | 3LPP |
| 6 | ELEP MF BL (DUP. S/STEEL) | 12” | 10 m | 3.000 | 35 | 50-100 | 9.27 | 3LPP |
| 7 | ELEP – AKAB TL MS. | 24” | 28.5 km | 3,000 | 35 | 50-100 | 21.98 +10 =31.98 or 16 .79 +10 =26.79 | 3LPP |

- Design Life - 30 years

- Expected Coating Disbondment in 30 years

Polypropylene coating breakdown value is 5 to 7% at 25 years the mean for the period is 2% from DnV RP B-401 standards with expected 0.75 to 1.5% per year thereafter.

- Reference Electrode Potential Ranges for Steel Protection (starting with the    	criteria for Protection to the beginning of overprotection)

Table 6.2: Criteria for Protection Vs Reference Electrode

| Environment | Reference Electrodes |  |  |
| --- | --- | --- | --- |
|  | Cu/CuSO4 (mV) | Ag/AgCl (mV) | Zinc (mV0 |
| Free Seawater Environment | -850 to -1000 | -800 to -1050 | +250 to +0 |
| Anaerobic Environment | -950 to -1100 | -900 to -1050 | +150 to +0 |
| High strength Steel UTS>700N/mm2 | -850 to -1000 | -800 to -900 | +250 to +100 |
| Land/ Swamp Environment | -850 to-1150 | Not used for land | Not used for land |

- Environmental Condition

Environment – mangrove swamp and brackish water of Resistivity 1000 -5000 ohm – cm (assumed)

Mean Minimum temperature 		230C

Minimum temp				180C

Mean Maximum Temp 		310C

Ground Temp 				25 -27.50C

Black Body Temp 			800C

Design Ambient Temp-Max		430C

Design Ambient Temp Min		180C

Humidity				100%

Site Altitude 				Sea level

Lightning				Frequent in the Raining Season

Average Annual Rainfall		3800 in

Mean Maximum hourly Rainfall	100 in

Design Wind Speed			35.6 m/s

Corrosion Environment 		Brackish Swamp/Waters (50 -100 ohm-cm)

- Environment / Resistivities  Vs Corrosion Current per M2

The assumption made for the current density  for bare steel metal areas for this desk top calculations is 22mA/m2 at the assumed resistivities of 1000 – 5000 ohm-cm of the swampy, brackish water  environment closed to the  brass sea

- Coat efficiency, Mean Coating Breakdown and coating Breakdown at end 	of 	life time

Table 6.3:  Coating Breakdown for Design Purpose

| SN | Type of Coat | Efficiency on Yard Fab. (%) (Initial) | Breakdown after 30 years (%) | Mean Coating Breakdown (%) |
| --- | --- | --- | --- | --- |
| 1 | Epoxy 3LPE or 3LPP | 99.9 - 99.5 | 7- 10 | 2 |
| 2 | Reinforced Coal Tar or Asphalt Enamel | 98 | 10 -20 | 5-10 |
| 3 | Polyken Wraps | 95 | 50 (15yrs) | 15 -30 |
| 4 |  |  |  |  |

# Cathodic Protection Design ( Basis For Design)

## Basis For Design

- The external protection of the pipelines along with the coat

- Preliminary cathodic protect shall be provided during the construction period of the 	pipelines

- The final CP system shall be implemented following the required field surveys for 	chemicals, soil resistivity for the ROW and brackish water crossing.

- Resistivity survey shall also be done for the selection of Ground bed locations.

- Design life is 30 years

- Design life for temporary CP system during construction is 3 years

- Choice of CP powering will depend on availability of power in the manifold

- All pipelines will be buried or submerged

- Soil resistivity for the temporary CP is assumed to be 1,500 Ohm-cm

- Resistivity of carbonaceous backfill is assumed to be 0.5 ohm-m

- The pipelines will be coated with 3LPP and in good quality

- The temporary CP current estimates will be made on deck-up choice

Reference electrode is saturated Cu/CuSO4  for soil and Ag/AgCl for seawater

## Choice Of CP System

The CP system shall be used for the external protection of pipelines against corrosion. In this specific case there is no margin for error in term of corrosion based on the limitation on the selected material corrosion tolerance (No room for external corrosion). A three year temporary CP shall be incorporated during the construction. This design will be concluded in this document.

The factors that will influence the choice of the CP system will include:

- Availability of Electrical Power System

- Pipelines will be buried or Submerged

- Pipeline / Environmental Considerations

- Polarisation Limits

The choice that are available are:

- The Type of CP System – ICCP, SA, or Hybrid

- Electrical Power System – National Electricity Board, Company Supplied Electricity, 		Remotely Powered Generators (Solar Powered, Thermo- Electrical, Wind Powered, 			Batteries ,Turbo Generator, Fuel Cells etc)

- Ground bed locations Selection,

- Anodes Choice

## Current  Requirement

### Current Density

Current density is the corrosion current released per square area of a material under electro-chemical influence. It follows that the total corrosion current will be the current density multiply entire surface area. This area can be significantly reduced by application of coat. Allowing only for exposed areas that the coat has not relevantly affected to corrosion. Cathodic Protection is the application of the reverse cathode current to reduce or arrest the electro chemical corrosion reactions.

Hence, the determination of the total current is used to evaluate the design properties using the given equation for pipeline:

Ic     = JF2rL ------------------------eq. 1

Where J – is the design electrical current density

F – Coating Breakdown Factor

2rL – Surface Area

### Current Demand Calculations

From above equation 1, the design electric current demand for each steel bulklines & pipelines taking into cognisance the coating breakdown factor is shown in table  below . They are evaluated to be 0.2mA/m2 in accordance with ISO 155------- (DEP 30.10.73.31 Gen.) Feb. 2013. Again, it should be noted that three relevant breakdowns used for design. These included,

- The Initial Breakdown

- The Mean Breakdown

- The Final Breakdown

The current need for group of pipelines which will be hooked at the manifold from same field or area is given by equation 2

Icmf = 1nIc   ---------------------------eq. 2

### Sizing of Transformer – Rectifiers

### Transformer –Rectifier for group of pipelines (n)

The transformer rectifier is used to convert AC to DC which is required for CP.

The size is based on the current demand and addition 50% and then using ohm’s law, the voltage is obtained reference equations 3 & 4

ICTR  =1nIc  + 50%1nIc --------------------eq.3

The Ohm’ s law’ which is given as

V=IR ----------------------------------------eq.4

Where, V is Voltage, and R is Resistance. The resistance is based on the entire CP system resistance

Table 7.1: Surface Areas and Current Demand for the Pipelines CP.

| SURFACE AREA & CURRENT DEMAND CALCULATIONS |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| SN | P/LINE | LINE DIA. | LENGTH (m) | BARE SURFACE AREA (m2) | MEAN EXPOSED SURFACE AREA (2%) | EXPOSED SURFACE AREA (LIFESPAN) (5%) | CORR. CURR. DENSITY (mA/m2 Bare Steel) | FINAL CURR. DEMAND (Amp) | MEAN CURR (Amp) | INITIAL CURR. (Amp) |
| 1 | NUN R – ELEP | 24” (0.61m) | 45,000 | 86,180.16 | 1,723.60 | 4,309.01 | 22 | 94.80 | 37.92 | 17.24 |
| 2 | ELEP – AKAB | 24” (0.61m) | 28,500 | 59,368.56 | 1,187.37 | 2,968.43 | 22 | 65.31 | 26.12 | 11.87 |
| 3 | OPUG –BBOU | 24” (0.61m) | 37,000 | 71,816.80 | 1,436.34 | 3,590.84 | 22 | 78.99 | 31.60 | 14.36 |
| 4 | SEIB – NUNR | 20” (0.51m) | 20,000 | 31,918.58 | 638.37 | 3,191.86 | 22 | 70.22 | 14.04 | 6.4 |
| 5 | BBOU – ELEP | 16 ” (0.41m) | 9,000 | 11,498.68 | 229.97 | 1,595.93 | 22 | 35.11 | 5.06 | 2.3 |
| 6 | NUN R - MF | 18” (0.46m) | 10 | 47.87 | 0.9574 | 2.395 | 22 | 0.052 | 0.02 | 0.01 |
| 7 | ELEP MF | 12” (0.30m) | 10 | 39.89 | 0.7978 | 1.995 | 22 | 0.044 | 0.02 | 0.01 |

# Design  For Temporary Protection System

The protection of the pipelines during the construction milestones is absolutely necessary. This will give progressive protection during the period of construction until the construction is completed and until the ICCP system is put in place.

Sacrificial Anode system is used for the temporary protection system.

The Anode choice is - High Output Magnesium (Mg) Anode

The Anode design life - 3 years

## Design Data

### Current Demand

The current demand is given in table 7.1above on surface area and CP current demand calculations. The initial current demand will be utilised for further works as it will represent early stage of the current demand.

## Characteristic of High Output Mg Anode (DEP 30.10.73.10 Gen)

Table 8.1:  Characteristics of the High Output Magnesium Anode

| SN | Description | Value |
| --- | --- | --- |
| 1 | Design Life (years) | 3 |
| 2 | Anode Utilisation Factor (µ) | 0.9 |
| 3 | Anode Efficiency (€) | 0.5 |
| 4 | Anode Protection Potential vs Cu/CuSO4 electrode (V) | 0.950 |
| 5 | Anode Closed Circuit Potential Seawater (V) | 1.65 |
| 6 | Anode Consumption Ca (Kg/Ay) | 8 |
| 7 | Length of design (m) | 0.431 |
| 8 | Diameter of Design Mg Anode D in m | 0.1 |
| 9 | Radius of Design r (meters) | 0.05 |
| 10 | Maximum Weight of Design Anode (KG) | 6.1 |
| 11 | Anode theoretical electrochemical capacity (Ah/kg) | 2200 |

Table 8.2:   High Output Magnesium Anode Composition

| MAGNESIUM ANODE COMPOSITION |  |  |
| --- | --- | --- |
| S/N | ELEMENTS | PERCENTAGE |
| 1 | Cu | 0.02% Maximium |
| 2 | Si | 0.10% Maximum |
| 3 | Fe | 0.003% Maximum |
| 4 | Ni | 0.002% Maximum |
| 5 | Mn | 1.0% Maximum |
| 6 | Mg | Remainder |

Equation for calculation is as follows:

Current (Amp) = Protected Area (m2) x Current density (mA/m2) / 1000  ……………eqn  5

Weight (Kg) = Current (Amp) x design life (years) x 8766 / Capacity of material (Amph/Kg) x  0.9 ….................eqn 6

Number of Anodes = Weight Required / Individual Net Anode Weight   ………eqn  7

Number of Anodes = Current Required / Individual Current Output   ……………eqn 8

MA = It x td x Ca/µ  …….eqn 9

Where   Ma = Mass of Anode (kg)

Td = Design life (years)

It = Current demand (Ampere)

Ca = Consumption rate of Anode (kg/A – y)

µ = Utilization factor

| ANODE REQUIREMENTS |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| S/N | LINE | DIAMETER |  | CURRENT DEMAND (It) | MASS OF ANODE FOR PROTECTION CURRENT (ma) | MASS OF SINGLE ANODE (ma) | No. OF ANODES |
|  |  | Inch | mm |  |  |  |  |
| 1 | SEIB- NUNR 20” x 20 Km | 20 | 508 | 6.40 | 170.66 | 6.1 | 14 |
| 2 | NUN R MF 18”x10 m | 18 | 457.2 | 0.01 | 0.26 | 6.1 | 1 |
| 3 | NUN R – ELEP 24”x 45 Km | 24 | 609.6 | 17.24 | 459.7 | 6.1 | 38 |
| 4 | OPUG – BBOU 24”x 37 Km | 24 | 609.6 | 14.36 | 382.9 | 6.1 | 31 |
| 5 | BBOU – ELEP 16”x 9 Km | 16 | 406.4 | 2.30 | 61.33 | 6.1 | 5 |
| 6 | ELEP MF 12”x 10 m | 12 | 304.8 | 0.01 | 0.26 | 6.1 | 1 |
| 7 | ELEP - AKAB 24”x 28.5 Km | 24 | 609.6 | 11.87 | 284.88 | 6.1 | 26 |

Table 8.3: Number of Magnesium Anode Required Per Line for Temporary Protection (on the right column)

NB :  ELEP – AKAB is a coastal line and requires permanent Anodes likely Zinc bracelet  type that will be factory manufactured and fitted.

The two Short MF lines may be protected with SA for Life Time.

## Temporal Anode Spacing

Each line has it’s Anode requirement or number. These numbers are spaced equally along the line with 1st and last Anode being half the spacing of the end points of the line.

The Anode installation shall be in accordance to this document, specifications thereof and the manufacturer instructions. The Anodes shall be surrounded by a Bentonite / Gypsium backfill to avoid it’s polarisation.

NOTE : At completion before the commissioning of the project, the temporal sacrificial CP Mg Anode system shall be decommissioned and the impressed current CP system commissioned as may be the case.

## Electrical Separation

The pipeline shall be electrically isolated from the piping as well as electrical and instrumentation earthing system. Isolation joints shall be installed,

- At both extremities of the buried section.

- At branch lines if they are not part of the main line.

The scope boundaries must be provided with electrical isolation to prevent interaction / electrical interference in the system. This will limit the possibility of electrical current licking in to adjacent installation.

# Permanent CD Design

## Design Basis

The Design shall be based on:

- Choice of CP system for each line (SA, ICCP, Hybrid System)

- All buried lines shall be impressed current system.

- All the coastal and submerged line shall be permanent SA (Bracelet anode type)

- Very short lines but buried shall be permanent SA Mg.

## Detail Design Survey

The following detail survey should be under taken:

- ROW --  Chemical Analysis Survey

- ROW --  Resistivity Survey.

- Ground bed Resistivity Survey for selection of ground bed locations either deep well 	or otherwise.

## Choice of Electrical Generation System.

Other requirements are :

- Cable marker

- Cable Connections to the lines.

- Removal of Coatings and Cleaning.

- Thermal Weld Connection (CAD Welding).

- Coating Repairs.

- Holiday Test.

- Junction Boxes

- Test Posts / Stations

# APPENDIX

- Wellhead Gas Gathering Schematics & Flow Parameters to Gas Processing Plant

- Proposed Pipeline Route

- Pipeline Materials Selection

## Appendix-1:	Well Gas Gathering Schematics

| Appendix 1 | EA/SPDC-BFPCUGS-GEP/BDM-PFS-001-A01 Wellhead Gas Gathering Schematics & Flow Parameters to Gas Processing Plant |  |
| --- | --- | --- |

## Appendix-2:	Proposed Pipeline Route

| Appendix 2 | EA/SPDC-BFPCUGS-GEP/SUY-DWG-001-A01 Proposed Pipeline Route |  |
| --- | --- | --- |

## Appendix-3:	Pipeline Materials Selection

| Appendix 3 | EA/SPDC-BFPC UGS-GEP/BDM-SCH-001-A01 Pipeline Materials Selection & Manifold Platforms Footprint Indication. BBOU & ELEP |  |
| --- | --- | --- |