GAS PROCESSING & METHANOL COMPLEX PROJECT NO. ex-11998A

| Project | FEED for Gas Gathering Pipelines, Gas Processing Plant & Methanol Plant |
| --- | --- |
| Client | Brass Fertilizer and Petrochemical Company Limited |
| Originating Company | Epic Atlantic Limited |
| Document Title | Produced Water Handling, Processing, & Disposal Plan |
| Document Number | EA-BFPCL-GPMC-GGPL-FEED-HSE-PLN-001 |
| Document Revision | R02 |
| Document Status | Issued as a Requirement |
| Originator | Obima Joe Oragwu |
| Endorsement | Aloy Onyenucheya Duru, COREN (R.7580) |
| Security Classification | Restricted |
| Issue Date | 25-Mar-26 |

Produced Water Handling, Processing, & Disposal Plan

| R01 | 07 Mar 2026 | Issued for | Joe Oragwu | Joe Oragwu / Aloy Duru | Aloy Duru COREN (R.7580) | Cyril Akika |
| --- | --- | --- | --- | --- | --- | --- |
| Rev # | Issue Date | Status Description | Originator | Reviewer | Endorsement (Epic) | Approval (BFPCL) |

Revision History

| Rev No. | Date | Description of Revision |
| --- | --- | --- |
| R01 | 07 Mar 2026 | For Review by RAEC, and Subsequent Discussion with NUPRC, in respect of BBOU & ELEP FDP |
| R02 | 25 Mar 2026 | Clarification that clearly brings out BFPCL options plan without ambiguity |
| A01 |  |  |
| A01 |  |  |

Revision Philosophy:

- All documents for review shall be issued at R01, with subsequent R02, R03, etc as required.

- All documents approved for issue or approved for design shall be issued at A01 with subsequent A02, A03, etc. as required. (Management of Change is required for A02, A03, etc.)

for the subsequent project phases, post FEED, it is expected that:

- All Detailed Design documents for review shall be issued at D01, with subsequent D02, D03, etc. as required.

- All documents approved for construction shall be issued at C01 with subsequent C02, C03, etc. as required. (Management of Change is required for C02, C03, etc.)

- All approved “As-Built” documents shall be issued at Z01, with subsequent Z02, Z03, etc as required. (Use versions Z01.1, Z01.2, Z01.3, etc to review the “As-Built” document to Z02).

Restricted

This document is made available subject to the condition that the recipient will neither use nor disclose the contents except as agreed in writing with the copyright owner.  Copyright is vested in Brass Fertilizer and Petrochemical Company Limited (BFPCL) © All rights reserved.

Neither the whole nor any part of this document may be reproduced or distributed in any form or by any means (electronic, mechanical, reprographic, recording, or otherwise) without the prior written consent of the copyright owner.

SUMMARY PAGE

| Client: | Brass Fertilizer and Petrochemical Company Limited |  |
| --- | --- | --- |
| Project: | FEED for Gas Gathering Pipelines, Gas Processing Plant & Methanol Plant | Project No. ex – 11998A |
| Facility: | Gas Gathering Pipelines, Gas Processing Plant & Methanol Plant | Doc No. EA-BFPCL-GPMC-GGPL-FEED-HSE-PLN-001 |

This document demonstrates BFPCL Management plan and commitment to follow the regulatory & policy framework towards produced water handling, treatment and disposal.  To this effect BFPCL has developed a Plan, including Framework, Road Map and Decision Flow Charts, Benchmarks and Best practices, for systems, procedures, and controls for safe handling, treatment, monitoring, and disposal of produced water generated in the project.

While the immediate operational focus is on the BBOU and ELEP production streams, the Plan, without prejudice, also extends to Okpokunou production, recognizing that the inlet streams and produced water streams may ultimately commingle within the processing facilities.

BFPCL plan is to ensure Regulatory Compliance regulators, and in alignment with:

- Petroleum Industry Act (2021)

- EGASPIN (administered by NUPRC)

- EIA Act

- National Oil Spill Detection and Response Agency (NOSDRA Act)

- National Environmental Standards and Regulations Enforcement Agency (NESREA Act)

The Water Treatment envisaged will start with 3-phase Separator, followed by a cascade of downstream treatment units designed to progressively remove oil, suspended solids, and fine particulates.

Five options will be evaluated as part of the concept selection & detail design, viz:

- Treatment to Specifications, and Offshore Disposal through GRP composite materials pipeline,

- Treatment to specifications, and Storage in Tanks, from where it will be disposed offshore by shuttle tankers

- Treatment to near potable water specification for industrial utility use

- Transportation of the Untreated Produced Water to nearby Brass Terminal with Produced Water Handling Facilities for treatment and further administration on behalf of BFPCL

- Transfer of the Untreated Produced Water to 3rd Party accredited company, for the Water Treatment and Handling on behalf of BFPCL

| REferences |  |
| --- | --- |
| Document No. | Description |
| EA-BFPCL-GMMC-GGPL-FEED-GEN-RPT-002 | Basis of Design |
|  | NUPRC / NMDPRA Guidelines |
|  | EGASPIN |

Table of Contents

Abbreviations	5

1.	Executive Summary	8

2.	Introduction	9

2.1.	Background Information	9

2.2.	Schematics of FEED Scope	10

3.	Regulatory Standards and Requirements	11

3.1.	Regulatory Instruments	11

3.2.	Regulatory Standards / Limits	11

3.2.1.	Disposal Methods in NUPRC Guidelines	11

3.2.2.	Effluent Discharge Limits (2026)	12

3.3.	Regulatory Monitoring and Reporting Requirements	12

3.3.1.	Statutory Operational Control	12

3.3.2.	Monitoring & Reporting Obligations	12

3.3.3.	Technical & Operational Verification	13

3.3.4.	Regulatory Compliance Roadmap	13

3.3.5.	Produced Water Characterization	13

3.3.6.	Treatment & Processing Technologies	13

3.3.7.	Handling and Processing Standards	14

3.3.8.	Technical Handling for Gas & Methanol Wastewater	14

4.	BFPCL Framework & Implementation	15

4.1.	BFPCL Framework	15

4.2.	BFPCL Roadmap	15

5.	Produced Water Characterisation & Risk Assessment	21

5.1.	Background Information	21

5.1.1.	Source & Composition	21

5.1.2.	Environmental & Health Risk Assessment	21

5.1.3.	Data & Monitoring Plan	21

5.2.	Treatment & Management Strategies	21

5.2.1.	Source Reduction & Production Practices	21

5.2.2.	On-Site Treatment Technologies	21

5.2.3.	Off-Site / Advanced Options	22

5.3.	Water Reuse & Resource Optimization	22

5.3.1.	Internal Reuse	22

5.3.2.	Beneficial Reuse Opportunities	22

5.4.	Monitoring, Reporting & Compliance Assurance	22

5.4.1.	Environmental Monitoring	22

5.4.2.	Reporting	22

5.4.3.	Corrective Actions	22

5.5.	Stakeholder Engagement & Community Safeguards	22

5.5.1.	Community Consultation	22

5.5.2.	Grievance Mechanisms	23

5.5.3.	Capacity Building	23

5.6.	Research, Innovation & Continuous Improvement	23

5.6.1.	Technology Trials	23

5.6.2.	Standards Review	23

5.6.3.	Collaborative Initiatives	23

5.7.	Summary of Best Practices	23

6.	Produced Water Management Policy	24

6.1.	Scope	24

6.2.	Discharge Standards	24

6.3.	Produced Water Management Hierarchy	24

6.3.1.	Prevention & Minimization	24

6.3.2.	Reuse & Reinjection	24

6.3.3.	Treatment & Controlled Discharge	24

6.3.4.	Disposal	24

6.4.	Technical Standards	24

6.4.1.	Characterization	24

6.4.2.	Treatment Philosophy	25

6.5.	Monitoring & Reporting	25

6.5.1.	Monitoring Requirements	25

6.5.2.	Reporting	25

6.5.3.	Risk Management & Incident Response	25

6.6.	Community & Environmental Safeguards	25

6.7.	Governance & Accountability	26

6.8.	Continuous Improvement	26

7.	PRODUCED WATER VOLUME MANAGEMENT	27

7.1.	Produced Water Volumes	27

7.2.	Treatment Philosophy & Process Design	27

7.2.1.	Treatment Objectives	27

7.2.2.	Process Description	27

7.2.3.	Chemical Treatment	27

7.2.4.	Monitoring Systems	28

7.3.	Disposal Strategy	28

7.3.1.	Primary Disposal: Reinjection	28

7.3.2.	Contingency Discharge	28

7.4.	Compliance With Discharge Limits	28

7.5.	Environmental Risk Assessment	28

7.6.	Monitoring & Reporting	29

7.6.1.	Sampling Frequency	29

7.6.2.	Reporting	29

7.7.	Sludge & Waste Management	29

7.8.	Integrity Management	29

7.9.	Emergency Response	29

7.10.	Training & Governance	29

7.11.	Decommissioning	29

8.	CONCLUSION	31

## Abbreviations

| Abbreviation | Meaning |
| --- | --- |
| AKAB | Akabel |
| API | American Petroleum Institute |
| BBOU | Bubouwe Bou Field |
| BFPCL | Brass Fertilizer and Petrochemical Company Limited |
| BOD | Biochemical Oxygen Demand |
| BWPD | Barrels of Water Per Day |
| CAP | Corrective Action Plan |
| Cd | Cadmium |
| COD | Chemical Oxygen Demand |
| COREN | Council for the Regulation of Engineering in Nigeria |
| CPF | Central Processing Facility |
| Cr | Chromium |
| DW-01 | Disposal Well 01 |
| EA | Epic Atlantic (Document Prefix) |
| EGASPIN | Environmental Guidelines and Standards for the Petroleum Industry in Nigeria |
| EIA | Environmental Impact Assessment |
| ELEP | Elepa Field |
| EOR | Enhanced Oil Recovery |
| EPF | Early Production Facility |
| ESG | Environmental, Social, and Governance |
| FDP | Field Development Plan |
| Fe | Iron |
| FEED | Front-End Engineering Design |
| GGPL | Gas Gathering Pipelines |
| GOSP | Gas-Oil Separation Plant |
| GPMC | Gas Processing & Methanol Complex |
| GPP | Gas Processing Plant |
| GSPA | Gas Supply and Purchase Agreement |
| HAZOP | Hazard and Operability Study |
| HSE | Health, Safety, and Environment |
| IGF | Induced Gas Flotation |
| ISO | International Organization for Standardization |
| LOF | Life-of-Field |
| MBWPD | Thousand Barrels of Water Per Day |
| MMscfd | Million Standard Cubic Feet per Day |
| MSDS | Material Safety Data Sheet |
| MTPD | Metric Tonnes Per Day |
| NESREA | National Environmental Standards and Regulations Enforcement Agency |
| NGL | Natural Gas Liquids |
| Ni | Nickel |
| NOSDRA | National Oil Spill Detection and Response Agency |
| NUPRC | Nigerian Upstream Petroleum Regulatory Commission |
| OIW | Oil-in-Water |
| OKNU | Okpokunou Field |
| OML | Oil Mining Lease |
| Pb | Lead |
| PIA | Petroleum Industry Act |
| RAEC | Regulatory Affairs and Environmental Compliance |
| RCA | Root Cause Analysis |
| SBM | Single Buoy Mooring |
| SCADA | Supervisory Control and Data Acquisition |
| TDS | Total Dissolved Solids |
| TSS | Total Suspended Solids |
| TVD | True Vertical Depth |
| ZLD | Zero Liquid Discharge |

# Executive Summary

This document outlines BFPCL’s management plan and commitment to comply with the applicable regulatory and policy framework governing produced water handling, treatment, and disposal.

The strategy adopts a "Triple-Shield" resilient framework to manage liquids from the OKNU Hub through the 125km trunkline to the AKAB hub. Six water handling, treatment and disposal options are considered.

The preferred option is  the Industrial Re-use of the produced water by the Methanol Plant. Produce water from the wet gas, GPP Processed Water and Methanol Reaction Water are all treat by  polishing and stripping and use MP Steam Reformer Feed and also as Utility/Firewater/Toilets water, A Water Treatment Unit at AKAB equipped with Double Pass Reverse Oxygen (RO) unit processes the water ”Boiler Feedwater Quality.” A methanol Stripper to remove methanol from the reaction water before it gets to the MP steam reformers. All or most of the produced water is consumed by the MP transitioning the project to Zero Liquid Discharge (ZLD).

Regional transfers,  marine shuttle exports or the offshore outfall disposal is integrated into this option to take care of excess water production during plant maintenance shut-downs or delayed start-ups..

A High-Capacity Finger Slug Catcher at AKAB hub to ensure that slugs do not overwhelm the system, To ensure uninterrupted gas production of 500 MMscfd, multiple redundant disposal "relief valves" are integrated, including.

BFPCL’s objective is to ensure full regulatory compliance and environmental stewardship, in alignment with the following Nigerian statutory and regulatory instruments:

- Petroleum Industry Act

- Environmental Guidelines and Standards for the Petroleum Industry in Nigeria administered by the Nigerian Upstream Petroleum Regulatory Commission

- Environmental Impact Assessment Act

- National Oil Spill Detection and Response Agency Act

- National Environmental Standards and Regulations Enforcement Agency Act

The final solution will be selected based on regulatory compliance, environmental protection, operational reliability, lifecycle cost, and long-term field development strategy.

# Introduction

## Background Information

Brass Fertilizer and Petrochemical Company Limited (BFPCL) is developing a 10,000 metric tonnes per day (MTPD) capacity greenfield methanol production facility in Akabel (AKAB) in Odioma, Brass LGA, Bayelsa state. The plant complex is to focus on phase-1 production of natural gas feedstock from 2 RAEC fields, - Elepa (ELEP) & Bubouwe Bou (BBOU) in OML 33, anchored on GSPA 270, and the import is the production from these two fields.

In as much as the focus is on BBOU & ELEP, we would like to draw our attention to Okpokunou (OKNU) production, anchored on a separate GSPA 200 which would invariably impact on produced water volumes, hence the water handling, processing, and disposal plan.

It is therefore of pragmatic imperative that Okpokunou production be included in the produced water handling, processing, and disposal plan in BBOU & ELEP, as the OKNU productions would be happening at the same time as BBOU & ELEP.

The Project involves the development, construction, and operation of a plant comprising:

Two trains of 5,000 MTPD methanol plant, producing a total of 10,000 MTPD of methanol

500 MMscfd Gas Processing Plant (“GPP”), which will extract NGL from the natural gas feedstock, prior to feeding the lean gas to the methanol plant.

Gas gathering pipelines and associated manifolds – BBOU-ELEP-AKAB Pipelines, comprising a combination of 20” x 8km BBOU-ELEP Pipeline Segment + 28” x 32km ELEP-AKAB Pipeline

Product Export Lines and SBM for the export of the methanol, and NGL of the GPP.

All offsite, utility, and off-plot facilities (including product storage, internal power generation, steam generation, and export jetty) for the operation of the processing plants.

Produced water is generated from Elepa, and Bubouwe Bou, under GSPA 270; and Okpokunou, under a separate GSPA 200, from formation water.  Initial water production will be 36 Mbwpd and will increase, as the field matures, up to, 40 Mbwpd over life-of-field (LOF).

The produced water will be:

- Treated to meet EGASPIN discharge limits administered by NUPRC.

- Primarily re-injected into a designated disposal well.

- Surface discharged only under contingency conditions and within regulatory limits.

The treatment system will be designed with +30% capacity margin and redundancy to ensure continuous compliance. Online oil-in-water (OIW) monitoring and automatic diversion systems will be included.

BFPCL commits to full regulatory compliance, environmental protection, and continuous monitoring in accordance with applicable Nigerian regulations.

## Schematics of FEED Scope

Schematics: - Phase-1 Gas Supply from BBOU & ELEP alongside Okpokunou Gas Supply

Satellite Imagery: - Phase-1 Gas Supply from BBOU & ELEP alongside Okpokunou Gas Supply

# Regulatory Standards and Requirements

## Regulatory Instruments

Key governing instruments include:

- Petroleum Industry Act (PIA) 2021

- Environmental Guidelines and Standards for the Petroleum Industry in Nigeria (EGASPIN)

- NUPRC Upstream Petroleum Environmental Regulations

- Applicable DPR legacy guidelines (where referenced)

- National Environmental (Surface & Groundwater Quality Control) Regulations

## Regulatory Standards / Limits

### Disposal Methods in NUPRC Guidelines

The disposal methods recommended under the current NUPRC regulations are outlined below. All identified methods have been duly considered in the BFPCL produced water treatment, handling, and disposal study, and have informed the development of the primary disposal pathways evaluated:

| Method | Regulatory Status | Requirements |
| --- | --- | --- |
| Re-injection | Preferred | Water is pumped back into the reservoir for pressure maintenance. Requires NUPRC approval and integrity tests. - Into depleted reservoirs or designated disposal wells - Requires reservoir compatibility study - Must prevent formation damage and groundwater contamination - Continuous pressure monitoring required |
| Surface Discharge (Onshore/Swamp Conditions) | Restricted | Discharging into freshwater rivers is increasingly discouraged. - Only after full treatment - Must meet EGASPIN effluent standards - Discharge point must be environmentally approved - Diffuser systems required in some cases |
| Offshore Discharge | Permitted | Allowed beyond a certain distance from the shoreline, provided oil content is within the 40 mg/L limit. - Online Oil-in-Water (OIW) analyzers installed that provide a continuous data feed to NUPRC and - an Automatic Diverter Valve triggered by the 40mg/L limit, sending the contaminated water back to a "slop tank" rather than into the ocean. |
| Evaporation / Containment | Limited Use | - Requires lined pits - No unlined earthen pits permitted - Must prevent seepage to groundwater |

### Effluent Discharge Limits (2026)

Regulations differentiate strictly between Inland/Nearshore (high-risk swamp and land areas) and Offshore (deep sea) operations; the following limits will be observed:

| Parameter | Inland / Nearshore | Offshore* |
| --- | --- | --- |
| Oil & Grease | < 10 mg/L | < 30 mg/L |
| Total Dissolved Solids (TDS) | < 2,000 mg/L | Ambient Seawater levels |
| Chemical Oxygen Demand (COD) | < 125 mg/L | Monitoring required |
| BOD (Biol. Oxygen Demand) | < 30 mg/L | < 50 mg/L |
| Phenols | < 0.5 mg/L | < 1.0 mg/L |
| Methanol Content | < 2.0 mg/L (Residual) | < 5.0 mg/L |
| pH | 6.5 – 8.5 | 6.5 – 8.5 |
| Heavy Metals (Pb, Hg, Cr) | Near-zero / Non-detectable | Strictly monitored |
| Turbidity | < 10 NTU | N/A |
| Temperature | Ambient ± 3°C | Ambient ± 5°C |

*>12 nautical miles

## Regulatory Monitoring and Reporting Requirements

### Statutory Operational Control

As a responsible corporate entity, BFPCL will implement the following requirements:

- Environmental Management Plan (EMP): will be prepared and approved to demonstrate how produced water will be minimized and treated.

- Environmental Risk Register (ERR): of potential water-related risks (e.g., pipe bursts, treatment failure) will be maintained.

- NUPRC Quarterly Effluent Monitoring & Reports (ECMR):

- Produced water management plan (PWMP)

- The Environmental Remediation Fund (ERF) from Section 103 of the PIA, all operators must contribute to enable the NUPRC to clean up the site immediately.

- Real-time monitoring (where applicable)

- Flow meters on discharge lines

- Secondary containment for treatment systems

- Sludge handling and hazardous waste disposal procedures

- Emergency spill response plan

- Host Community Development Trust, all operators must contribute 3% annual opex  to this fund

### Monitoring & Reporting Obligations

BFPCL in compliance with statutory requirements will implement the following:

- Monthly effluent reporting to NUPRC

- Annual Environmental Performance Report

- Environmental audit every 3 years

- Immediate reporting of non-compliance events

- Record retention for regulatory inspection

Failure to comply may attract:

- Administrative penalties

- Suspension of operations

- Environmental remediation orders

### Technical & Operational Verification

To stay compliant, BFPCL like other operators in the Niger Delta must now implement:

- Continuous Monitoring: Real-time sensors for Oil-in-Water (OIW) content.

- Quarterly Bioassays: Testing the toxicity of treated water on local aquatic species (e.g., tilapia or shrimp) every 3 months.

- Integrity Audits: Annual pressure testing of re-injection wells to ensure produced water isn't leaking into freshwater aquifers.

### Regulatory Compliance Roadmap

To ensure your project remains "audit-ready" BFPCL will implement:

- Baseline Study: Ensure the Environmental Impact Assessment (EIA) that specifically covers the chemical footprint of methanol byproducts, not just oil.

- Water Balance Audit: the NUPRC-required  "Water Map"—showing how many barrels of water enter the plant vs. how many are recycled or disposed of.

- Facility Commissioning: BFPCL invite NUPRC/NMDPRA for a "Pre-Commissioning Inspection" of the treatment plant before starting production.

### Produced Water Characterization

BFPCL will:

- Conduct full physicochemical analysis

- Determine oil-in-water (OIW) concentration

- Measure heavy metals, TDS, BTEX, PAHs, salinity, and naturally occurring radioactive materials (NORM)

- Establish baseline environmental data before discharge

Routine sampling and laboratory certification are mandatory.

### Treatment & Processing Technologies

- Primary physical separation 
 Gravity separation, API oil–water separators, hydrocyclones, and dissolved gas/air flotation (DGF/DAF) systems remove bulk oil and suspended solids before advanced treatment.

- Secondary & tertiary treatment
. Adsorption using activated carbon or local bio-adsorbents (e.g., modified agricultural wastes like banana or orange peels) can reduce hydrocarbons and metals. 
• Membrane filtration (micro-, ultra-, nano-filtration) targets dissolved contaminants and polishing to meet discharge criteria. 
• Biological methods (e.g., constructed wetlands, algal photobioreactors, SBRs with microalgae) offer eco-friendly polishing and nutrient removal.

- Modular hybrid systems
• Combining gravity, biological, and membrane units can achieve higher pollutant removal in a compact footprint suitable for onshore Niger Delta sites.

- Reuse and beneficial application
• Treated produced water can be reused for oilfield operations (e.g., drilling fluids, fracture water) or industrial non-potable uses, reducing freshwater demand and disposal volumes.

### Handling and Processing Standards

Produced water in the Niger Delta is highly saline and contains hydrocarbons, heavy metals, and production chemicals. The Regulatory mandate of a multi-stage treatment process will be followed:

Treatment Objectives

Technologies like hydrocyclones, gas flotation units, and filtration will be used to meet specific effluent limits before any form of release. Key limits include:

- Oil & Grease Content: Typically limited to 30 mg/L for nearshore/inland and 40 mg/L for offshore (though recent NUPRC directives push for stricter <10-20 mg/L in sensitive swamp areas).

- Salinity (TDS): Must be monitored; inland discharge is generally restricted if it significantly alters the salinity of freshwater ecosystems.

- Heavy Metals: Strict limits on Lead, Mercury, and Chromium to protect the Niger Delta's biodiversity

### Technical Handling for Gas & Methanol Wastewater

Because the water isn't just "oily" but contains dissolved chemicals, standard hydrocyclones aren't enough. Our facility will deploy:

- Stripping Columns: To recover dissolved methanol from process water before it enters MP Steam Reformer Feed or the waste stream. This is both a "Wealth" (product recovery) and "Compliance" (COD reduction) move.

- Biological Aerated Filters (BAF): Bacteria are used to "eat" the residual methanol and glycols (TEG/MEG) in the water.

- Advanced Oxidation Processes (AOP): Using ozone or UV/H₂O₂ to break down complex chemicals that bacteria can't digest.

# BFPCL Framework & Implementation

## BFPCL Framework

The Company’s Framework encompasses:

- Road Map and Decision Flow Charts,

- Benchmark & Best Practices towards compliance;

- Benchmark & Best Practices towards continuous improvement that will strengthen regulatory standing and ESG positioning:

- Target ≤ 20 mg/L OIW (international best practice)

- Adopt hydrocyclones + induced gas flotation systems

- Implement reinjection where feasible (reduces surface discharge risk)

- Deploy digital monitoring & SCADA integration

- Conduct regular third-party lab verification

- Align with ISO 14001 Environmental Management Systems

- Standard Operating Procedures (SOPs), including Key Performance Indicators (KPIs):

- Development of Procedures & Methods

- Controls for Safe Handling, and Treatment,

- Monitoring, and Disposal of produced water generated in accordance to Petroleum Industry Act (2021)

- Key Performance Indicators (KPIs)

- Oil-in-Water: < 10-20 mg/l

- TSS: < 5-30 mg/l

- Reinjection-Ready Design

- Strategic Value, including:

- ESG Risk Reduction

- Regulatory Compliance Alignment

- Scalable for High Water-Cut

- Recommended Treatment Flow:

- Separation  Hydrocyclone  Induced Gas Flotation  Dual Media Filter  Cartridge Filter  Reinjection/Reuse

## BFPCL Roadmap

The Road map considers 6 concept selections (see table below) to be evaluated as part of detail design:

Industrial Re-use of the produced water has been adjudged the best option.  It utilizes all the well-produced water from wet gas pushed all along the 125km, the produced water from both the 500 MMscfd Gas Processing Plant (GPP) and the 10,000 MTPD Methanol Plant (MP). This complies with the NURPC Zero Liquid Discharge (ZLD) strategy for 2026.  At full operation, the water requirement for the methanol plant industrial loop will be barely met, which means zero water balance for discharge.

The features of this option include; a high-capacity finger type slug catcher at AKAB hub to manage possible slug surges, a central Water Treatment Plant (WTP) with 2-stage Reverse

Oxygen (RO) capacity to treat water to “Boiler Feedwater Quality” (removes 99.9% of total dissolved solids (TDS) and chlorides, which are "catalyst killers) , an Ion Exchange and  a Methanol Stripper, critical  to remove alcohol from the reaction water before it reaches the steam reformers.

This option is combined with the Offshore Outfall option   or the Transfer to Brass Terminal or barging to third party to dispose of redundant water. Excess water may occur during plant downtimes for maintenance, delayed start-up of second train, high water-cuts from wells in later years of life.

| Priority | Option | Key Facilities | Sub-Facilities | CAPEX (Est.) | OPEX (Est.) | Pros | Cons | Regulator Ease (NUPRC/NMDPRA) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | Industrial Re-use (The Core Goal) | Central WTP & MP Utility Loop | 2-Stage RO, Ion Exchange, Methanol Stripper | $45M - $60M | Medium | Sustains 10k MTPD MP steam needs; high ESG rating. | Very high-water purity specs; risk of "Catalyst Poisoning" if RO fails. | Highest: Directly aligns with 2026 "Green Industry" mandates. |
| 2 | Hybrid: Re-use + BR Transfer (18km) | Inter-Terminal Pipeline (8") | AKAB Pumping Skid, LACT Metering, TPA Contract | $12M - $18M | Low | Lowest cost synergy; uses existing cluster capacity for Phase 1 surplus. | Dependent on BR Terminal uptime; complex TPA (Third Party Access) legalities. | High: NUPRC favors "Facility Sharing" over new construction. |
| 3 | Hybrid: OKNU Source Sep + Barge | OKNU Loading Jetty + Storage Tanks | Hydrocyclones at OKNU, 20k bbl Double-hull Barges | $30M - $45M | High | Keeps 125km line "Dry" (max gas flow); removes salt at source. | Security/Logistics risks in the swamp; high river transport costs. | High: Proactive waste management before trunkline transport. |
| 4 | Hybrid: OKNU Source Sep + Spiking | Multiphase Trunkline (125km) | AKAB Finger Slug Catcher (15k bbl), HP Injection Pumps | $35M - $40M | Medium | Minimal OKNU footprint; one pipe for everything. | Huge liquid slugs at AKAB; high corrosion risk; reduces gas velocity. | Moderate: Requires high-spec leak detection & telemetry. |
| 5 | Offshore Outfall & Emergency Hybrids | 22.2km Marine Pipeline | Offshore Pumps, Diffuser Heads, 30ppm Monitoring | $22M - $28M | Low | Absolute control; essential for Phase 1 surplus and maintenance trips. | Perception of "dumping"; strictly monitored for 30ppm OIW limits. | Moderate: Approved only as a secondary or emergency backup. |
| 6 | 3rd Party Outsourcing | On-site Reception & Loading | Transfer Pumps, Custody Meters, Storage Tanks | $12M - $18M | Very High | Shifts technical burden to contractor. | Zero control over disposal cycle; high per-barrel "Gate Fees." | Low: Regulators prefer operators to own their waste lifecycle. |

The well re-injection (PWRI) option has not been considered since there are no wells available for it.

Roadmap Flow Chart

The Decision Flow Chart

PRODUCED WATER MANAGEMENT FLOWCHARTS

Secondary Treatment (Hydrocyclone / IGF)

Secondary Treatment (Hydrocyclone / IGF)

Primary Separation

Primary Separation

Primary Separation

LEVEL 2- GOVERNANCE FLOW (OPTION A)

Sampling

Lab Analysis

Compliance Check

Report to NUPRC

Exceed Limits?

No

Yes

Notify Regulator

Routine

Reporting

Root Cause Analysis

Corrective Action Plan

Verification Testing

Secondary Treatment (Hydrocyclone / IGF)

Secondary Treatment (Hydrocyclone / IGF)

Primary Separation

Primary Separation

Primary Separation

# Produced Water Characterisation & Risk Assessment

## Background Information

### Source & Composition

Produced water typically contains:

- High salinity and dissolved solids

- Hydrocarbons (oil & grease), organic compounds

- Heavy metals (Fe, Pb, Ni, Cr, etc.)

- Other inorganic constituents

These contaminants vary by field and over time and often exceed regulatory limits without treatment.

### Environmental & Health Risk Assessment

- Toxicity to ecosystems: Untreated produced water can harm aquatic life and soil quality, affecting fisheries and agriculture.

- Human exposure risks: Communities relying on surface/groundwater can be adversely affected if produced water is improperly discharged.

### Data & Monitoring Plan

- Baseline sampling: Establish pre-production water quality baselines.

- Routine testing: Frequent sampling of produced water for parameters like oil & grease, TDS, metals, BOD/COD.

- Trend analysis: Use time-series data to detect deterioration or spikes in contaminants.

## Treatment & Management Strategies

A robust produced water management strategy combines prevention, treatment, reuse, and disposal.

### Source Reduction & Production Practices

- Downhole oil-water separation: Separates water before reaching surface facilities, reducing downstream handling volumes.

- Reservoir management: Reduce water cut through well interventions and pressure maintenance to minimize produced water volumes.

### On-Site Treatment Technologies

A layered approach combining physical, chemical, and biological treatment:

- Primary separation: Gravity separators or Gas-Oil Separation Plants (GOSP) remove bulk oil and water phases.

- API separators: Used for initial removal of free oil and suspended solids.

- Induced gas flotation & skimming: Enhance removal of dispersed oil droplets.

- Chemical dosing: Demulsifiers, coagulants and flocculants assist aggregation of pollutants.

- Adsorption & filtration: Media filters (e.g., walnut shell filters), bio-adsorbents (groundnut shells, sugarcane bagasse, etc.) can reduce metals and residual hydrocarbons.

- Biological treatment: Emerging techniques using microbes or algae can reduce organic loads.

### Off-Site / Advanced Options

- Reinjection for disposal or EOR: Treated water can be reinjected into subsurface formations for disposal or reservoir pressure support.

- Zero Liquid Discharge (ZLD) approaches: Advanced evaporation/desiccation technologies can eliminate liquid discharge and generate solid by-products.

## Water Reuse & Resource Optimization

### Internal Reuse

- Operational reuse: Use treated produced water for drilling, work-overs, or pressure maintenance where permitted.

- Reduced freshwater demand: Prioritize reuse to mitigate freshwater consumption, especially in water-scarce zones.

### Beneficial Reuse Opportunities

- Agricultural irrigation or industrial uses: Only if water quality meets safe reuse thresholds and community consent.

- Community supply potential: Explore long-term treatment pathways that could support local water demands, subject to stringent safety standards.

## Monitoring, Reporting & Compliance Assurance

### Environmental Monitoring

- Continuous monitoring systems at discharge points.

- Performance checks for treatment systems.

- Independent audits and third-party verification where appropriate.

### Reporting

- Periodic compliance reports to NUPRC & environmental regulators.

- Public transparency on water quality and management performance.

### Corrective Actions

- Non-compliance triggers mandatory review and mitigation plans.

- Incident response plans in case of spills or accidental discharges.

## Stakeholder Engagement & Community Safeguards

### Community Consultation

- Regular engagement with host communities on environmental concerns, water impacts, and management plans.

### Grievance Mechanisms

- Establish clear channels for communities to report water quality issues or grievances.

### Capacity Building

- Support local training and education on produced water risks and safeguards.

## Research, Innovation & Continuous Improvement

### Technology Trials

Pilot novel treatment and reuse technologies, including bio-adsorbents and advanced separation systems.

### Standards Review

Regularly update local discharge and reuse criteria to align with international best practices and emerging science.

### Collaborative Initiatives

Industry-government partnerships for data sharing, research and sustainable water management strategies.

## Summary of Best Practices

- Strict compliance with environmental regulations and effluent limits.

- Comprehensive characterization of produced water to inform treatment design.

- Implementation of layered treatment systems combining physical, chemical, and biological methods.

- Promotion of reuse and resource optimization to reduce freshwater demand.

- Robust monitoring, reporting, and adaptive management to ensure long-term compliance.

- Meaningful stakeholder engagement, especially with local communities.

# Produced Water Management Policy

## Scope

This policy applies to:

  - All onshore, swamp, and shallow offshore assets in the Niger Delta

  - Flow stations, central processing facilities (CPF), early production facilities (EPF)

  - Produced water handling, storage, treatment, reuse, discharge, and reinjection systems

  - Contractors and third-party operators

## Discharge Standards

Produced water shall not be discharged unless it meets prescribed limits for:

  - Oil & Grease

  - Total Suspended Solids (TSS)

  - Total Dissolved Solids (TDS)

  - pH

  - Heavy Metals (Fe, Pb, Cr, Ni, Cd, etc.)

  - Temperature

Stricter limits apply for inland/swamp discharge compared to offshore discharge.

## Produced Water Management Hierarchy

The Company adopts the following hierarchy (highest priority first):

### Prevention & Minimization

  - Reservoir management to reduce water cut

  - Well interventions to control water production

### Reuse & Reinjection

  - Pressure maintenance

  - Enhanced Oil Recovery (EOR)

  - Operational reuse (drilling/workover fluids)

### Treatment & Controlled Discharge

  - Multi-stage treatment systems

  - Continuous monitoring before discharge

### Disposal

  - Approved subsurface disposal wells

  - No uncontrolled surface discharge

## Technical Standards

### Characterization

Produced water shall be characterized for:

- Hydrocarbon content

- Salinity

- Metals

- Naturally Occurring Radioactive Materials (where applicable)

- Dissolved gases

Baseline environmental data must be established before production.

### Treatment Philosophy

A multi-barrier treatment approach shall be implemented:

Stage 1 – Primary Separation

- Three-phase separators

- Heater treaters

- Free water knockouts

Stage 2 – Secondary Treatment

- Hydrocyclones

- Induced Gas Flotation (IGF) units

- Skim tanks

Stage 3 – Tertiary/Polishing

- Media filters (e.g., walnut shell filters)

- Cartridge filters

- Adsorption systems

- Chemical polishing

Discharge is permitted only after compliance verification.

## Monitoring & Reporting

### Monitoring Requirements

- Daily operational monitoring

- Weekly laboratory analysis (minimum)

- Continuous Oil-in-Water analyzer at discharge point

- Quarterly third-party validation

### Reporting

- Monthly compliance reports to NUPRC

- Immediate reporting of exceedances

- Annual environmental performance report

### Risk Management & Incident Response

- Produced water spill response plan aligned with NOSDRA requirements

- Secondary containment for storage tanks

- Emergency shutdown systems

- Root cause analysis for any exceedance

## Community & Environmental Safeguards

- Stakeholder engagement prior to discharge approvals

- Public grievance mechanism

- Protection of wetlands, mangroves, fisheries, and groundwater

- No discharge into potable water sources

## Governance & Accountability

| S/N | Role | Responsibility |
| --- | --- | --- |
|  | Asset Manager | Overall compliance |
|  | Production Superintendent | Operational integrity |
|  | Environmental Manager | Monitoring & reporting |
|  | HSE Team | Audit & training |
|  | Contractors | Compliance with this policy |

## Continuous Improvement

- Annual performance review

- Technology upgrades evaluation

- Water reuse expansion targets

- Benchmarking against international best practice

# PRODUCED WATER VOLUME MANAGEMENT

## Produced Water Volumes

The total produced water volume is given in the table below:

| Produced Water | Volume [bwpd] | Origin |
| --- | --- | --- |
| Prod Wells | 36,000-40,000 |  |
| GPP | 2,500-4,000 | knock out from 500 MMscfd from a drop from 40°C to 15°C from propane refrigeration or JT valves & condensate water mix |
| Methanol Plant | 35,000-40,000 | By product of CO2 hydrogenation in methanol production or syngas production |
| Total | 96,000-120,000 |  |

| Parameter | Current | Peak Forecast |
| --- | --- | --- |
| Average Daily | 36,000 bwpd | 40,000 bwpd |
| Design Capacity | 52,000 bwpd | — |

System designed with +30% contingency capacity.

## Treatment Philosophy & Process Design

### Treatment Objectives

- Reduce OIW to ≤ 20 mg/L

- Remove suspended solids

- Control scaling and corrosion

- Enable safe reinjection

### Process Description

Produced water treatment stages:

- Three-Phase Separator

- Hydrocyclone Package

- Induced Gas Flotation (IGF) Unit

- Dual Media Filters

- Cartridge Polishing Filters

Sludge collected in slop tank for periodic removal.

### Chemical Treatment

- Demulsifier

- Corrosion inhibitor

- Scale inhibitor

- Biocide (controlled dosage)

All chemicals approved and MSDS attached.

### Monitoring Systems

- Online Oil-in-Water analyzer

- Flow meters

- Pressure indicators

- High-OIW alarm & automatic diversion to slop tank

## Disposal Strategy

### Primary Disposal: Reinjection

Produced water will be re-injected into Disposal Well DW-01.

Reinjection Design:

- Injection rate: 15,000 bwpd

- Reservoir depth: 8,200 ft TVD

- Fracture gradient margin maintained

- Continuous pressure monitoring

Reservoir compatibility studies confirm no scaling risk.

### Contingency Discharge

If reinjection unavailable:

- Treated water discharged to controlled outfall

- Discharge point coordinates documented

- OIW continuously monitored

- Immediate shutdown if limits exceeded

## Compliance With Discharge Limits

System designed to meet:

- Oil in Water: ≤ 29 mg/L

- No visible sheen

- TSS within EGASPIN limits

- Heavy metals within allowable thresholds

Design target OIW: ≤ 15 mg/L (internal standard)

## Environmental Risk Assessment

HAZOP conducted on produced water system.

Identified risks:

- Separator upset

- IGF failure

- Pipeline rupture

- Sludge overflow

Mitigation measures:

- Redundant pumps

- Secondary containment

- Automated shutdown

- Spill kits at discharge point

Environmental sensitivity assessment confirms proximity to mangrove ecosystem; therefore, reinjection prioritized.

## Monitoring & Reporting

### Sampling Frequency

- OIW: Continuous monitoring

- Full lab analysis: Monthly

- Heavy metals: Quarterly

### Reporting

- Monthly compliance report to NUPRC

- Annual Environmental Performance Report

- Immediate notification to NUPRC and NOSDRA in case of exceedance

## Sludge & Waste Management

Estimated sludge generation: 1.2 m³/day.

Management:

- Stored in closed sludge tank

- Removed quarterly

- Disposed via licensed waste contractor

- Waste manifest documentation maintained

## Integrity Management

- Corrosion coupons installed

- Chemical dosing optimization

- Quarterly pipeline inspection

- Cathodic protection system installed

## Emergency Response

Integrated with Oil Spill Contingency Plan.

In event of exceedance:

- Immediate diversion

- Shut down discharge

- Notify NUPRC within regulatory timeframe

- Root cause investigation

## Training & Governance

- Environmental Officer appointed

- Operators trained quarterly

- Competency matrix maintained

- Internal audit conducted annually

## Decommissioning

At end-of-field:

- Reinjection wells plugged per regulation

- Treatment system dismantled

- Contaminated soil remediated

- Final environmental closure report submitted

# CONCLUSION

BFPCL confirms that produced water generated at OML 33 will be:

- Properly characterized

- Adequately treated

- Preferentially re-injected

- Continuously monitored

- Fully compliant with NUPRC regulatory requirements

The system will be designed with environmental protection as the primary objective in this sensitive Niger Delta swamp environment.

Top of Form

Bottom of Form